IBM LAN Server installation with NE2000 Compatible NDIS driver

--------------------------------------------------------------

for

NE2000 Compatible EtherPair/EtherCoax Family LAN Adapter Driver

For IBM LAN Server NDIS Environments

1.  DOS LAN REQUESTER

---------------------

a. At the DOS prompt, insert the IBM LAN SupportProgram (LSP)

Version 1.30 (or 1.31) Diskette into your disk drive. (A: or B: )

b. Type Dxmaid at A:> prompt , then press Enter.(#���)

The IBM logo appear ,press Enter.

c. Press Enter twice.

The Environment Information screen appears.

d. Specify the following values for the fields on this screen:

������������������� Enviroment   Information �������������������Ŀ

�                                                                �

� Use the Space Bar to toggle between choice:                    �

�                                                                �

�       Configuration for this computer?                Yes      �

�       Use existing configuration information?         Yes      �

�       Do you have adapter option diskettes?           Yes      �

�       Are you configuring for two adapters?           No       �

�       Do you need 802.2 interface suport?             Yes      �

�                                                                �

� Type change as need to drive and directory information below:  �

�                                                                �

�       Source for LSP                                  A:\      �

�       Target for new configuration                    C:\lsp   �

�                                                       -        �

������������������������������������������������������������������

e. Make sure the value are correct . Press Enter to store your choices.

Process Adapter Option Diskette screen appear.

g. Inseret the driver diskette in your source drive and specify the

path: A:\NDIS(or b:\). Then press Enter.

g. The necessary files are copied from the DriverDisk to your

target directory on the hard disk.

g. Press Enter three times.

h. Insert the LSP diskette and press Enter.

The Current Configuration screen appears with the NE2000 Compatible

ethernet adapter and protocols already selected ("IBM IEEE 802.2"

and "IBM OS/2 NETBIOS").

i. Press F4. LSP is installed completedly.

j. Use a text editor to edit the CONFIG.SYS file. Specify at least

20 files in the FILES= line, like this:

files=20

Once you have saved the CONFIG.SYS file, reset the computer.

k. Insert the DOS LAN Requester Install Diskette 1.

l. Type: install. Press Enter.

IBM logo appears, press Enter.

m. Press Enter twice more.

The program prompts you for the pathname of the DOS LAN

Requester directory.

n. Specify the pathname for the DOS LAN Requester directory.

(C:\DOSLAN is the default.)  Press Enter.

The program asks you which services you require.

o. Highlight either the "Send messages" or "Send, View, Edit, and

Log messages" option. (Normally, you select the second option.)

Press Enter to confirm your choice.

p. Specify the machine's ID and the domain ID. The domain ID must

be the domain server ID and must be the same for all

workstations on the network.

q. Follow the prompts to complete the installation.

2.   OS/2 LAN Server

-----------------------

a. copy ETHNE.OS2 & UL000XO.NIF from a:\NDIS to c:\ibmcom\macs.

b. push "OS/2 LAN Service"  icon.

c. push "Installation and configuration" icon.

IBM logo appear.

d. press <ok>.

e. select <advance> , press Enter.

f. select " Install or Configuration this Workstation".

g. select server type (add server or domain controllor).

" install or remove " appear.

h. select "Configure a component".

i. select "LAN Adapter and protocol support" .

j. push "configure" button.

k. select "Configure workstation".

l. Select "NE2000 Compatible ethernet adapter card" in Network Adapter

dialog presss <Add> button,and select 802.2 press <Add> IBM NetBIOS

press <Add> ,then press <OK>.

m. press <OK> four times.

n. Press <ok> to backup old CONFIG.SYS & AUTOEXEC.BAT and create new CONFIG.SYS

and AUTOEXEC.BAT.

o. ShutDown the computer and restart it.