Windows for Workgroup installation with NE2000 Compatible NDIS driver

---------------------------------------------------------------------

for

NE2000 Compatible EtherPair/EtherCoax Family LAN Adapter Driver

For Windows for Workgroup NDIS Workstation Environments

1. Installation Introduction for Windows for Workgroup

-------------------------------------------------------

a. Select "Control Pannal" icon in Main group in Windows enviroment.

b. Select "Network" icon.

c. Select "Adapters" item in "Option" dialog.

d. push Add button.

e. Select "Unlisted or Updated Network Adapter" .

"install driver" appaers.

f. Key in specified drive and path name.(A:\ndis or B:\ndis)

g. Insert your Driver Disk in specified drive, then press Enter.

h. press <OK>.

i. press <close>.

j. press <OK> twice.

k. Restart computer.

2. Example of CONFIG.SYS file

FILES=30

buffers=30

STACKS=9,256

DEVICE=c:\windows\HIMEM.sys

device=C:\WINDOWS\protman.dos /i:C:\WINDOWS

device=C:\WINDOWS\ethne.dos

device=C:\WINDOWS\workgrp.sys

LASTDRIVE=y

3. Example of PROTOCOL.INI file in Windows for Workgroup

[network.setup]

version=0x3100

netcard=ul000xd,1,UL000XD

transport=ms$netbeui,MS$NETBEUI

lana0=ul000xd,1,ms$netbeui

[protman]

DriverName=PROTMAN$

PRIORITY=MS$NETBEUI

[UL000XD]

DriverName=ETHNE$

[MS$NETBEUI]

DriverName=netbeui$

SESSIONS=10

NCBS=32

BINDINGS=UL000XD

LANABASE=0

4. Example of AUTOEXEC.BAT

C:\WINDOWS\net start

C:\WINDOWS\SMARTDRV.EXE

@ECHO OFF

PROMPT $p$g

PATH C:\WINDOWS;

SET TEMP=C:\WINDOWS\TEMP