MS Windows for Workgroups Installation With LongShine LAN Cards

===============================================================

This driver diskette includes an OEMSETUP.INF file for Windows for Workgroups.

Using the OEMSETUP.INF file, the user can select whichever LongShine lcs8634 network

adapters are to be installed based on Windows dialog menus.

Installation procedures:

1. Proceed to install Windows for Workgroups.  If you already have an

installed Windows for Workgroups, open the Control Panel Window to

change the network settings.

2. When Windows asks for the type of network adapter, it will show a

list of all network adapters supported by Windows.  You must select

'Unlisted or Updated Network Adapter'.

3. Windows will ask for an OEMSETUP.INF file. Since the  OEMSETUP.INF

file is included in the  driver diskette, insert the driver diskette

in drive A:

Input the full path name where the OEMSETUP.INF file is located.

(i.e. A:\WFW)

4. Ya Windows dialog menu listing the network adapters

in the OEMSETUP.INF file. Select the network adapter that you will be using.

5. After selecting the network adapter, Windows will look for the NDIS driver

of the network adapter you have selected.

At this time, the ETHER driver diskette should still be in drive A:.

6. Windows will copy the NDIS driver file and update the Windows network

system automatically.  Be sure to configure the network adapter if you

are using I/O port or IRQ other than the default.

At this point, you can finish the installion or continue the process

according to your preferences.

7. After installation is completely finished, restart your system for the

updates to take effect.

* Windows for Workgroups is a registered trademark of MicroSoft Corp.