NWPATCH

NetWare Driver Patch Utility

User Document

Version 1.05       23 December 1988

-------------------------------------------------------------------

NWPATCH.EXE is a patch utility for Novell NetWare (*) drivers

that operate with Megahertz LapLAN adapter boards.  This utility

will patch the following files:

NetWare v2.0a:	(MHZ driver versions 2.05 or later)

SHELLDRV.OBJ - WORKSTATION

ACOMDRV.OBJ  - SERVER   BCOMDRV.OBJ - SERVER

CCOMDRV.OBJ  - SERVER   DCOMDRV.OBJ - SERVER

NetWare v2.1x, including ELS NetWare Level II:

(MHZ driver versions 2.10.0 or later)

WD8003SH.OBJ - WORKSTATION

WD8003AD.OBJ - SERVER   WD8003BD.OBJ - SERVER

WD8003CD.OBJ - SERVER   WD8003DD.OBJ - SERVER

NetWare WDELS Level I:

ANET3.COM  - WORKSTATION

NET$OS.EXE - SERVER

A COPY of the file to be patched should be put in your default

drive/directory before the patch program is executed.  It is not

safe to patch a driver unless there is a backup copy of the

original driver saved on a different disk or directory.

The patch program can then be started by typing NWPATCH.  The

user should know:

o which Novell version of NetWare the driver is from;

o whether the driver is for a workstation or server;

o for a server driver, whether it is to be the primary,

secondary, etc.;

o for NetWare v2.0a drivers, what the MHZ driver version

number or the creation date of the original driver is;

o for NetWare ELS, what the MHZ driver version number

is (using the ANET3.COM program, typing "ANET3 i" will

cause the version number to be displayed).

Follow the instructions given by the patch utility to change

the:

PC INTERRUPT (IRQ)

BASE I/O ADDRESS

SHARED RAM MEMORY BASE ADDRESS

The patch utility DOES NOT do error checking, so please ensure

that the new values entered are VALID!  When the parameters

displayed match the values desired, enter 'E' or 'e' to save

them in the appropriate driver file.

To quit the program without making any changes in the driver

file, enter 'Q' or 'q'.

== NETWARE GENERATION ==

If you used NWPATCH to change a driver for NetWare v2.1x, be

sure to note the following rules when you run the NetWare shell

or system generation programs:

o Do not select Default Configuration from the Configuration

Level menu.

o Select the "WD Star/EtherCard PLUS ..." driver option from

the Available LAN Drivers menu.

o Then select Option 0 from the Driver Configurations menu,

even though the text shown for Option 0 does not match the

changes that you made with the NWPATCH program.

(*) NetWare is a registered trademark of Novell, Inc.

== VERSION HISTORY ==

<< Version 1.05 -- Current Version:  December 1988 >>

Changes were made to allow operation with MHZ shell Version 2.09

of the NetWare v2.0a driver.

<< Version 1.04 -- June 1988 >>

Original version.