This README file describes how to enable the RPL function on

workstations with a hard drive.  Two utilities, NDCRPLEN.EXE and

NDCRPLDS.EXE, are provided for enabling the RPL function on

workstations with hard drives.

To use your adapter's (ND1104/ND11x0/ND1111) RPL function and

boot your PC workstation from the network server, you must enable

the RPL function on the workstation by turning the hard disk (if

present in the workstation) into a non-bootable device. The

utility, NDCRPLEN.EXE, allows you to deactive your hard disk's

boot partition and enable the RPL function.  Another utility,

NDCRPLDS.EXE, allows you to re-activate your hard disk's boot

partition and disable the workstation's RPL function.

Note:

* The NDCRPLDS.EXE program assumes your hard disk boot device to

be Drive C:.  If you don't use drive C: to boot your workstation

you must use the DOS FDISK.EXE command to activate your hard

disk's boot partition.

* The NDCRPLEN.EXE and NDCRPLDS.EXE programs are in RPL

directory of this diskette.