TMS380/2000 NDIS MAC 2.0x Drivers for DOS and OS/2

General information on optional parameters for TR2KNDIS drivers.

1. Driver File Names and files.

DOS:	File Name:	TR2KNDIS.DOS  	DOS NDIS 2.0x MAC Driver

OEMSETUP.INF	WFW Installation file

TR2KFPA.COD	PacketBlaster adapter code

OS2:	File Name:	TR2KNDIS.OS2	OS2 NDIS 2.0x MAC Driver

TR2KNDIS.MSG	OS2 Message file

TR2KNDIS.NIF	OS2 Installation file

2. Typical CONFIG.SYS entries:

DEVICE=[drive:directory]TR2KNDIS.OS2

or

DEVICE=[drive:directory]TR2KNDIS.DOS

This line should be entered immediately following the line

which installs the PROTMAN.OS2 or PROTMAN.DOS device

driver.

OS/2 NOTE: You must specify a DPATH in CONFIG.SYS to the subdirectory

containing the TR2KNDIS.MSG file to enable driver error

messages and alleviate the "Cannot find message file" error

at boot time.

3. Add the following section to PROTOCOL.INI

[TR2KNDIS_NIF]

DriverName = TR2NDS$

4. Add TR2KNDIS_NIF to the bindings list of the protocol in the

PROTOCOL.INI. An example protocol section follows.

[NETBEUI_XIF]

DriverName = NETBEUI$

Bindings = TR2KNDIS_NIF

TR2KNDIS Parameters

The PROTOCOL.INI file defines the configuration of NDIS Protocol

and NDIS MAC drivers. The following table summarizes TR2KNDIS

parameters. For additional information about the PROTOCOL.INI

file and network device drivers, refer to the Microsoft LAN

Manager Network Device Driver Guide.

�������������������������������������������������������������������������Ŀ

�  Parameter         �         Range                � Default  � Default  �

�                    �                              �   DOS    �   OS/2   �

�������������������������������������������������������������������������Ĵ

�  DRIVERNAME        � TR2NDS$ or TR2NDSn$          � Required � Required �

�  IOADDRESS or PORT � See description.             �   0xA20  �   0xA20  �

�  BIT16             �                              �     -    �     -    �

�  LOOKAHEAD         � See description.             �     -    �     -    �

�  MAXTRANSMITS      � 3 - 50                       �     3    �     6    �

�  NETADDRESS        � "400000000000"-"7FFFFFFFFFFF"�    BIA   �    BIA   �

�  NOEARLYRELEASE    �                              �     -    �     -    �

�  NUMRXBUFFERS      � 3 - 100                      �     3    �     6    �

�  PCI               � See description.             �    N/A   �   0xA20  �

�  PIU               � 265 - 2048                   �    265   �    265   �

�  PROMISCUOUS       � See description.             �     -    �     -    �

�  RXBUFFERSIZE      � 256 - 17960                  �    2048  �    2048  �

���������������������������������������������������������������������������

DRIVERNAME

Identifies the network device driver. The DRIVERNAME is TR2NDS$

for the first driver and TR2NDS2$ for the second, and so on.

This parameter is required.

IOADDRESS or PORT

If no PORT is specified, the driver will assume the default

address. Valid addresses reside on a 32-bit boundary between

hexadecimal 0x100 and 0xFFE0. This parameter is required when

multiple adapter support is desired.

BIT16

Directs the driver to use 16 bit I/O data transfers to and from the

network adapter.  Without this keyword, the driver will attempt to

auto-detect the adapter data path width.

LOOKAHEAD

Directs the driver to use the Receive Lookahead method in lieu of

the default Receive Chain method.  NDIS requires the upper protocol

to support either method.  Some protocols have been found to only

support Receive Lookahead.  Receive Chain will typically give better

performance on DMA and PIO based adapters.

MAXTRANSMITS

Sets the number of transmits that can be queued simultaneously

by the driver.

NETADDRESS

This entry allows the driver to override the burned in address

(BIA). The entry must be a 12 character string of hexadecimal

digits, and must be enclosed in quotes.

NOEARLYRELEASE

This entry informs the driver to disable early token release.

Early token release reduces the average time another network

adapter must wait to transmit a frame. This entry is not

followed by an equal sign.

NUMRXBUFFERS

Sets the number of receive buffers to allocate.

PCI

(Required for OS/2 only) Instructs the driver that the adapter

port given on the PCI line of PROTOCOL.INI is a PCI adapter.

Valid I/O addresses for PCI are 0x100 to 0xFFE0.

PIU

Instructs the driver to implement Lying Sends on all transmits smaller

than the given PIU (Program Interchange Unit) size.  The driver by default

will verify each transmit on the physical ring before confirming the

transmit to the upper protocol.  Personal Communications 3270 ver 3.0x

reqires all PIU transfers to confirm the transmit before it is physically

transmitted on the ring. The PIU option enables this 'Lying' send.

PROMISCUOUS

Enable driver PROMISCUOUS function to allow adapter to copy all

frames to the upper protocol.  Used for LAN analysis tools. Must

install TR2KCAF.COD (or TR2KFPA.COD if PacketBlaster is used) in

the root directory of the default drive to support this feature.

RXBUFFERSIZE

Sets the size of buffers used to queue received frames in the

driver. Several buffers may be required to queue a large

frame. Performance is improved if a received frame fits in one

buffer. Use the following formula to determine maximum frame

size. MediaMaxSize equals 17960 @ 16 Mbps, or 4490 @ 4 Mbps.

MaxFrameSize =

min(min(8, NumReceiveBuffers) x ReceiveBufferSize, MediaMaxSize)

Note:

Adapters equipped with the FPA PacketBlaster option must use special

microcode to enable this feature.  Copy the file TR2KFPA.COD to the

root directory of the default drive to allow this code to be downloaded

to the adapter hardware. This will increase overall adapter performance.

Sample PROTOCOL.INI file:

[TR2KNDIS_NIF]

DRIVERNAME = TR2NDS$

PORT = 0xFCE0

MAXTRANSMITS = 10

NUMRXBUFFERS = 12

RXBUFFERSIZE = 1024

NETADDRESS = "400000000312"