This is the 4.16 release of TI2000's NDIS 3.0 Windows for Workgroups driver

Fixes and additions:

1) Added support for pci bios

Restrictions:

NONE

To install:

1) Select Network from the Program Manager window

2) Select Network Setup from the network window

3) The network setup will display the current installed network drivers and

the protocols bound to that adapter.

4) Click on Add Adapter

5) Select "Unlisted or Updated Network Adapter"

6) Click on OK

7) Enter the driver volume and directory, typically A:\, and click on OK

8) Select TMS380/2000 Token Ring Adapter

9) Click on OK

10) The driver files are copied to the windows directories.

11) The protocol.ini and system.ini are modified, click OK

12) To start the adapter the computer must be restarted.

An example protocol.ini file is provide on our install disk.

Keywords supported:

PORT - Specifies the i/o address of the adapter.  Default: A20

NETWORKADDRESS - Directs the driver to use this node address rather than the

networkaddress in the adapter's flash memory.

DMACHANNEL - Specifies the busmaster dma channel.  Default: pseudodma

INT - Specifies the interrupt. Default: 2

RINGSPEED - Specifies the ring speed. Default: 16meg/second

INST8 - Directs driver to use eight bit inputs/outputs to the adapter.

BIT16 - Directs driver to use sixteen bit inputs/outputs to the adapter.

WS0 - Same as bit16.

Note:  INST8 and BIT16 or WS0 are mutually exclusive. You can not specify

INST8 and BIT16 or WS0.

To use a keyword edit the protocol.ini in the windows directory.  This is

usually \windows.  In the section [TR2NDS]  add the keywords. Below is

an example using each of the keywords above.  For most situations you will

not need any keywords.

[TR2NDS]

DriverName=TR2NDS$

Adapters=TR2NDS

PORT=A00

NETWORKADDRESS=001A00007548

DMACHANNEL=6

INT=3

RINGSPEED=4

INST8=1

BIT16=1

WS0=1

#