TOSHIBA ETHERNET CARD DRIVER

FOR USE WITH 3COM/3PLUS

INSTALLATION GUIDE (DRAFT)

June 28, 1989

Copyright (c) 1989  Toshiba Corporation.

All Rights Reserved

[ Notice ]

The LAN driver software in this diskette is of beta version, and not yet

certified by 3Com corporation.  The first version of the driver software

will be released after the successful completion of the certification by

3Com corporation.

3Com, 3Plus and 3+ are registered trademarks of 3Com corporation.

Ethernet is a registered trademark of Xerox corporation.

MS-DOS is a trademark of Microsoft corporarion.

IBM is a registered trademark of International Business Machine Corporation.

[ The 3Plus Driver ]

This diskette contains Toshiba Ethernet card driver for use with 3Com's

3Plus network operation system. The driver software file name is TSBETH.SYS.

The TSBETH.SYS is a MS-DOS device driver, and installed at boot time

according to the command line specified in the CONFIG.SYS file.

This short document describes the overview of Toshiba LAN cards and how to

install this driver software in your workstation.

[ Toshiba LAN Cards ]

There are three versions of the Toshiba LAN cards.  Each supports both

of standard (thick) and thin Ethernet.  The cards are:

Type 1. Externally-powered Toshiba size card

with an AC adapter

without an AC adapter

Type 2. IBM XT compatible card

Type 3. Internally-powered Toshiba size card

This table identifies which card to use for each personal computer:

Computer    Type 1    Type 2    Type 3

----------------------------------------

T1200       yes

T1600       yes

T3100       yes

T3100/20    yes

T3100e                yes       yes

T3200                 yes

T5100       yes

T5200                 yes       yes

T8500                 yes       yes

Before you insert the adapter in your PC, set the swithes and the jumper

settings on the interface card properly.  The default I/O base address is

280H and the default interrupt request (IRQ) level is 9.  More detail

information is described in the Toshiba LAN Installation Manual.

[ Installing the Driver Software ]

After installing the 3Com/3Plus network software as specified in the 3Com

Installation and Configuration Guide, copy the Toshiba Ethernet driver file

TSBETH.SYS from this diskette to the \3DRIVERS directory with new name

called ETH.SYS as below:

CD C:\3DRIVERS

COPY  A:TSBETH.SYS  C:ETH.SYS

If you changed the I/O address and IRQ level, you also must to modify

the DEVICE=\3DRIVERS\ETH.SYS command line in your CONFIG.SYS file to reflect

them.  And if you want to change the default shared memory address and size

(32Kbytes from C800:0), you need to modify above command line. Use the format:

DEVICE = \3DRIVERS\ETH.SYS  IRQ-Level I/O-Base Memory-Base Memory-Size

where,

IRQ-Level   : IRQ level of Toshiba LAN card (in decimal), default is 9.

This number must be same with the jumper setting on the card.

I/O-Base    : I/O base address (in hexadecimal), default is 280.

This number must be same with the jumper setting on the card.

Memory-Base : Shared memory base segment address (in hexadecimal), default

is C800.  You must select Memory-Base not to conflict to other

ROMs, RAMs (esspecailly EMS).

Memory-Size : Shared memory size (in decimal, unit is KB), default is 32.

Memory-Size must be one of 64, 32, 16 (KB), and the larger size

selection gives the higher availablity.

Note that memory area must not across the 64K boundary.

For example, if you change IRQ to 3, I/O Base Address to 300H, Memory Base

Address to D000:0, and Memory Size to 64KB, device statement line should be:

DEVICE = \3DRIVERS\ETH.SYS  3 300 D000  64