TOSHIBA ETHERNET CARD DRIVER FOR USE WITH

NOVELL ADVANCED NETWARE AND SFT SOFTWARE

RELEASE NOTES

July 20, 1989

Copyright (c) 1989  Toshiba Corporation.

All Rights Reserved.

**********

* NOTICE *

**********

The ToshibaLAN drivers on this diskette are certified by Novell Inc. to work

with Advanced Netware, SFT Netware, and ELS Level II Versions 2.1/2.1x.

*) NetWare is a registered trademark of Novell, Inc.

*) IBM is a registered trademark of International Business Machine

Corporation

****************

* INTRODUCTION *

****************

This diskette contains ToshibaLAN Ethernet Card drivers for use with Novell

Advanced NetWare, SFT Netware, and ELS Level II Software Versions V2.1/V2.1X.

These drivers operate with the following ToshibaLAN Ethernet Cards.  These

LAN cards support both normal (thick) Ethernet and thin Ethernet.

Part Number	Description

-----------	-------------------------------------------------------------

PA5462U	Externally-powered Toshiba-size Ethernet Card for T1200,

T1600, T3100, T3100/20,	T3100e, T5100, T5200, and T8500.

(This product includes an AC adapter)

PA5463U	Externally-powered Toshiba-size Ethernet Card for T1200,

T1600, T3100, T3100/20,	T3100e, T5100, T5200, and T8500.

(Same card as PA5462, but this product does not	include an

AC adapter.  To use this card, you must separately purchase

an AC adapter, part number PA5701)

*** NOT AVAILABLE IN THE U.S.A. ***

PA5466U	IBM XT-compatible Ethernet Card for T3100e, T3200, T5200,

and T8500 (or for IBM half-size slot)

PA5467U	Internally-powered Toshiba-size Ethernet Card for T3100e,

T5200, and T8500

*****************************

* Contents of this Diskette *

*****************************

This diskette has a volume name "TSBETHER", and includes the following files:

+--------------+-----------------------------------+

| File Name    |  File Contents                    |

+--------------+-----------------------------------+

| README.DOC   |  This file.                       |

| TSBETHER.LAN |  The LAN driver description file. |

| TSBETHAD.OBJ |  The Server A driver object file. |

| TSBETHBD.OBJ |  The Server B driver object file. |

| TSBETHCD.OBJ |  The Server C driver object file. |

| TSBETHDD.OBJ |  The Server D driver object file. |

| TSBETHSH.OBJ |  The Shell driver object file.    |

+--------------+-----------------------------------+

******************

* How To Install *

******************

To install the driver software, copy the LAN driver description file,

TSBETHER.LAN, onto both Novell's "AUXGEN" and "SHGEN-1" diskettes.

Then follow the normal NETGEN and/or SHGEN procedures.

To use Toshiba Ethernet interface cards, select the following selection item

in the LAN driver selection menu:

Toshiba Ethernet Card  V1.00EC (890210)

In the LAN driver configuration menu, you must select the proper hardware

configuration depending on your personal computer type and the other

optional hardware.  There are 16 selections:

Configuration

Number      Hardware Configuration

-------------   -------------------------------------------------------

0         IRQ = 9, I/O Base = 280h, RAM at D000:0 for 64K, no DMA

1         IRQ = 5, I/O Base = 2A0h, RAM at D000:0 for 64K, no DMA

2         IRQ = 3, I/O Base = 2C0h, RAM at C000:0 for 64K, no DMA

3         IRQ = 9, I/O Base = 340h, RAM at C000:0 for 64K, no DMA

4         IRQ = 3, I/O Base = 2A0h, RAM at D800:0 for 32K, no DMA

5         IRQ = 5, I/O Base = 2C0h, RAM at D800:0 for 32K, no DMA

6         IRQ = 3, I/O Base = 280h, RAM at D000:0 for 32K, no DMA

7         IRQ = 9, I/O Base = 340h, RAM at D000:0 for 32K, no DMA

8         IRQ = 9, I/O Base = 280h, RAM at C800:0 for 32K, no DMA

9         IRQ = 3, I/O Base = 2A0h, RAM at C800:0 for 32K, no DMA

10         IRQ = 3, I/O Base = 2C0h, RAM at C000:0 for 32K, no DMA

11         IRQ = 4, I/O Base = 340h, RAM at C000:0 for 32K, no DMA

12         IRQ = 9, I/O Base = 2C0h, RAM at D800:0 for 16K, no DMA

13         IRQ = 3, I/O Base = 340h, RAM at D000:0 for 16K, no DMA

14         IRQ = 9, I/O Base = 280h, RAM at C800:0 for 16K, no DMA

15         IRQ = 3, I/O Base = 2A0h, RAM at C000:0 for 16K, no DMA

The following table designates the applicable hardware configurations

in the standard configuration (No additional hardware is used).

Toshiba

Computer 	Recommended Configuration Numbers

--------	------------------------------------------------

T1200		3, 8, 14

T1600		8, 14

T3100		0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 12, 13, 14, 15

T3100/20	0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 12, 13, 14, 15

T3100e		3, 8, 10, 14

T3200		8, 14

T5100		8, 9, 14

T5200		8, 14

T8500		0, 1, 4, 5, 6, 7, 8, 9, 12, 13, 14

+++++++++++++++++++++++++++++++++++++++++++++++++++

+	Before you install the Netware software on the 	+

+	server or workstation, set up the switches and 	+

+	jumpers on the ToshibaLAN card according to the +

+	hardware configuration you selected.		+

+++++++++++++++++++++++++++++++++++++++++++++++++++

******************************

* General Installation Notes *

******************************

- Memory address and size

For effective networking, configure your LAN card with the largest

amount of memory possibe.

If you use expanded memory (EMS), configure your LAN card memory

address	and size not to conflict with EMS memory RAM at D000:0

for 64K (Addresses D000:0-DFFF:0).

- IRQ's 3,4, and 5

Communication ports COM1, COM2, and COM3 often use these interrupts;

please refer to system setup for use of these interrupts.

****************************************

* Machine Specific Installation Notes *

****************************************

- T1200

The T1200 interprets IRQ 9 as IRQ 2.

To use IRQ3, you must disable the built-in modem in SETUP1.

IRQ 5 is used by the hard disk drive (for models T1200H and T1200HB).

I/O Address 2C0h is used by the real time clock (RTC).

- T1600

RAM at C000:0 for 32K is used by display ROM (addresses

C000:0-C7FF:0).

- T3100e

To use PA5467 with IRQ 3, the expansion slot must be set to

`Toshiba Modem' mode, in the setup option of TEST3. In the other

cases, the bus mode must be set to `Others' mode.

- T3200

RAM at C000:0 for 32K is used by display ROM (addresses

C000:0-C7FF:0).

- T5100

RAM at C000:0 for 32K is used by display ROM (addresses

C000:0-C7FF:0).

- T5200

RAM at C000:0 for 32K is used by display ROM (addresses

C000:0-C7FF:0).

- T8500

RAM at C000:0 for 32K is used by display ROM (addresses

C000:0-C7FF:0).	To use IRQ level 3, the machine configuration

must be changed	by setting the DIP switches in the front panel.

- Some Toshiba personal computers have two or more expansion slots

(Toshiba-size and/or IBM XT/AT slot(s)).  The following table

identifies which card to use for each computer.

+----------+---------+---------+---------+

|          | PA5462U | PA5466U | PA5467U |

|          | PA5463U |         |         |

+----------+---------+---------+---------+

| T1200    |    Ok   |         |         |

| T1600    |    Ok   |         |         |

| T3100    |    Ok   |         |         |

| T3100/20 |	Ok   |         |         |

| T3100e   |    Ok   |    Ok   |    Ok   |

| T3200    |         |    Ok   |         |

| T5100    |    Ok   |         |         |

| T5200    |    Ok   |    Ok   |    Ok   |

| T8500    |    Ok   |    Ok   |    Ok   |

+----------+---------+---------+---------+

In the T5200 and T8500, though the two types of LAN cards are

applicable, Toshiba recommends using the PA5467U for the first LAN

card.  If you use the T8500 or T5200 as a NetWare server, and need

more than one LAN card, you can use the PA5466U as the second LAN

interface card.

For more information about how to configure your ToshibaLAN card, please

refer to the "ToshibaLAN Installation Manual" and the "ToshibaLAN Driver

Installation Manual."