�����������������������������������������������������������������Ŀ

�		   NDIS DRIVER INSTALLATION FOR NE12 ADAPTER	       �

�������������������������������������������������������������������

The  NE12	NDIS  driver programs for DOS are developed according to

Microsoft's NDIS specification  version 2.0.1.  This driver belongs

to the Media Access Control (MAC) drivers,  which provide low-level

access to the NE12 network adapter. The following network operating

systems can use this MAC driver to operate the NE12 adapter  within

that environment:

�� MicroSoft LAN Manager

�� MicroSoft Windows For Workgroups

�� WIN/TCP PathWay Access for DOS

�� IBM LAN Support Program

�� DECnet PathWorks

�� 3COM 3+ Open

�� Banyan VINES

�� Any other Network Operating System which

comforms to the above listed specification.

A) Installing NDIS driver under MS LAN Manager

������������������������������������������������

Microsoft's LAN Manager software provides an installation utility

named SETUP.EXE, on the Setup diskette which allows users to install

additional options, such as new drivers, into LAN Manager. To install

this driver, follow the appropriate installation procedure below:

A.1) Enter the LAN Manager directory and create a "NE12" sub-

directory under the \DRIVERS\ETHERNET directory, as below:

C> CD\LANMAN.DOS\DRIVERS\ETHERNET

C> MD NE12

A.2) Insert the NE12 driver diskette into a floppy drive and

copy the NDIS files to the MS LAN Manager directories

on the hard disk, as shown:

NE12 Diskette	  ����> Workstation Hard Drive

������������������      ������������������������������

\NDIS\NE12.DOS	  ����> \LANMAN.DOS\DRIVERS\ETHERNET

\NE12\NE12.DOS

\NDIS\PROTOCOL.INI ����> \LANMAN.DOS\DRIVERS\ETHERNET

\NE12\PROTOCOL.INI

A.3) Use a plain-ASCII text editor to modify the CONFIG.SYS file

in the workstation's root directory:

DEVICE=<drive:\>LANMAN.DOS\DRIVERS\ETHERNET\NE12

\NE12.DOS

A.4) Use a plain-ASCII text editor to modify the contents of the

PROTOCOL.INI file in the \LANMAN.DOS\ directory as shown in

the example below:

[NETBEUI_XIF]

:

BINDINGS = "NE12.NIF"

:

;protocol.ini section for the NE12 card

[NE12.NIF]

DRIVERNAME = NE12$

IOBASE = 0x300

INTERRUPT = 3

A.5) Reboot the workstation.  The NDIS MAC driver will now be

loaded into the system automatically.

B) Installing NDIS driver under MS Windows For Workgroups

�����������������������������������������������������������

B.1) In the Network Group, choose the Network Setup icon.

B.2) In the Network Setup dialog box, choose the Driver button.

B.3) Choose the Add Adapter button.

B.4) In the list of network cards, select Unlisted or Updated Network

Adapter.

B.5) Insert this driver disk in drive A, enter drive and pathname A:\NDIS

B.6) After install the Adapter driver, PROTOCOL.INI will be modified for

the adapter.

B.7) The new driver does not take effect until you reboot your computer.

C) Installing NDIS driver under WIN/TCP PathWay Access for DOS

���������������������������������������������������������������

C.1) Insert the driver diskette into a floppy drive and copy

the files listed below into the directory \LANMAN\.

Driver diskette	  ����> LAN Manager Directory

����������������        ���������������������������

\NDIS\NE12.DOS	  ����> \LANMAN\NE12.DOS for NE12 card

\NDIS\PROTOCOL.INI ����> \LANMAN\PROTOCOL.INI

C.2) Add the following statement to the CONFIG.SYS file:

DEVICE = <drive:\>LANMAN\NE12.DOS

C.3) Add the following statements to the AUTOEXEC.BAT batch file:

NDIS -d:1

WINBIND

WINTCP

C.4) Reboot your computer.  The NE12 NDIS MAC driver will now

run under the WIN/TCP PathWay Access system.

D) Installing NDIS driver under IBM LAN Support Program

���������������������������������������������������������

D.1) Create a sub-directory named "NE12" under the directory

where your IBM LAN Support Program is located:

[LAN Support Program Dir]> MD NE12

D.2) Insert the NE12 driver diskette into a floppy drive and

copy the files into the newly created sub-directory under

the directory where your IBM LAN Support program is located:

Driver diskette ���> IBM LAN Support Program\NE12\Directory

����������������   ����������������������������������������

\NDIS\NE12.DOS	  ����> <drive:\path\>NE12\NE12.DOS

\NDIS\PROTOCOL.INI ����> <drive:\path\>NE12\PROTOCOL.INI

D.3) Change the original NDIS device driver statement in the

CONFIG.SYS to read:

DEVICE = <drive:\path\>NE12.DOS for NE12 card

D.4) Change the PROTOCOL.INI file so that it reads like the

example below:

BINDINGS = NE12.NIF

; protocol.ini section for the NE12 card

[NE12.NIF]

DRIVERNAME = NE12$

IOBASE = 0x300

INTERRUPT = 3

D.5) Reboot your computer.  The NE12 NDIS MAC driver will now

run under the IBM LAN Support system.

E) Installing NDIS driver under DECnet PathWorks

��������������������������������������������������

DECnet PathWorks provides a utility named NETSET.EXE to install the

NDIS driver.  Refer to DECnet's installation manual for the details.

After your DECnet PathWorks setup is completed,  you can modify the

existing CONFIG.SYS and PROTOCOL.INI files to use NE12 NDIS driver.

E.1) Create a sub-directory named "NE12" under the directory

where your DECnet PathWorks Program is located:

[DECnet PathWorks Program Dir]> MD NE12

E.2) Insert the NE12 driver diskette into a floppy drive and

copy the files into the newly created sub-directory under

the directory where your DECnet PathWorks program is located:

Driver diskette ��> DECnet Pathworks Program\NE12\Directory

����������������    ���������������������������������������

\NDIS\NE12.DOS	   ����> <drive:\path\>NE12\NE12.DOS

\NDIS\PROTOCOL.INI  ����> <drive:\path\>NE12\PROTOCOL.INI

E.3) Change the original NDIS device driver statement in the

CONFIG.SYS to:

DEVICE = <drive:\path\>NE12.DOS

E.4) Change the PROTOCOL.INI file so that it reads like the

example below:

; protocol.ini section for the NE12 card

[NE12.NIF]

DRIVERNAME = NE12$

IOBASE = 0x300

INTERRUPT = 3

[IPX4MAC]

:

BINDINGS = NE12.NIF

[DATALINK]

:

BINDINGS = NE12.NIF

:

E.5) Reboot your computer. The NE NDIS MAC driver will now run

under your DECnet PathWorks system.

F) Installing NDIS driver under 3Com 3+ Open

�����������������������������������������������

F.1) Enter the \3OPEN\DOSWKSTA\LANMAM\DRIVER\ sub-directory and

create a sub-directory named NE12 under it:

C> CD\3OPEN\DOSWKSTA\LANMAM\DRIVER

C> MD NE12

F.2) Insert the NE12 driver diskette into a floppy drive and

copy the NDIS files to the newly created sub-directory on

the hard disk:

NE12  Diskette  �> 3Com \3OPEN\DOSWKSTA\LANMAM\DRIVER\NE12\

���������������    �������������������������������������������

NDIS\NE12.DOS   �> 3OPEN\DOSWKSTA\LANMAM\DRIVER\NE12

\NE12.DOS

NDIS\PROTOCOL.INI �> 3OPEN\DOSWKSTA\LANMAM\DRIVER\NE12

\PROTOCOL.INI

F.3) Change the original NDIS device driver statement in the

CONFIG.SYS to:

DEVICE = c:\3open\doswksta\lanmam\driver\runtop\NE12.DOS

F.4) Change the PROTOCOL.INI file so that it reads like the

example below:

; protocol.ini section for the NE12 card

[NE12.NIF]

DRIVERNAME = NE12$

IOBASE = 0x300

INTERRUPT = 3

F.5) Reboot your computer.  The NE12 NDIS MAC driver will now

run under your 3+ Open system.

G) Installing NDIS.DOS under Banyan VINES

����������������������������������������������

G.1) Create a DOS bootable diskette and insert it in drive A:.

G.2) Login to your VINES network.

G.3) Run PCCOPY and then select "NDIS Ethernet" from the PCCOPY

menu. PCCOPY will copy the NDIS-specific files to the boot-

able diskette.

G.4) Modify the CONFIG.SYS file to include the following statements:

device = protman.dos

device = NE12.DOS for NE12 card

G.5) Change the PROTOCOL.INI file so that it resembles the example

shown below:

[PROTOCOL MANAGER]

drivername = PROTMAN$

[VINES_XIF]

drivername = NDISBAN$

BINDINGS = NE12.NIF

; protocol.ini section for the NE12 card

[NE12.NIF]

DRIVERNAME = NE12$

IOBASE = 0x300

INTERRUPT = 3

G.6) Run PCCONFIG and select "Network Card Setting".

G.7) Choose 'NDIS Ethernet' from the menu. enter the correct

hardware settings and save the new values. Exit PCCONFIG.

G.8) Reboot your computer. The NE12 NDIS MAC driver will now

run under your Banyan VINES system.

���������������������������������������������������������������Ŀ

�		Protocol.ini for the NE12 Ethernet adapter	     �

�����������������������������������������������������������������

The PROTOCOL.INI file is used to provide binding information and

driver configuration for NDIS drivers.  The following items just

show you the driver setting for NE12 adapters.  Typically, the

required operating parameters  will have been preset by way of

jumpers in the adapter.

The PROTOCOL.INI file may be edited with a plain-ASCII text editor

in order to change/add/delete any of the parameters listed in it.

The newly edited/added/deleted settings will not take effect until

the system is COLD booted (i.e. the system MUST be turned off and

back on again before the new settings will take effect).

DRIVERNAME :  This driver must be named to identify the NE12

network device driver.

type  : [ascii]

range : [NE12$]	 for NE12 card

IOBASE	 : Indicates the base I/O address for the network card.

type  : [hex]

range : [0x300,0x320,0x340,0x360] for NE12 card

INTERRUPT  : Indicates the  hardware interrupt  being used by the

network card.

type  : [decimal]

range : [2,3,4,5,10,11,12,15] for NE12 card

Example PROTOCOL.INI

������������������������������������������������������

Following is an example fragment from a PROTOCOL.INI file.  This

fragment lists the options described above.

; protocol.ini section for the NE12 card

[NE12.NIF]

DRIVERNAME = NE12$

IOBASE = 0x300

INTERRUPT = 3

================= FILE NDIS\INSTALL.DOC -- END OF FILE ================