NETWARE CLIENT 32 FOR WINDOWS 95

To install SMC ODI LAN driver during Setup:

1. Copy the driver to the \ENGLISH folder (or the install folder

you created) before installing Client 32.

To update a SMC LAN driver after installing Client 32:

1. Do either of the following:

* For 32-bit (*.LAN) drivers, copy the new driver to the client

computer's NOVELL\CLIENT32 folder.

* For a 16-bit (*.COM) driver, copy the new driver to the

client computer's NOVELL\NWCLIENT folder.

2. Follow the directions in the next section for installing

third-party LAN drivers.

3.  Installing 3rd-Party LAN Drivers

If you want to use an ODI LAN driver from a vendor other than

Novell, you need to specify the driver in the Client 32 Setup

initialization file.

If you have a newer driver than the one provided with the Client32

release, copy all the files from the ENGLISH folder on the

Client 32 CD-ROM to a Client 32 install folder on your hard

disk or on a network drive and copy the driver to this install

folder.

To install a third-party ODI LAN driver:

1. Before running Client 32 Setup, open NWSETUP.INI (in the

install folder) in a text editor.

2. Search for the .INF file that corresponds to LAN driver you want

to use. The .INF files for third-party LAN drivers are named

ODIxxxx.INF, where "xxxx" is the manufacturer of the network

adapter. For example, ODISMC.INF.

3. Delete the semicolon at the front of the line (DONE).

4. Save your changes and exit the editor.

5. Run Client 32 Setup.

Do Not Remove Novell ODINSUP

Do not remove Novell ODINSUP from the Network control panel,

unless you know the ODI adapter it is bound to. Removing Novell

ODINSUP also removes NetWare Client 32 for Windows 95 if

there is only one adapter in use.