STANDARD MICROSYSTEMS CORPORATION

DOS ODI Driver

For EtherCard PLUS Family LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1992 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  How to use EZStart to copy the DOS ODI driver from the

SuperDisk to your PC.

o  How to setup a SMC Ethernet workstation to communicate with

both Novell NetWare 386 and IBM AS400 networks concurrently.

COPYING THE DRIVER

From the EZStart Driver Installation screen, select 'Copy Files'

to copy all the files used by this driver from the SuperDisk to

the Destination Directory that you specify.

SETUP PROCEDURE

1. Copy the following files to your workstation:

LSL.COM

ODINSUP.COM

IPXODI.COM

NETX.COM

These files may be acquired from Novell Technical Support or

from the Novell section on CompuServe (type GO NOVELL at the

CompuServe prompt, currently the DOSUP7.ZIP download).  When

copying these files to your workstation, it does not matter

under which subdirectory you place them.  For convenience,

copy all these files to the same directory.  \LSP is the

subdirectory that we recommend.

2. Check to see that the IBM LAN Support Program has correctly

installed the following files in the \LSP subdirectory of

your workstation:

NETBIND.EXE

PROTMAN.DOS

DXMEOMOD.SYS

DXMAOMOD.SYS

If they have not been installed, copy them from the LAN Support

Program diskette to your workstation.  The DXMEOMOD.SYS and

DXMAOMOD.SYS files should be version 1.3 or higher.

3. Ensure that the following lines are correct in your system

NET.CFG under the Protocol and Link Driver sections:

Protocol ODINSUP

Bind SMC8416

Link Driver SMC8416

Frame Ethernet_802.3

Frame Ethernet_802.2

Frame Ethernet_II

Frame Ethernet_SNAP

Protocol IPX 0 Ethernet_802.3

You need all four frame types for the system to load properly.

The Protocol IPX line is only necessary when the 802.3 frame

type is not the first entry.

4. Using a text editor, open the PROTOCOL.INI file located on your

workstation.  Usually, this file is in the \LSP directory.

Make a backup of the current PROTOCOL.INI file and then edit

PROTOCOL.INI to add the following lines:

[PROTMAN_MOD]

DriverName = PROTMAN$

[DXMEO_NIF]

DriverName = DXMEO$

Bindings = SMC8416

5. Using a text editor, edit the CONFIG.SYS file on your

workstation to add the following lines:

device=\lsp\protman.dos

device=\lsp\dxma0mod.sys 001

device=\lsp\dxme0mod.sys

device=\lsp\eimpcs.sys

device=\lsp\ecyddx.sys files=8 drives=6

6. Using a text editor, edit the AUTOEXEC.BAT file on your

workstation to add the following lines:

cls

cd\ (add the directory name where SMC8416 resides)

lsl

SMC8416

ipxodi

odinsup

netbind

netx

f:

7. Reboot the workstation.

The workstation displays the F: prompt.

Your workstation can now communicate with both Novell NetWare 386

and AS400 networks.

SMC and Standard Microsystems are registered trademarks; and

EtherCard Elite 16 Ultra, EtherCard Elite 16T Ultra, EtherCard Elite

16C Ultra, Elite Series, EliteView, EZStart, SuperDisk, and UltraChip

are trademarks of Standard Microsystems Corporation.  Other products

and company names are registered trademarks or trademarks of their

respective holders.