STANDARD MICROSYSTEMS CORPORATION

Novell LAN WorkPlace v4.0 for DOS Driver

For Ethernet LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  How to use EZStart to copy the Novell LAN WorkPlace version 4.0

for DOS driver from the SuperDisk to your PC.

o  How to load the Novell LAN WorkPlace for DOS driver and

establish a network connection.

o  Error messages.

Note: This driver supports all 8216 (Ultra), 80xx (Elite), and

80xx (PLUS) Series ISA and Micro Channel Ethernet adapters.

Note: For custom installations involving multiple frame types, refer

to your Novell NetWare ODI Shell for DOS manual.

COPYING THE DRIVER FILES

From the EZStart Driver Installation screen, select 'Copy Files'

to copy all the files used by this driver from the SuperDisk to the

Destination Directory recommended by EZStart, or to a different

directory that you specify.

LOADING THE DRIVER

To install the driver:

1. If you copied the batch file SMCODI.BAT to the root directory of

your PC's hard disk, you can use any text editor to add the

following statement as the last line in your AUTOEXEC.BAT file:

CALL SMCODI

Alternatively, you can use any text editor to add the following

lines as the last lines in the workstation's AUTOEXEC.BAT file:

LSL.COM

SMC8000.COM

IPXODI.COM

NETX.EXE

F:LOGIN

CD \

2. Insert the LAN WorkPlace Version 4.0 for DOS Install disk into

the disk drive.

3. Move to the drive where you inserted the disk. Then type: INSTALL

4. Press the Enter key.

5. Select a target drive and press Enter.

6. When prompted whether you want to install the Windows application

portion of LAN WorkPlace, select either "Yes" or "No". Either

choice will not effect your adapter installation.

7. When prompted to gather IP information, such as IP address,

router address, and subnet mask, check with your System

Administrator to obtain this information.

8. Press Enter to continue.

9. Follow the screen prompts until you are asked whether you

want to update your system files. Refer to the information

you obtained in step 7, or to the "Installing and Configuring

the Software" section in your Novell LAN WorkPlace for DOS

user's guide.

10. When prompted as to whether you want to update your system files,

select "Yes". The program will automatically create a backup of

your DOS system files if your system files already exist.

11. Follow the screen prompts to copy the remaining LAN WorkPlace for

DOS files until you complete the installation.

12. After you complete the installation, exit to DOS.

13. Use any text editor to edit the NET.CFG file in the root

directory of your PC's hard drive. The following example shows

how the NET.CFG contents should look. Lines preceded with >

indicate text you must add to the NET.CFG file. The other lines

are preceded with #, which is a REMARK character; these lines

are not executed because they are preceded with #. Removing #

activates these lines the next time you boot your PC.

Note: In your NET.CFG file, the lines Link Support, Link Driver

SMC8000, and Protocol TCPIP must be left-justified. (Do not

precede them with the > character, as shown above. This

character is shown to let you know that you must type this

line in the NET.CFG file). All other lines must be indented.

In the following example, no values appear next to the

parameters in the Protocol TCPIP section. However, you will

see values next to the parameters in the Protocol TCPIP section

of your NET.CFG file. Do not change these values.

#

# NET.CFG File for SMC8000

#

> Link Support

>     Buffers 8 1586

>     MemPool 4096

Link Driver SMC8000

Frame Ethernet_802.3

>    Frame Ethernet_II

#    Port #1 280

#    Mem #1 000D0000 2000/10

#    Int #1 3

#    PS/2 Slot?

#

Protocol TCPIP

>    Bind         SMC8000

ip_address

ip_router

ip_netmask

tcp_sockets

udp_sockets

raw_sockets

nb_sessions

nb_commands

nb_adapter

nb_domain

14. Save the NET.CFG.

15. Reboot your computer.

ERROR MESSAGES

"Could not find a SMC Ethernet adapter"

The driver was unable to find an SMC Ethernet LAN adapter

installed in the system. Check for adapter conflicts and

retry the installation. Check that the adapter is seated

properly in the adapter slot.

"This Adapter must be configured in NET.CFG"

This message will appear if you install two adapters

in your system. In this case, you will need to add the

adapter configuration for each adapter in a 'NET.CFG' file.

Refer to the above information on installing multiple

adapters.

"Adapter hardware failure detected (initialization)"

The adapter did not pass the initialization process.

Check the adapter for configuration conflicts.

"Adapter hardware failure detected (self-test)"

The adapter did not pass the initialization self-test

process due to a hardware problem. Check the adapter

for configuration conflicts.

"Other system RAM already occupies adapter RAM"

The driver has detected that the adapter RAM conflicts with

other RAM in the system. Reconfigure the adapter to use

a different RAM address and retry the installation.

"Other system ROM already occupies adapter RAM"

The driver has detected that the adapter RAM conflicts with

ROM in the system. Reconfigure the adapter to use a

different RAM address and retry the installation.

"Adapter IRQ higher than 7 not valid in 8-bit slot"

The IRQ number specified cannot be used if the adapter

is inserted into an 8-bit slot. Select a different IRQ

number.

"I/O base address chosen for driver not valid (illegal

boundary)"

The I/O base address specified in the NET.CFG file cannot be

used because it does not start on supported address start

boundary. Change the setting in the NET.CFG file to a

supported adapter setting.

"I/O base address chosen for driver not valid (out of range)"

The I/O base address specified in the NET.CFG file cannot be

used because it is not within supported address range settings

of the adapter. Change the setting in the NET.CFG file to a

supported adapter setting.

"IRQ chosen for driver does not match adapter configuration"

The IRQ setting specified in the NET.CFG file cannot be

used because it does not match the adapter soft-configuration

or jumper setting found during initialization. Change the

setting in the NET.CFG file to a supported adapter setting.

"Adapter IRQ higher than 7 not valid in 8-bit slot"

The IRQ setting specified in the NET.CFG file cannot be

used because it is not valid for an adapter installed in an

8-bit slot. Change the setting in the NET.CFG file

to a supported adapter setting.

"IRQ chosen for adapter not valid (out of range)"

The IRQ setting specified in the NET.CFG file cannot be

used because it is not within the support IRQ settings

of the adapter. Change the setting in the NET.CFG file

to a supported adapter setting.

"Cannot access adapter RAM properly"

The adapter could read or write the adapter RAM. Verify that

another adapter does not share the same memory space as the

LAN adapter.

"Adapter RAM base address on illegal boundary"

The RAM base address setting specified in the NET.CFG file

cannot be used because it does not start on supported

RAM address start boundary. Change the setting in the

NET.CFG file to a supported adapter setting.

"RAM base address chosen for driver not valid (out of range)"

The RAM base address setting specified in the NET.CFG file

cannot be used because it does not start on supported

RAM address start boundary. Change the setting in the

NET.CFG file to a supported adapter setting.

"RAM size chosen for driver not valid (out of range)"

The RAM base address setting specified in the NET.CFG file

cannot be used because it is not within the support RAM

base address settings of the adapter. Change the setting

in the NET.CFG file to a supported adapter setting.

"RAM base address chosen for driver does not match adapter

configuration"

The RAM base address setting specified in the NET.CFG file

cannot be used because it does not match the adapter soft-

configuration or jumper setting found during initialization.

Change the setting in the NET.CFG file to a supported adapter

setting.

"RAM size chosen for driver does not match adapter

configuration"

The RAM size setting specified in the NET.CFG file cannot

be used because it does not match the adapter RAM size found

during initialization. Change the setting in the NET.CFG file

to a supported adapter setting.

"Configuration corrupted; run software setup"

The driver failed to initialize the adapter due to invalid

data in configuration EEROM. Run the software setup utility

and reconfigure the adapter.

SMC and Standard Microsystems are registered trademarks; and

EtherCard Elite 16 Ultra, EtherCard Elite 16T Ultra, EtherCard Elite

16C Ultra, Elite Series, EliteView, EZStart, SuperDisk, and UltraChip

are trademarks of Standard Microsystems Corporation.  Other products

and company names are registered trademarks or trademarks of their

respective holders.