STANDARD MICROSYSTEMS CORPORATION

Novell NetWare Lite v1.1 ODI Driver

For Ethernet LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  How to use EZStart to copy the Novell NetWare Lite version 1.1

ODI driver from the SuperDisk to your PC and install the driver.

o  How to install the driver.

o  Error messages.

Note: This driver supports all 8216 (Ultra), 80xx (Elite), and

80xx (PLUS) Series ISA and Micro Channel Ethernet adapters.

COPYING THE DRIVER FILES

From the EZStart Driver Installation screen, select 'Copy Files' to

copy all the files used by this driver from the SuperDisk to the

Destination Directory recommended by EZStart, or to a different

directory that you specify.

The EZStart default for copying files is a:\, as the Destination

Directory.  Please make a backup copy of your SuperDisk before EZStart

copies files to the SuperDisk to configure it as a "LAN Drivers

Diskette".

INSTALLING THE DRIVER FILES

1. Follow the installation instructions for installing NetWare Lite.

2. When prompted for a network interface card, select "Other".

3. When prompted for the "LAN Drivers Diskette",  insert the

SuperDisk.

4. Select "SMC8000.COM".

5. Follow the remaining Novell instructions to complete the

installation.

ERROR MESSAGES

"Could not find a SMC Ethernet adapter"

The driver was unable to find an SMC Ethernet LAN adapter

installed in the system. Check for adapter conflicts and

retry the installation. Check that the adapter is seated

properly in the adapter slot.

"This Adapter must be configured in NET.CFG"

This message will appear if you install two adapters

in your system. In this case, you will need to add the

adapter configuration for each adapter in a 'NET.CFG' file.

Refer to the above information on installing multiple

adapters.

"Adapter hardware failure detected (initialization)"

The adapter did not pass the initialization process.

Check the adapter for configuration conflicts.

"Adapter hardware failure detected (self-test)"

The adapter did not pass the initialization self-test

process due to a hardware problem. Check the adapter

for configuration conflicts.

"Other system RAM already occupies adapter RAM"

The driver has detected that the adapter RAM conflicts with

other RAM in the system. Reconfigure the adapter to use

a different RAM address and retry the installation.

"Other system ROM already occupies adapter RAM"

The driver has detected that the adapter RAM conflicts with

ROM in the system. Reconfigure the adapter to use a

different RAM address and retry the installation.

"Adapter IRQ higher than 7 not valid in 8-bit slot"

The IRQ number specified cannot be used if the adapter

is inserted into an 8-bit slot. Select a different IRQ

number.

"I/O base address chosen for driver not valid (illegal

boundary)"

The I/O base address specified in the NET.CFG file cannot be

used because it does not start on supported address start

boundary. Change the setting in the NET.CFG file to a

supported adapter setting.

"I/O base address chosen for driver not valid (out of range)"

The I/O base address specified in the NET.CFG file cannot be

used because it is not within supported address range settings

of the adapter. Change the setting in the NET.CFG file to a

supported adapter setting.

"IRQ chosen for driver does not match adapter configuration"

The IRQ setting specified in the NET.CFG file cannot be

used because it does not match the adapter soft-configuration

or jumper setting found during initialization. Change the

setting in the NET.CFG file to a supported adapter setting.

"Adapter IRQ higher than 7 not valid in 8-bit slot"

The IRQ setting specified in the NET.CFG file cannot be

used because it is not valid for an adapter installed in an

8-bit slot. Change the setting in the NET.CFG file

to a supported adapter setting.

"IRQ chosen for adapter not valid (out of range)"

The IRQ setting specified in the NET.CFG file cannot be

used because it is not within the support IRQ settings

of the adapter. Change the setting in the NET.CFG file

to a supported adapter setting.

"Cannot access adapter RAM properly"

The adapter could read or write the adapter RAM. Verify that

another adapter does not share the same memory space as the

LAN adapter.

"Adapter RAM base address on illegal boundary"

The RAM base address setting specified in the NET.CFG file

cannot be used because it does not start on supported

RAM address start boundary. Change the setting in the

NET.CFG file to a supported adapter setting.

"RAM base address chosen for driver not valid (out of range)"

The RAM base address setting specified in the NET.CFG file

cannot be used because it does not start on supported

RAM address start boundary. Change the setting in the

NET.CFG file to a supported adapter setting.

"RAM size chosen for driver not valid (out of range)"

The RAM base address setting specified in the NET.CFG file

cannot be used because it is not within the support RAM

base address settings of the adapter. Change the setting

in the NET.CFG file to a supported adapter setting.

"RAM base address chosen for driver does not match adapter

configuration"

The RAM base address setting specified in the NET.CFG file

cannot be used because it does not match the adapter soft-

configuration or jumper setting found during initialization.

Change the setting in the NET.CFG file to a supported adapter

setting.

"RAM size chosen for driver does not match adapter

configuration"

The RAM size setting specified in the NET.CFG file cannot

be used because it does not match the adapter RAM size found

during initialization. Change the setting in the NET.CFG file

to a supported adapter setting.

"Configuration corrupted; run software setup"

The driver failed to initialize the adapter due to invalid

data in configuration EEROM. Run the software setup utility

and reconfigure the adapter.

SMC and Standard Microsystems are registered trademarks; and

EtherCard Elite 16 Ultra, EtherCard Elite 16T Ultra, EtherCard Elite

16C Ultra, Elite Series, EliteView, EZStart, SuperDisk, and UltraChip

are trademarks of Standard Microsystems Corporation.  Other products

and company names are registered trademarks or trademarks of their

respective holders.