STANDARD MICROSYSTEMS CORPORATION

Banyan Vines

For Ethernet LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1994 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes how to:

o  How to use EZStart to copy the Banyan Vines driver files from

the SuperDisk to your PC.

o  Prepare your PC's operating system so you can attach to the

network.

Note: This driver supports all 8216 (Ultra), 80xx (Elite),

80xx (PLUS), and 8416 (EtherEZ) Series ISA, EISA, and Micro

Channel Ethernet adapters.

COPYING THE DRIVER FILES

From the Driver Installation screen, select 'Copy Files' to copy all

the files used by this driver from the SuperDisk to the Destination

Directory recommended by EZStart, or to a different directory that

you specify.

PREPARING YOUR OPERATING SYSTEM

The following example shows how to create a boot diskette for a

Banyan client workstation that uses an SMC EtherEZ LAN adapter and

SMC8000.DOS NDIS driver.

Note: You need to have an existing Banyan network operating

system running to perform this task.

1. Create a DOS boot diskette.

2. Using any text editor, add the following lines to your

CONFIG.SYS file:

DEVICE=A:\PROTMAN.DOS/I:A:\

DEVICE=A:\SMC8000.DOS

3. Make sure that the SMC NDIS driver file (SMC8000.DOS) and the

PROTOCOL.INI file are both copied to the boot diskette.

4. Run PCCOPY (a Vines utility) to copy the PC configuration

software and the LAN driver files from the VINES server to your

boot diskette. Specify drive A as your Destination Directory

when prompted by the operating system. Make sure to select

NDIS Ethernet as your LAN driver option.

5. Run PCCONFIG (another VINES utility) from the boot diskette

to configure your client PC for the network:

o  Select "Network Card Settings".

o  Press the Enter key.

o  Select "NDIS Ethernet".

o  Press Enter.

o  Enter the interrupt that the adapter is using.

o  Press Enter.

o  On the "PROTOCOL.INI bindings =" line, type the entry name

that matches the "Bindings=" statement in your PROTOCOL.INI

file.

o  Press Enter.

o  Press F10 to save your configuration.

o  Press Esc to exit the current screen.

6. Reboot your computer.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and EtherEZ,

EtherCard Elite 16 Ultra, EtherCard Elite 16T Ultra, EtherCard Elite

16C Ultra, EliteSeries, EZStart, and SuperDisk are trademarks of

Standard Microsystems Corporation.  Other product and company names

are registered trademarks or trademarks of their respective holders.