STANDARD MICROSYSTEMS CORPORATION

Wollongong PathWay Access v2.0 for DOS

For Ethernet LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1994 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  How to use EZStart to copy the Wollongong PathWay Access

version 2.0 for DOS NDIS driver files from the SuperDisk

to your PC.

o  How to prepare your PC's operating system so you can attach

to the network.

Note: This driver supports all 8216 (Ultra), 80xx (Elite),

80xx (PLUS), and 8416 (EtherEZ) Series ISA, EISA, and Micro

Channel Ethernet adapters.

COPYING THE DRIVER FILES

From the Driver Installation screen, select 'Copy Files' to copy all

the files used by this driver from the SuperDisk to the Destination

Directory recommended by EZStart, or to a different directory that you

specify.

PREPARING YOUR OPERATING SYSTEM

Use the following steps to install the SMC8000 driver to support

PathWay only:

1. Using any text editor, add the following three lines to your

CONFIG.SYS file:

device=c:\pathway\pwtcp.sys

device=c:\pathway\protman.exe/i:\<directory path>

device=c:\lanman\SMC8000.dos

In the above lines, "/i:\<directory path>" is optional and

specifies any valid directory containing PROTOCOL.INI (the

default directory is \LANMAN).

2. Create a PROTOCOL.INI file that contains the following lines:

[Protocol Manager]

DriverName = protman$

[SMC8000-NIF]

Drivename = SMC8000$

3. In your AUTOEXEC.BAT file, verify that you have a line similar

to the following:

NDIS -d:2

In the above line, the number "2" indicates that the [SMC8000-NIF]

section in the PROTOCOL.INI file is the second entry. If it was

the first entry, the above line would read:

NDIS -D"1

4. Proceed with the rest of the instructions in the Wollongong

PathWay Installation Guide.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and EtherEZ,

EtherCard Elite 16 Ultra, EtherCard Elite 16T Ultra, EtherCard Elite

16C Ultra, EliteSeries, EZStart, and SuperDisk are trademarks of

Standard Microsystems Corporation.  Other product and company names

are registered trademarks or trademarks of their respective holders.