STANDARD MICROSYSTEMS CORPORATION

DOS ODI Driver

For Ether 10/100 Fast Ethernet EISA Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1995 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  How to use EZStart to copy the DOS ODI driver from the

SuperDisk to your PC.

o  How to setup an Ethernet workstation to communicate with

both Novell NetWare 386 and IBM AS400 networks concurrently.

NOTE: Ensure that the line speed (10 Mbps vs. 100 Mbps) configured

for the adapter matches the speed of the hub to which it is

attached. It is important to set the adapter at the correct

line speed (10/100 Mbps) using the EISA Configuration Utility.

Refer to pages 3-3 and A-9 in the user guide for further

details.

COPYING THE DRIVER

From the EZStart Driver Installation screen, select 'Copy Files'

to copy all the files used by this driver from the SuperDisk to

the Destination Directory that you specify.

SETUP PROCEDURE

1. Copy the following files to your workstation:

LSL.COM       (provided on the SuperDisk)

ODINSUP.COM   (provided by Novell)

IPXODI.COM    (provided by Novell)

NETX.COM      (provided by Novell)

These files may be acquired from Novell Technical Support or

from the Novell section on CompuServe (type GO NOVELL at the

CompuServe prompt, currently the DOSUP9.ZIP download).  When

copying these files to your workstation, it does not matter

under which subdirectory you place them.  For convenience,

copy all these files to the same directory.  \LSP is the

subdirectory that we recommend.

2. Check to see that the IBM LAN Support Program has correctly

installed the following files in the \LSP subdirectory of

your workstation:

NETBIND.EXE

PROTMAN.DOS

DXMEOMOD.SYS

DXMAOMOD.SYS

If they have not been installed, copy them from the LAN Support

Program diskette to your workstation.  The DXMEOMOD.SYS and

DXMAOMOD.SYS files should be v1.3 or higher.

3. Ensure that the following lines are correct in your system

NET.CFG under the Protocol and Link Driver sections:

Protocol ODINSUP

Bind SMC9232

Link Driver SMC9232

Frame Ethernet_802.3

Frame Ethernet_802.2

Frame Ethernet_II

Frame Ethernet_SNAP

Protocol IPX 0 Ethernet_802.3

You need all four frame types for the system to load properly.

The Protocol IPX line is only necessary when the 802.3 frame

type is not the first entry.

4. Using a text editor, open the PROTOCOL.INI file located on your

workstation.  Usually, this file is in the \LSP directory.

Make a backup of the current PROTOCOL.INI file and then edit

PROTOCOL.INI to add the following lines:

[PROTMAN_MOD]

DriverName = PROTMAN$

[DXMEO_NIF]

DriverName = DXMEO$

Bindings = SMC9232

5. Using a text editor, edit the CONFIG.SYS file on your

workstation to add the following lines:

device=\lsp\protman.dos

device=\lsp\dxma0mod.sys 001

device=\lsp\dxme0mod.sys

device=\lsp\eimpcs.sys

device=\lsp\ecyddx.sys files=8 drives=6

6. Using a text editor, edit the AUTOEXEC.BAT file on your

workstation to add the following lines:

cls

cd\ (add the directory name where SMC9232 resides)

lsl

SMC9232

ipxodi

odinsup

netbind

netx

f:

7. Reboot the workstation.

The workstation displays the F: prompt.

Your workstation can now communicate with both Novell NetWare 386

and AS400 networks.

This commercial computer software and documentation is provided

with RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of

the Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge, NY  11788.

SMC and Standard Microsystems are registered trademarks; and

Ether 10/100, EZStart, and SuperDisk are trademarks of Standard

Microsystems Corporation. Other products and company names are

registered trademarks or trademarks of their respective companies.