STANDARD MICROSYSTEMS CORPORATION

Novell NetWare DOS ODI Workstation Driver

For Ether 10/100 Fast Ethernet EISA Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1995 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  How to use EZStart to copy the Novell NetWare DOS ODI

workstation driver from the SuperDisk to your PC.

o  How to load the Novell NetWare DOS ODI driver and establish a

network connection.

o  Information about installing multiple adapters.

o  Error messages.

Note: For custom installations involving multiple frame types,

refer to your Novell NetWare ODI Shell for DOS manual.

NOTE: Ensure that the line speed (10 Mbps vs. 100 Mbps) configured

for the adapter matches the speed of the hub to which it is

attached. It is important to set the adapter at the correct

line speed (10/100 Mbps) using the EISA Configuration Utility.

Refer to pages 3-3 and A-9 in the user guide for further

details.

USING LANALYZER FOR WINDOWS

The SMC DOS ODI client driver (SMC9232.COM) conforms to version 4.0

of the DOS ODI specification. To use any 4.0 DOS ODI driver with

Lanalyzer for Windows, a file provided on your SuperDisk, RXMONSTK.COM,

must be loaded before loading the driver:

Example

LSL

RXMONSTK

SMC9232

Another file provided on the SuperDisk, RXMONSTK.MSG is also required

to use Lanalyzer for Windows and must reside in the same directory as

RXMONSTK.COM.

When using EZStart to copy files used by the DOS ODI client driver

from the SuperDisk to a Destination Directory on the target client,

RXMONSTK.COM and RXMONSTK.MSG are automatically copied to the same

directory.

COPYING THE DRIVER FILES

From the EZStart Network Drivers and Documentation Selection

screen, select 'Ether 10/100 DOS ODI Driver'. Then, when the

Network Drivers and Documentation screen appears, select 'Copy

Files' to copy all the files used by this driver from the

SuperDisk to the Destination Directory recommended by EZStart, or

to a different directory that you specify.

SUPPLIED FILES

Files provided in the Novell NetWare DOS ODI Client Directory include:

o  INSTALL.DOC     This file.

o  RELEASE.DOC     Provides a description of updates to the driver.

o  SMC9232.COM      SMC9232 Novell NetWare ODI Client driver.

o  SMC9232.INS      SMC9232 Novell NetWare ODI Client installation file.

o  NET.CFG         Novell network configuration file for the driver.

o  LSL.COM         Needed for Novell NetWare ODI DOS protocol stack.

o  SMCODI.BAT      Batch file that automatically executes Client software.

o  RXMONSTK.COM      File required to run Lanalyzer for Windows.

o  RXMONSTK.MSG     File required to run Lanalyzer for Windows.

LOADING THE DRIVER

To install the driver:

1. Follow the instructions in the Novell NetWare ODI Shell for

DOS manual.

2. Use any text editor to add the following lines to the

workstation's AUTOEXEC.BAT file:

CD \DOS_ODI  (or path to DOS ODI driver files)

LSL.COM      (provided on the SMC SuperDisk)

SMC9232.COM  (provided on the SMC SuperDisk)

IPXODI.COM   (provided by Novell)

NETX.EXE     (provided by Novell)

F:LOGIN

CD \

INSTALLING MULTIPLE ADAPTERS

If two adapters are installed in the same workstation:

1. Use any text editor to add the following lines to the NET.CFG

file, located in the \DOS_ODI subdirectory on the SuperDisk

Driver Disk:

LINE DRIVER SMC9232

SLOT X

LINK DRIVER SMC9232

SLOT X

In the above lines, X is the slot number of the adapter, in

hexadecimal. The first adapter should be the one with the

lowest slot number.

2. Copy the NET.CFG file to the directory where the SMC9232.COM

DOS ODI driver is located.

ERROR MESSAGES

"Could not find an SMC Ethernet adapter"

The driver was unable to find an SMC Ethernet LAN adapter

installed in the system. Check for adapter conflicts and

retry the installation. Check that the adapter is seated

properly in the adapter slot.

"This Adapter must be configured in NET.CFG"

This message will appear if you install two adapters

in your system. In this case, you will need to add the

adapter configuration for each adapter in a 'NET.CFG' file.

Refer to the above information on installing multiple

adapters.

"Adapter hardware failure detected (initialization)"

The adapter did not pass the initialization process.

Check the adapter for configuration conflicts.

"Adapter hardware failure detected (self-test)"

The adapter did not pass the initialization self-test

process due to a hardware problem. Check the adapter

for configuration conflicts.

"Other system RAM already occupies adapter RAM"

The driver has detected that the adapter RAM conflicts with

other RAM in the system. Reconfigure the adapter to use

a different RAM address and retry the installation.

"Other system ROM already occupies adapter RAM. "

The driver has detected that the adapter RAM conflicts with

ROM in the system. Reconfigure the adapter to use a

different RAM address and retry the installation.

"Adapter IRQ higher than 7 not valid in 8-bit slot"

The IRQ number specified cannot be used if the adapter

is inserted into an 8-bit slot. Select a different IRQ

number.

"I/O base address chosen for driver not valid (illegal

boundary)"

The I/O base address specified in the NET.CFG file cannot be

used because it does not start on supported address start

boundary. Change the setting in the NET.CFG file to a

supported adapter setting.

"I/O base address chosen for driver not valid (out of range)"

The I/O base address specified in the NET.CFG file cannot be

used because it is not within supported address range settings

of the adapter. Change the setting in the NET.CFG file to a

supported adapter setting.

"IRQ chosen for driver does not match adapter configuration"

The IRQ setting specified in the NET.CFG file cannot be

used because it does not match the adapter soft-configuration

or jumper setting found during initialization. Change the

setting in the NET.CFG file to a supported adapter setting.

"Adapter IRQ higher than 7 not valid in 8-bit slot"

The IRQ setting specified in the NET.CFG file cannot be

used because it is not valid for an adapter installed in an

8-bit slot. Change the setting in the NET.CFG file

to a supported adapter setting.

"IRQ chosen for adapter not valid (out of range)"

The IRQ setting specified in the NET.CFG file cannot be

used because it is not within the support IRQ settings

of the adapter. Change the setting in the NET.CFG file

to a supported adapter setting.

"Cannot access adapter RAM properly"

The adapter could read or write the adapter RAM. Verify that

another adapter does not share the same memory space as the

LAN adapter.

"Adapter RAM base address on illegal boundary"

The RAM base address setting specified in the NET.CFG file

cannot be used because it does not start on supported

RAM address start boundary. Change the setting in the

NET.CFG file to a supported adapter setting.

"RAM base address chosen for driver not valid (out of range)"

The RAM base address setting specified in the NET.CFG file

cannot be used because it does not start on supported

RAM address start boundary. Change the setting in the

NET.CFG file to a supported adapter setting.

"RAM size chosen for driver not valid (out of range)"

The RAM base address setting specified in the NET.CFG file

cannot be used because it is not within the support RAM

base address settings of the adapter. Change the setting

in the NET.CFG file to a supported adapter setting.

"RAM base address chosen for driver does not match adapter

configuration"

The RAM base address setting specified in the NET.CFG file

cannot be used because it does not match the adapter soft-

configuration or jumper setting found during initialization.

Change the setting in the NET.CFG file to a supported adapter

setting.

"RAM size chosen for driver does not match adapter

configuration"

The RAM size setting specified in the NET.CFG file cannot

be used because it does not match the adapter RAM size found

during initialization. Change the setting in the NET.CFG file

to a supported adapter setting.

"Configuration corrupted; run software setup"

The driver failed to initialize the adapter due to invalid

data in configuration EEROM. Run the software setup utility

and reconfigure the adapter.

This commercial computer software and documentation is provided

with RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of

the Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge, NY  11788.

SMC and Standard Microsystems are registered trademarks; and

Ether 10/100, EZStart, and SuperDisk are trademarks of Standard

Microsystems Corporation. Other products and company names are

registered trademarks or trademarks of their respective companies.