STANDARD MICROSYSTEMS CORPORATION

EISA Configuration File

For Ether 10/100 Fast Ethernet EISA Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1995 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  How to use EZStart to copy the EISA configuration files from

the SuperDisk to your PC.

o  The EISA configuration file for configuring an EISA

bus-compatible computer for use with SMC Ether 10/100 Fast

Ethernet EISA Adapters

NOTE: Ensure that the line speed (10 Mbps vs. 100 Mbps) configured

for the adapter matches the speed of the hub to which it is

attached. It is important to set the adapter at the correct

line speed (10/100 Mbps) using the EISA Configuration Utility.

Refer to pages 3-3 and A-9 in the Ether 10/100 user guide for

further details. Refer also to the section in the EISA configuration

file !SMCA010.CFG (provided on this SuperDisk) titled

"FUNCTION="Line speed selection".

COPYING SUPERDISK FILES

From the EZStart Driver Installation screen, select 'Copy Files'

to copy all the files from the SuperDisk to the Destination

Directory recommended by EZStart, or to a different directory

that you specify.

COPYING THE EISA CONFIGURATION FILE (!SMCA010.CFG)

The EISA configuration file (!SMCA010.CFG) is located in the

root directory of the SuperDisk. You can use this file in one

of three ways:

1. When prompted by your EISA Configuration Utility for this file,

you can insert the SuperDisk and specify the path as A:\.

2. You can copy !SMCA010.CFG using EZStart to the same directory

where your EISA Configuration Utility (ECU) is located

(e.g. C:\ECU). Your ECU should default to this path when searching

for the file, but if not, when prompted by your ECU for this file,

you can specify the appropriate path (e.g. C:\ECU).

3. If you have used EZStart to copy all SuperDisk files to a

directory on your computer's hard disk, and you are running EZStart

from the hard disk and not from the SuperDisk (floppy drive),

you can use EZStart to copy the EISA configuration file to

A:\ to create an installation diskette. When prompted by your

ECU for this file, you can insert the installation diskette and

specify the path as A:\.

EISA CONFIGURATION FILE

!SMCA010.CFG        For SMC Ether 10/100 Fast Ethernet

EISA Adapters.

To configure your computer, follow the instructions provided in your

PC's User Guide.

This commercial computer software and documentation is provided

with RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of

the Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge, NY  11788.

SMC and Standard Microsystems are registered trademarks; and

Ether 10/100, EZStart, and SuperDisk are trademarks of Standard

Microsystems Corporation. Other products and company names are

registered trademarks or trademarks of their respective companies.