STANDARD MICROSYSTEMS CORPORATION

DOS 3.0 or greater EZSETUP Program

For SMC Ethernet 8-/16-Bit and LAN Adapters

USER GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1994-1996 Standard Microsystems Corporation.

All rights reserved.

This installation guide:

o  Explains how to install the EZSETUP program (EZSETUP.EXE)

from the SuperDisk.

o  Explains how to use EZSETUP.

EZSETUP

In addition to the EZStart configuration, diagnostics, and driver

installation utility, SMC also provides the EZSetup configuration

utility on the SuperDisk. The EZSetup utility is designed to facilitate

SMC adapter configuration in environments where multiple adapters are

to be installed using the same configuration, or in environments where

the complete functionality of EZStart (diagnostics, driver

installation, etc.) is not required.

SUPPORTED ADAPTERS

EZSETUP operates with SMC 8003/8013, 8115T, 8216T/C and

8416B/T/A Ethernet adapters:

o 8003EB  8-bit coaxial adapter configurable by software only

o 8003W   8-bit twisted pair adapter configurable by software only

o 8003EP  8-bit coaxial adapter configurable by software or jumpers

o 8003WP  8-bit twisted pair adapter configurable by software or jumpers

o 8013EP  16-bit coaxial adapter configurable by software or jumpers

o 8013W   16-bit twisted pair adapter configurable by software or jumpers

o 8013EW  16-bit coaxial and twisted pair adapter configurable by

software or jumpers

o 8013EPC 16-bit coaxial adapter configurable by software or jumpers

o 8013WC  16-bit twisted pair adapter configurable by software or jumpers

o 8013EWC 16-bit coaxial and twisted pair adapter configurable by

software or jumpers

o 8115T   16-bit token ring adapter configurable by software or jumpers

o 8216T   16-bit twisted pair adapter configurable by software or jumpers

o 8216C   16-bit coaxial adapter configurable by software or jumpers

o 8416B   16-bit coaxial adapter configurable by software

o 8416T   16-bit twisted pair adapter configurable by software

o 8416BA  16-bit coaxial and AUI adapter configurable by software

o 8416TA  16-bit twisted pair and AUI adapter configurable by software

o 8416BT  16-bit coaxial and twisted pair adapter configurable by software

o 8416BTA 16-bit coaxial, AUI and twisted pair adapter configurable

by software

These adapters contain non-volatile memory which eliminates most

of the jumper requirements of earlier adapters.  This allows

you to configure the adapter through software.

PROGRAM INSTALLATION AND EXECUTION

Hard Disk

1. Copy EZSETUP.EXE from the SuperDisk to a directory

on your PC's hard disk. You can use the Copy Files function of

EZStart to perform this task.

2. Change to the directory where you copied the EZSetup files.

3. Type: EZSetup

4. Press the Enter key.

Floppy Disk

If you prefer, you can use the following procedure to run EZSetup

from a floppy disk:

1. Insert the SuperDisk into a disk drive.

2. Change to the floppy drive where the SuperDisk is installed.

For example, if the disk is in drive A, type A: and press Enter.

3. From the DOS prompt, type EZSETUP and press Enter.

RUNNING EZSETUP

1. Interactive Method

Type EZSETUP and follow the instructions displayed on the screen.

When you have more than one adapter which is supported by this

program installed in your system, all adapters are listed by their

I/O Base addresses.

If you have more than one adapter, but you see only one I/O

Base address listed, your adapters are probably configured to the

same I/O Base address. Remove all the adapters except one from your

computer, then change each adapter's I/O Base address. You can then

add and configure the other adapters.

Refer to the adapter's user guide for further installation and

configuration instructions.

2. Command Line Method

The command line consists of three components: 1) the command,

EZSETUP, 2) the adapter identification, and 3) the configuration

parameters.

Specify the adapter identification only when you have more than

one adapter installed.  You can use either the I/O base address or

the node address.  To identify the adapter, the command for using

the node address is -node:x, where x is the adapter's 12-digit

node address.

Command Line Options:

Command       Description

==============================================================

-?            Displays all EZSETUP command line options.

-factory      Forces the adapter to the default factory-set

values for the SOFT jumper position, ignoring

the specific command options for these values

(I/O base address 240, IRQ 2 Disabled, RAM base

address CE000; For 8013 adapters, CC000).

-default      Forces the adapter to the following settings

when the jumper is set to SOFT: I/O base address

240, IRQ 2, RAM base address CE000 (CC000 for

8013 adapters).

-io:x         Identifies the current I/O base address of the

adapter you want to configure.  The factory

setting for the SOFT jumper location is 240.

-irq:x        Specifies the interrupt level (IRQ).  Possible

values for x are 2, 3, 4, 5, and 7.  For 8013

adapters, possible values are 2, 3, 4, 5, 7, 10,

11, and 15.  Also enables the IRQ specified.

-newio:x      Specifies a new I/O base address.  Acceptable

values for x are 200, 220, 240, 260, 280, 2A0,

2C0, 2E0, 300, 320, 340, 360, 380, 3A0, 3C0,

and 3E0(hex).

-node:x       Identifies the node address of the adapter you

want to configure.  x is the 12-digit node address

printed on the NODE ADDRESS label attached to the

adapter.  This command is an alternative to using

the /io command to identify the adapter.  Example:

0000C024D058.

-ram:x        Specifies the RAM base address.  Possible values

for x are any hexadecimal address that is a multiple

of 8K between 80000 - FE000.  For 8013 adapters, the

value has to be a multiple of 16K between 80000 -

FFE000. To specify address above 1 Megabytes (above

FC000), 6 digits of address has to be specified. See

your adapter's user installation guide.

-winsize:x    Specifies the size of RAM window in Kbytes. Possible

values for x are 8, 16, 32 and 64. This option is

currently only valid for 8416B/T/A.

-rom:x        Specifies the base address of the boot ROM.

Possible values for x are any hexadecimal address

between 80000 to FC000. The address must be on the

boundary of the ROM size.  See your adapter's user

guide for details.

-romsize:x    Specifies the size of the ROM window in Kbytes.

Possible values for x are 0 (ROM disabled), 16,

32, and 64.

-wait:x       Specifies whether or not to add wait states.

Possible values for x are y (yes) and n (no).

Most computers work with /wait set to NO.

Computers with fast I/O bus clock rates may

need /wait set to YES.  If the adapter fails

its diagnostic with /wait:n, set /wait to YES.

-netconn:x    Specifies which network connection to use.

Possible values for x are : 1 ( for Auto Media

Detect), 2 (for BNC or 10BaseT),

3 (for AUI or 10BaseT), or 4 (for Twisted Pair-

No Link Integrity). This option is valid for

8003WC, 8013WC, 8013EPC, 8013EWC, 8216T/C

and 8416 B/T/A adapters.

For 8115T, possible values for x are: 1 (for STP_4),

2 (for UTP_4), 3 (for STP_16) and 4 (for UTP_16).

-link_int:x   Specifies whether link integrity is enabled

disabled. Possible values for x are d (disabled)

or e (enabled). This options is valid only for

8013WP and 8013EW adapters.

The remaining options are valid only for 8416B/T/A adapters.

-pnpboot:x    Specifies whether the pnp boot bit is enabled or

disabled. Possible values for x are d (disabled)

or e (enabled).

-pnp:x        This option enables the PnP feature of the SMC adapter

at I/O address = x. This options is valid only for

8416B/T/A adapters.

-pnpactv      This option modifies the I/O address values in the

PnP resource string for all SMC PnP adapters. The

adapters will remain PnP enabled.

-nopnp        This option disables the PnP feature on all SMC PnP

adapters.

-pnpdefault   This option sets all PnP values to the default and will

enable PnP.

-pnpdefault:max This option sets all PnP values to the MAXIMUM possible

range of values and will enable PnP.

-pnpstatus    This option displays the status of the PnP values of all

PnP adapters in the machine.

-iomap:x      This option sets the SMC adapter at I/O address =x

to I/O Mapped Mode.

-memmap:x     This option sets the SMC adapter at I/O address =x

to Memory Mapped Mode.  Use -ram:xxxxx to specify a

new ram address.

The following is an example of a properly constructed command

line:

ezsetup -io:240 -newio:280 -irq:5 -ram:C0000 -rom:D0000

The command, ezsetup, starts the program. The adapter identi-

fication, -io:240, tells the system which adapter to

configure, and the remaining information tells the system

how to configure the adapter.  In this example, the I/O base

address is changed to 280, the IRQ level is changed to 5,

the RAM base address is changed to C0000, and the ROM base

address is changed to D0000.

To enable Plug and Play (PnP) on the adapter at I/O base 240(hex) use:

ezsetup -pnp:240

To disable Plug and Play (PnP) on the adapter at I/O base 240(hex) use:

ezsetup -nopnp:240

Make sure that the configuration you select for your network

software driver matches what you select in EZSETUP.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and EtherEZ,

EtherCard Elite 16 Ultra, EtherCard Elite 16T Ultra, EtherCard Elite

16C Ultra, EliteSeries, EZStart, and SuperDisk are trademarks of

Standard Microsystems Corporation.  Other product and company names

are registered trademarks or trademarks of their respective holders.