STANDARD MICROSYSTEMS CORPORATION

SuperDisk

For 8016C/PC Ethernet LAN Adapters

This software is licensed by SMC for use by its customers only.

Copyright (c) 1994 Standard Microsystems Corporation.

All rights reserved.

Version 1.3       6 October 1994

Compressed Driver File SMCD13.EXE

DISK INFORMATION

This SMC SuperDisk is designed to be used with SMC's EZStart Program,

which is on this SuperDisk.  EZStart configures and tests your adapter,

copies the files you need from this disk to your PC, and displays or

prints appropriate instruction files that are located on this disk.

SYSTEM REQUIREMENTS

To use EZStart, your system must meet the following minimum requirements:

o  MS-DOS or PC-DOS operating system, version 3.13 or later, which

is required to run the EZStart program

o  Card and socket services software -- compliant with PCMCIA

specification version 2.01 or higher -- must already be installed

on your computer. Otherwise, you cannot access your computer's

PCMCIA slot(s). For more information, refer to the SMC EliteCard

PCMCIA Ethernet Adapter User Guide.

o  To take advantage of EZStart's graphics features, a VGA or better

monitor; however, EZStart can operate in text mode on any monitor

o  A minimum amount of available hard disk space (typically 50KB)

for installing a single driver from the SuperDisk using EZStart

OR

At least 1.44MB of available hard disk space if you intend to

extract all of the drivers from the SuperDisk

Note: Refer to the "Installing A Network Driver" section later in

this document or the User Manual for more information on

copying driver files from the SuperDisk.

USING CARD AND SOCKET SERVICES SOFTWARE

Your SMC EliteCard PCMCIA Ethernet adapter is supplied with Card

and Socket Services (CSS) software. If your computer does not have

CSS software, or the version of CSS software predates version 2.00,

install the CSS software supplied with your SMC EliteCard PCMCIA

Ethernet adapter. Otherwise, when you run EZStart, a message tells

you that CSS has not been detected; EZStart then closes and returns

you to the DOS prompt.

The EliteCard PCMCIA Ethernet Adapter User Guide contains

instructions for installing the supplied CSS software.

Note: If you have a Compaq Concerto, use the CSS software supplied

with your computer, regardless of the version. Appendix A in

the EliteCard PCMCIA Ethernet Adapter User Guide contains

additional information about using your EliteCard on various

hardware platforms.

If you install the supplied CSS software, you will find that CSS

places statements in your CONFIG.SYS file that automatically load

the ATADRV.SYS and SRAMDRV.SYS drivers each time you boot your system.

If you will not be using a PCMCIA hard drive card or an SRAM memory

card, you can delete these drivers from your CONFIG.SYS file to make

more memory available.

Note: If you change system resources (such as disabling a serial or

parallel port and then enabling it at a later time), your

system may allocate the same resources to more than one port,

resulting in an IRQ or RAM conflict. If this occurs, remove

your SMC EliteCard PCMCIA adapter from the socket, then run

the CSALLOC /D command. For more information, refer to

Chapter 5 in the EliteCard PCMCIA Ethernet Adapter User Guide.

If you experience other problems connecting your laptop computer

to your network when using your EliteCard, the problem may be

caused by your computer BIOS. Contact your computer dealer to

see if a BIOS upgrade is available for your computer.

USING EZSTART

EZStart is an all-purpose utility program that simplifies the

following tasks:

Install card and socket services software compliant with

PCMCIA specification version 2.01 or higher, if this

software is not currently installed on your computer.

Install the network driver

Install and configure PC Agent/SNMP software, and view

network management information it collects

In addition, EZStart makes it easy to perform optional

diagnostic procedures:

Manually configure the adapter by selecting a value for

each configuration option on an individual basis

Test two or more adapters communicating with each other

over your actual network

LOADING EZSTART

EZStart can run from the diskette, or for faster operation it can be

copied onto a hard disk and run from there.

Note: To run EZStart from the hard disk, copy all the files from

the SuperDisk to your computer's hard disk.  However, make sure

that the compressed driver file, SMCD13.EXE, EZSTART.EXE, and

ADAPTER.EXE reside in the same directory.

To load EZStart, from the DOS command line, type:

[path]ezstart<Enter>

To load EZStart in Text Mode (instead of Graphics Mode), type

[path]ezstart/T<Enter>

To load EZStart from Microsoft Windows, select the File menu in

Program Manager.  When the Run dialog box appears, type:

[path]ezstart

in the Command Line box, then select the OK button.

INSTALLING A NETWORK DRIVER

EZStart accesses network driver files through the compressed

driver file, SMCD13.EXE.  The file name, SMCD13.EXE, specifies

the family of adapters supported ("D" denotes 8016C/PC PCMCIA

card), and the version of the SuperDisk ("13" denotes 1.3).

EZStart copies the network driver and any other supporting files

from the compressed driver file to a location compatible with

your network operating system.

Creating an Installation or Options Diskette:

If your network operating system requires an installation or

options floppy diskette, you can build it by following these steps:

Run EZStart from the hard disk as described above in the

section titled LOADING EZSTART.

If card and socket services is not already installed,

or if your computer has an installed version of card

and socket services with a version earlier than 2.01,

then exit EZStart to return to the DOS prompt, change

to the drive containing the SuperDisk, and type

"install". For more information, refer to the SMC

EliteCard PCMCIA Ethernet Adapter User Guide.

Note: To access your computer's PCMCIA slot(s), you must have

card and socket services software compliant with PCMCIA

specification version 2.01 or higher installed on your

computer.

Select the Drivers button from the EZStart main screen.

When prompted for the Destination Directory by the EZStart

Driver Installation window, specify a floppy drive

(e.g., A:\).

Insert a floppy disk into the specified drive and press

Enter. The driver and appropriate configuration files will

be copied to the floppy diskette.

The floppy disk can now be used as an installation or options

diskette. When configuring your networking software to use the SMC

driver, insert the floppy diskette when prompted to insert an

installation or options diskette.

Extracting All Driver Files to Hard Disk:

If you prefer, you can extract all of the network drivers

from the SuperDisk to your hard disk by:

Changing your default directory to the floppy drive where

the SuperDisk resides.

Typing SMCD13 <Destination Directory> at the DOS prompt

(where <Destination Directory> is the path where you want

the extracted SuperDisk contents to reside).

Note: There must be at least 1.44MB of available hard disk space.

Pressing Enter.

Drivers for many LAN operating systems and software utilities are

also available through EZStart. The following list describes them:

Utilities/     Contents

Drivers

DOS ODI        Open data-link interface driver to be used with

ODI protocol stacks.

EZSTART        Documentation for the EZStart Program.

MS LAN         (.DOS and .OS2) Microsoft LAN Manager drivers.

MANAGER

NDIS           DOS and OS/2 NDIS drivers to be used with NDIS-

compliant network operating systems and protocol

stacks. Subdirectories contain documentation and

.INI files for specific network operating systems.

PCAGENT        Documentation and driver for SMC's PC Agent/SNMP

network-management utility.

In addition to the driver or utility program itself, each

driver has the following associated files:

o  An INSTALL.DOC or other .DOC file that explains how to install

and use the software; and

o  A RELEASE.DOC file that states the current version, release date,

and any improvements made in this version of the software.

SUPPORTED ADAPTERS

The software on this SuperDisk operates with the SMC 8016C/PC

PCMCIA Ethernet LAN adapter.

TECHNICAL SUPPORT

For technical support, first contact your place of purchase. If you

need additional assistance, call our Technical Support departments:

New York (8:30 am - 6:00 pm Eastern Time)

(800) 992-4762  (from U.S.A. and Canada)

(516) 435-6250

(516) 434-9314  (Fax)

California (7:00 am - 5:00 pm Pacific Time)

(800) 638-5323  (from U.S.A. and Canada)

(714) 707-5607

(714) 707-2491  (Fax)

SMC offers Bulletin Board Services (BBS) from which you can obtain

the latest information on adapters and drivers for SMC products via

modem:

California    (714) 707-2481

9600 baud, 8 data bits, no parity, 1 stop bit

New York      (516) 434-3162

9600 baud, 8 data bits, no parity, 1 stop bit

Germany       49 (89) 928806-50

2400 (V.22bis) or 4800 (V.32bis) baud, 8 data

bits, no parity, 1 stop bit

France        33 (1) 39.73.57.00

1200 to 14K baud, 8 data bits, no parity, 1 stop bit

United        44 (0) 932 350681

Kingdom       2400 or 19.2 (HST only), 8 data bits, no parity,

1 stop bit

INTERNET Support:  SMC Technical Support can also be obtained through

the INTERNET. Our address is:  techsupt@ccmail.west.smc.com

CompuServe Support:  For information and drivers for SMC network

products, you can also access the SMC Forum through CompuServe. Log

into CompuServe, and at the "!" prompt type "GO SMC".

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and

EliteCard, Elite Series, EZStart, SuperDisk, UltraChip, and UMAC/LMAC

are trademarks of Standard Microsystems Corporation.  Other products

and company names are registered trademarks or trademarks of their

respective companies.