STANDARD MICROSYSTEMS CORPORATION

IBM LAN Server v3.0 OS/2 Driver

For TokenCard EliteSeries LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  How to use EZStart to copy the IBM LAN server driver from the

SuperDisk to your PC.

o  How to install the IBM LAN server driver.

COPYING THE DRIVER FILES

From the EZStart Driver Installation screen, select 'Copy Files'

to copy all the files used by this driver from the SuperDisk to the

Destination Directory recommended by EZStart, or to a different

directory that you specify.

The EZStart default Destination Directory is C:\NDIS. Please make

a backup copy of your SuperDisk before EZStart copies files to the

SuperDisk to create an "option" diskette.

INSTALLING THE DRIVER

1. Insert the SMC or IBM LAN Adapter Protocol Support (LAPS)

Version 1.00 diskette into your computer's disk drive.

2. Type: LAPS

3. Press the Enter key.

The IBM logo appears.

4. Select the "INSTALL" button.

5. Specify the target drive (usually C:\) and press Enter or

select "OK".

6. Select the Configure button.

7. Select "Configure LAN transports" followed by "Continue".

The Configure Workstation screen appears.

8. From the Network Adapters window, select the "SMC Token-Ring

Adapter for OS/2" followed by the "Add" button.

This adapter name appears in the Current Configuration window.

9. From the Protocols window, select "IBM IEEE 802.2" followed by

the "Add" button. Then select "IBM OS/2 NETBIOS" followed by

"Add". (You may also need to select the "IBM NetWare Requester"

option and "Add", depending on your network.)

Each time you select the "Add" button, the selected option

appears in the Current Configuration window, below the chosen

driver.

10. Select "OK".

11. Select "Exit".

The CONFIG.SYS Update screen appears.

12. Specify the drive where the CONFIG.SYS file resides.

13. Select the "Continue" button.

14. Select "OK".

15. Select "Exit".

16. Reboot your computer. Use the OS/2 Shutdown process to exit from

the program in an orderly fashion.

This completes the installation procedure.

SMC and Standard Microsystems are registered trademarks; and

TokenCard, TokenCard Elite, TokenCard Elite/A, Elite Series,

EliteView, EZStart, and SuperDisk are trademarks of Standard

Microsystems Corporation.  Other products and company names are

registered trademarks or trademarks of their respective holders.