STANDARD MICROSYSTEMS CORPORATION

NDIS Drivers

For TokenCard EliteSeries LAN Adapters

RELEASE INFORMATION

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

Version 1.07                   14 January 1994

VERSION HISTORY

-- Version 1.07 (Current Version) --

Fixed known 4Mbps LMAC ring insertion problem.

-- Version 1.06 --

Known Problem

This version will not load in an IBM Value Point.

-- Version 1.00

Initial release.  Features include ReceiveLookahead architecture for high

performance, support for two adapters with a single driver, and UMAC/

LMAC features for improved maintenance and quick support of new

products.

Known Problems

NDIS Driver:

Summary:  NDIS driver fails to initialize the second  adapter

correctly in DOS and OS/2 1.3 environments.

In both cases the driver fails to bind to the second

adapter correctly.

Advice:   Refer to the sample PROTOCOl.INI file in the NDIS

directory of the SMC SuperDisk Drivers Diskette

LAN Support Program (LSP) for DOS and LAN Adapter Protocol

Support (LAPS) Program for OS/2:

Summary:  When using LAPS or LSP to configure two adapters

in the system, generates a "DriverName=SMC81002$"

in the PROTOCOL.INI entry for the second card instead of

"DriverName=SMC8102$".

Advice:   Manually edit the PROTOCOL.INI file to change "81002"

to "8102".

Summary:  When updating using LAPS or LSP, if you have the correct

statement "DEVICE=TRINIT.EXE (.SYS)" in the CONFIG.SYS

file, the program may suggest that you remove that statement.

Advice:   Make sure that the "DEVICE=TRINIT.EXE (.SYS)" statement

is located above the "DEVICE=PROTMAN" statement in the

CONFIG.SYS file.

Licensed by Standard Microsystems for use by its customers only.

TokenCard, TokenCard Elite and SuperDisk are trademarks, and SMC and Standard

Microsystems are registered trademarks of Standard Microsystems Corporation.

Other trademarks mentioned herein belong to their respective companies.