XIRCOM TOKEN RING ADAPTER

Additional Information

SmartRing Software Version 3.12

September 8, 1992

README.DOC Updated February 15, 1994

========================================================================

Welcome to a new level of connectivity convenience.  The Xircom Token

Ring Adapters are the easiest to use token ring adapters available.

Be sure to fill out and return your Product Registration Card and record

your model and serial number on page ii of the Xircom User's Guide.

The following information may not be contained in the User's Guide.

This README.DOC was updated on February 15, 1994, to incorporate

additional notes for users of Xircom's 4Mbps Pocket Token Ring Adapters,

models PT04BT and PT04B3.  See "PT04 Notes" following the "Utilities"

section of this document for specific details.

GENERAL

-------

This release of SmartRing(tm) software supports all Xircom Token Ring

Adapters:  Pocket Token Ring Adapter, External Token Ring Adapter, and

Pocket Token Ring Adapter II.

ENHANCEMENTS

------------

The enhancements and features of this release are:

1)   Enhanced Parallel Port (EPP) capabilities in all DOS drivers.

2)   Support for IBM OS/2 version 2.00

3)   Sign-on messages for DOS SmartRing(tm) drivers displaying parallel

port mode (non-bidirectional, bidirectional, or enhanced).

4)   TRMLID.COM (ODI driver) -  "max frame size" parameter specification

is supported for NET.CFG.

5)   TRSETUP.EXE - includes auto media sensing capability for External

Token Ring Adapters, and may now be loaded as a device driver

in CONFIG.SYS, as well as being run from the command line.

UTILITIES

---------

New utilities include:

1)   TRSETUP.OS2 - setup utility for OS/2 with same functionality as

TRSETUP.EXE, used as device driver.

2)   EPP.SYS and EPP.COM - utilities for enabling/disabling EPP mode in

EPP-capable machines.

PT04 NOTES

----------

This release of SmartRing Software is being provided to users of Xircom's

4Mbps Pocket Token Ring Adapters to allow added functionality and bug

fixes.  This release fixes the following problem:

1)  When using Windows, machines no longer hang upon exit and re-entry

into Windows.

The following items are known limitations of this software.

1)	  When using NetWare 4.0 or 4.01, use of the VLM.EXE shell (rather

than NETX.EXE) is not supported.

2)   The DOS ODI driver (TRMLID.COM) does not support promiscuous mode

for use with network analyzer products such as Novell's LANalyzer.

3)   IBM ThinkPad models 700 and 720 are only supported at 25mhz

speed.

4)   OS/2 2.1 is not supported with SMARTND.OS2.  A beta driver is

available for use with OS/2 2.1.  Contact Technical Support for

further information.

The following are workarounds that can be used with this release:

1)   NEC UltraLite Versa:

If your Pocket Token Ring Adapter gives an EEPROM failure, type

MODE LPT1=COM1.  If the adapter then works, add the MODE LPT1=COM1

command to either your AUTOEXEC.BAT batch file, or the batch file

you use to start your network.

2)   IBM ThinkPads 700 and 720 (with 25/50 mhz processor speed):

Slow down processor speed to 25 mhz in the ThinkPad setup.

MICROSOFT WINDOWS(tm) 3.1 NOTES

-------------------------------

If you are using Microsoft Windows(tm) version 3.1 with SMARTIPX, you

will receive error messages stating that you need to use a newer version

of Novell's IPX.  A subsequent version of SMARTIPX will contain updated

IPX code.  The ODI driver, TRMLID, will operate correctly in this

environment.  See the User's Guide for installation instructions for the

ODI driver.

MICROSOFT LAN MANAGER NOTES

---------------------------

If you are using Microsoft LAN Manager version 2.0 or 2.1, the

directory structure of the SmartRing Software diskettes has been

modified to allow you to use LAN Manager's setup utility for easier

installation.  The instructions for installation are as follows:

LAN MANAGER VERSION 2.1 FOR DOS AND OS/2

----------------------------------------

1.      Begin installation of LAN Manager 2.1 using "setup."

2.      When prompted to select the available network adapter

driver, choose "Other Driver" at the bottom of the

screen.

3.      When prompted, insert the SmartRing Software diskette

labeled "NDIS Drivers" and press Enter.

4.      Select "Xircom Token Ring Adapter" from the menu

5.      Continue with the installation until it is completed.

6.      Reboot your PC.

LAN MANAGER VERSION 2.0 FOR DOS AND OS/2

----------------------------------------

1.      Begin installation of LAN Manager 2.0 using "setup."

At the "Import Network Drivers" screen, choose YES.

2.      Insert the SmartRing Software diskette labeled "NDIS

Drivers" and press Enter.

3.      Select "Xircom Token Ring Adapter" from the list of

drivers to import.

4.      Continue with the installation.

5.      At the Network Drivers menu, select "Xircom Token Ring

Adapter" from the list of drivers to install.

6.      Continue with the installation until it is complete.

7.      Reboot your PC.

NDIS NOTES

----------

If you are using the NDIS driver for DOS (SMARTND.DOS), it is recommended

to modify your PROTOCOL.INI file to include the following parameter in

the [XIRCDOS_NIF] section:

MaxFrameSize = 2048

Refer to your User's Guide for specific information on installing and

configuring the SMARTND.DOS driver.

OS/2 NOTES

----------

If you are using OS/2 version 1.3 or earlier, use the SMARTND.OS2 driver

located in the \NDIS_OS2 subdirectory, and the remaining configuration

files from the \NDIS_OS2\V_1X subdirectory (such as the .NIF file and

other MDGDLC files).

Enhanced Parallel Port (EPP) operation is not supported in this release

of SMARTND.OS2.  An EPP-capable version of SMARTND.OS2, along with the

EPP.OS2 utility program, is available on the Xircom INFOSYS Bulletin

Board System (BBS) at the number given at the end of this README.DOC.

See the User's Guide for information on using the BBS.

Parallel port mode is not displayed by SMARTND.OS2 in this release.  A

version of SMARTND.OS2, which displays parallel port mode

(non-bidirectional, bidirectional, or enhanced), is available on the BBS.

Local (pass-through) printing with the External Token Ring Adapter is

not supported under OS/2 at this time.

OS/2 INSTALLATION (VERSION 2.00)

--------------------------------

1)   Configure OS/2 2.00 for an IBM 16/4 Token Ring adapter.

2)   Copy SMARTND.OS2, XIRTOK.NIF to subdirectory containing

IBMTOK.OS2, IBMTOK.NIF (\IBMCOM\MACS).

3)   Edit CONFIG.SYS as follows:

a)   Change the line:   DEVICE=C:\IBMCOM\MACS\IBMTOK.OS2

to read:   DEVICE=C:\IBMCOM\MACS\SMARTND.OS2

4)   Edit PROTOCOL.INI in \IBMCOM subdirectory as follows:

a)   Change all references of:   IBMTOK_NIF

to:   XIRTOK_NIF

- under the '[IBMLXCFG]' section,

change:  IBMTOK_nif = IBMTOK.nif

to:  XIRTOK_nif = XIRTOK.nif

- under the '[LANDD_nif]'section(if it exists),

change:  Bindings = IBMTOK_nif

to:  Bindings = XIRTOK_nif

- under the '[NETBEUI_nif]'section(if it exists),

change:  Bindings = IBMTOK_nif

to:  Bindings = XIRTOK_nif

b)   Add the following section to the end:

[XIRTOK_NIF]

Drivername = SMARTND$

OS2V2COMPATIBLE = YES

AutoOpen = NO

EPP INSTALLATION

----------------

The DOS drivers on this disk support personal computers (PCs) that

have Enhanced Parallel Port (EPP) capability.  This high-speed parallel

port technology has been incorporated into Intel's 25Mhz SL chip

set in the 82360SL I/O controller.

To enable EPP mode on an EPP-compatible computer, the EPP.COM or EPP.SYS

file must be loaded prior to loading a Xircom EPP-capable network driver.

During loading, the network device driver will report that the driver is

in ENHANCED MODE, except for SMARTND.OS2.  If you are not certain that

your PC supports EPP, you may load EPP.SYS or EPP.COM.  The EPP utility

will display a message reporting that ENHANCED PARALLEL PORT HARDWARE

HAS BEEN IDENTIFIED or THIS COMPUTER IS NOT EQUIPPED WITH ENHANCED

PARALLEL PORT HARDWARE.

MACHINE SPECIFICS

-----------------

The following Enhanced Parallel Port (EPP)-capable machines have been

tested as of the time of this release.  This list will change as new

machines with EPP capability become available.

Compaq Contura

Compaq LTE Lite/20 & Lite/25**

Dell 325 NC

Everex Carrier SL/25

GRiD 1660

IBM N45 SL

NCR 3170 Safari

Zenith MastersPort 386SLe

Zenith Z-Note 320 and 325

** To enable EPP on the Compaq Lite, first use the machine's

SETUP program to change the default LPT port from LPT1 to LPT2.

If you are using any Compaq machine with SMARTND.DOS and the Xircom

Parallel Port Multiplexor (PPX), the machine's parallel port will come

up in non-bidirectional mode.  A beta version of SMARTND.DOS is available

on the Xircom BBS which allows bi-directional operation.

When using a Mitsuba Ninja or Packard Bell 286 LPV under NetWare,

Interrupt 7A must be disabled by creating a SHELL.CFG file containing the

line:  INT7A=OFF

TECHNICAL SUPPORT

-----------------

If you have any other questions or problems or would like to verify the

latest version of software, please contact Xircom Technical Support

between the hours of 8:00 a.m. and 4:30 p.m. (Pacific Time in the U.S.).

Current drivers are available on Xircom's electronic Bulletin Board

System.  Refer to the Preface in your User's Guide for details.

XIRCOM (U.S.A.)

TEL: (800) 874-4428 inside the United States

TEL: (818) 878-7600 direct

BBS: (818) 878-7618 (24-hour data line)

XIRCOM EUROPE

TEL: +32/(0) 3 326 34 94

BBS: +32/(0) 3 326 23 68 (24-hour data line)

From other areas, contact your local reseller or distributor.

Enjoy your Xircom Token Ring Adapter!

===============================[  END  ]================================