Packet Driver

-------------

The ZX312 packet driver is loaded using the following parameters:

ZX312 [-ndw] <pktint> [U] [bus=X] [dev=X] [media=<media>]

The optional switch parameters preceded by a dash effect the following:

-n: Novell IPX frames are slightly changed to maintain compatibility

with some drivers in Netware 2.X file servers.

-d: Delayed driver initialization on the first bind request.

-w: Windows compatibility, used if software using the packet driver is

run in a DOS-box under Windows 3.X.

The <pktint> parameter is the only required parameter and represents the

software interrupt to be used for packet driver-to-protocol communication.

Permissible values are 0x60 through 0x6F.

The 'bus=' and 'dev=' parameters are required in installations with more

than one ZX312 and select a specific adapter board to be used.

The optional 'media=' parameter selects a specific media type to be

used. The driver defaults to automatic media detection. Valid media types

are BNC, TP, TPFD, AUI, or AUTO. TPFD will select full duplex operation

in connection with a full duplex twisted pair Ethernet hub.

Specifying the <pktint> parameter followed by the option 'U' will attempt

to unload the packet driver. Successful unloading is not possible under

all circumstances.