H P S C R E E N  3.2                                    page 1

����������������������������������������������������������������������������

(C) Copyright Hewlett-Packard Company 1993

Table of Contents                                       page

1.           INTRODUCTION                                 3

1.1.         General Information                          3

1.2.         What you should know                         4

1.2.1.       EGA and CGA User                             4

1.2.2.       Windows User                                 4

1.2.3.       DOS 4.0                                      4

1.2.4.       43 Row Mode                                  4

1.2.5.       Requirements                                 4

1.2.6.       Printer Mode                                 4

1.2.7.       DOS GRAPHICS Command Conflict                5

2.           INSTALLATION                                 5

2.1.         Installation on a flexible disk              5

2.2.         Installation on hard disk                    5

3.           OPERATION                                    6

3.1.         Starting HPSCREEN                            6

3.2.         Screendump                                   7

4.           CONTROLLING HPSCREEN                         8

4.1.         Main Menu                                    8

4.2.         Submenu HPSCREEN                             9

4.3.         Submenu Printer                              9

4.4.         Submenu Printer Output                       9

4.5.         Submenu Display Mode                        10

4.6.         Submenu Paper Size                          11

4.7.         Submenu Aspect Ratio                        11

4.8.         Submenu Print Size                          12

4.9.         Submenu Orientation                         12

4.10.        Submenu Background                          13

4.11.        Submenu Color                               14

4.12.        Submenu Gamma Correction                    15

4.13.        Submenu Top Offset                          16

4.14.        Submenu Left Offset                         16

4.15.        Submenu Hardcopy Spacing                    16

4.16.        Submenu Symbolset                           17

4.17.        Submenu Font                                17

4.18.        Submenu Hardcopy Eject                      17

4.19.        Submenu Hardcopies                          17

4.20.        Save Configuration                          18

4.21.        Load Configuration                          19

4.22.        Batch Processing with HPSCREEN              19

����������������������������������������������������������������������������

H P S C R E E N  3.2                                    page 2

����������������������������������������������������������������������������

Table of Contents                                       page

5.           DISPLAY SPECIALS                            20

5.1.         General                                     20

5.2.         VGA                                         20

5.3.         EGA                                         20

5.4.         Hercules                                    20

6.           PRINTER SPECIALS                            21

6.1.         General                                     21

6.2.         LaserJet                                    21

6.3.         DeskJet                                     21

6.4.         PaintJet                                    21

6.5.         QuietJet                                    21

6.6.         ThinkJet                                    21

6.7.         Ruggedwriter                                22

7.           TECHNICAL INFORMATION                       22

7.1.         General                                     22

7.2.         Display Modes                               22

7.3.         Ordered Dithering                           25

7.4.         Funny Patterns                              25

8.           PALETTE                                     25

9.           Changing the hotkey                         25

����������������������������������������������������������������������������

H P S C R E E N  3.2                                    page 3

����������������������������������������������������������������������������

1.  INTRODUCTION

1.1.  General Information

HPSCREEN is a software utility which supports screendumps from HP

Vectra, IBM PC/XT/AT/PS2 and 100% Vectra compatibles.  HPSCREEN is

a memory resident program.  Its memory need is 43KByte.  The standard

versions of the following display adaptors are supported:

XGA  (Extended Video Graphics Array)

VGA  (Video Graphics Array)

EGA  (Enhanced Graphics Adaptor)

MCGA (Multi Color Graphics Adaptor)

CGA  (Color Graphics Adaptor)

MDA  (Monochrome Display Adaptor)

HGC  (Hercules Graphics Adaptor)

MMC  (Multi Mode Card)

The following printers are supported:

LaserJet Series II, IID, IIP, III, IIID, IIIP IIISi, 4, 4Si

LaserJet, LaserJet+, Laserjet 500+ and LaserJet 2000

DeskJet ,DeskJet+, DeskJet 500, DeskJet 500c and DeskJet 1200c

PaintJet, PaintJet XL, PaintJet XL300

QuietJet and QuietJet+

ThinkJet

RuggedWriter

Features of HPSCREEN 3.2:

Automatic detection of the installed display adaptor

Automatic detection of the current display mode

Hardcopies of various display adaptors

Variable sizing and positioning of the hardcopy on paper

Rescaled hardcopies

Rotation of the hardcopy

Various designs of the hardcopy

Color hardcopies on HP PaintJet/XL, DeskJet 500C, DeskJet 1200C

Printing colors by using gray shades on black and white printers

80 (instead of 77) columns for a text hardcopy

"Hot key" menu driven

Customized Hotkey

This software is not guaranteed to function in every application or

with non-standard hardware and software.  This utility should not be

used with similar utilities obtained elsewhere.  Hewlett-Packard does

not assume any warranty or liability whatsoever.

Trade Marks

The following companies own trade marks mentioned in this document:

IBM INTERNATIONAL BUSINESS MACHINES, MICROSFT, HERCULES

����������������������������������������������������������������������������

H P S C R E E N  3.2                                    page 4

����������������������������������������������������������������������������

1.2.    What you should know

1.2.1   Graphic Program User

If you want to use a graphic program we recommend to configure

HPSCREEN in the following way:

BEFORE starting the graphic program from the DOS level call the

HPSCREEN menu by the "hot key", configure HPSCREEN, leave the

HPSCREEN menu and then start your application program.

If you call the HPSCREEN menu DURING a graphic program session, it

may occur that your screen will not be restored properly after

leaving the HPSCREEN menu.

1.2.2.  Windows User    (Windows 3.0 must be in real mode!)

If you want to use the "hot key" menu during the window session, you

have to start HPSCREEN with the following parameter on the

commandline:

HPSCREEN /WIN #��

Without this parameter you cannot popup HPSCREEN during a window

session.  If you have installed HPSCREEN with the /WIN parameter you

can popup while running a window application but you MAY NOT use the

mouse while the "hot key" menu is active!  If you do so, first the

HPSCREEN menu will be destroyed, then the top of your window screen

will also be destroyed. Your mouse pointer will also shown twice.

1.2.3.  DOS 4.0 and DOS 5.0

DOS 4.0 and higher stores all keystrokes during a screendump

operation. Therefore you can not cancel the screendump by <ESC>.

1.2.4. 43 Row Mode

HPSCREEN does not support the 43 row mode. Do not use the "hot key"

menu.

1.2.5  Requirements

To use HPSCREEN you need a HP Vectra, IBM PC/XT/AT/PS2 or 100%

compatible computers, DOS version 2.0 or later.

1.2.6. Printer Mode

Some HP printers understand two types of printer control languages

(PCL resp. Alternate Mode). HPSCREEN supports ONLY the PCL mode.

Therefore you have to configure your printer as a PCL printer. Please

refer to your printer manual.

����������������������������������������������������������������������������

H P S C R E E N  3.2                                    page 5

����������������������������������������������������������������������������

1.2.7. DOS GRAPHICS Command Conflict

The HPSCREEN utility replaces the function of the DOS GRAPHICS

command. The GRAPHICS command does not work on the HP printers.

If the GRAPHICS command is executed after HPSCREEN utility is

loaded, it will disable the utility.

2.  INSTALLATION

2.1.  Working copy

First make a working copy of the HPSCREEN disk supplied.  For help,

follow the instructions in your DOS manual.  Make sure you work only

with the working copy.

2.2.  Installing HPSCREEN

HPSCREEN disk contains an installation program which executes all the

required installation operations.

2.2.1. Installation on a flexible disk

- Insert the HPSCREEN disk in drive A:

- Type A:  and press <Enter>

- Type Install and press <Enter>

- Follow the instructions of the installation program

The installation program copies HPSCREEN.EXE and PALETTE.EXE into the

subdirectory of your choice.  The installation program also provides

the possibility to print the manual on every HP printer in PCL mode.

2.2.2.  Installation on the hard disk

We recommend to use a subdirectory for HPSCREEN ; i.e.,

C:\HPSCREEN, on the hard disk.

- Insert the HPSCREEN disk in drive A:

- Type A: and press <Enter>

- Type Install and press <Enter>

- Follow the instructions of the installation program

The installation program copies HPSCREEN.EXE and PALETTE.EXE into the

desired subdirectory.  The installation program also provides the

possibility to print the manual on every HP printer in PCL mode.

����������������������������������������������������������������������������

H P S C R E E N  3.2                                    page 6

����������������������������������������������������������������������������

3.  OPERATION

3.1 Starting HPSCREEN

Start HPSCREEN by typing in at the DOS level:

HPSCREEN [configurationfile] [/WIN] #��

The parameters in square brackets [] are optional.

Option          Explanation

configuration-  is a file created by the function

file            "Save Configuration" (refer to 4.19. to 4.21.).

The parameters saved in this file are transferred

to HPSCREEN. HPSCREEN is now configured as the

configuration file.

The default filename is HPSCREEN.CFG. HPSCREEN

automatically checks the current directory for

this file and loads it.

/WIN            this option is necessary to popup the HPSCREEN menu

during a window session.  Please refer to 1.2.2.

/HOTKEY:35      this option is necessary to use another keystroke

combination instead of the default <Alt><+>.

Please refer to 9.

Example 1

HPSCREEN #��

HPSCREEN scans in the current directory for the file HPSCREEN.CFG.

If the file exists the saved configuration in HPSCREEN will be

tranferred to HPSCREEN.

Example 2

HPSCREEN MY-CONF.CFG #��

HPSCREEN loads the configuration of the file MY-CONF.CFG.

����������������������������������������������������������������������������

H P S C R E E N  3.2                                    page 7

����������������������������������������������������������������������������

3.2. Screendump

To start the screendump press

<Shift PrtSc>   or  <PrtSc>  on IBM PS2 systems.

HPSCREEN will become active and makes a short beep to announce that

it will start the screendump.  HPSCREEN will read the data from the

screen, will process it and then send the data to the printer.

If you make a full scale screendump of a graphic screen on a LaserJet

printer it may take time up to 20 minutes to transfer all the data.

Because HPSCREEN has to calculate before printing, it may happen that

the data transfer starts after some seconds (1 to 5).

Press the <Esc> key on your computer if you wish to stop your

printer before your screen image is completly printed. The Esc key

stops the utility from sending screen data to your printer. Printing

will stop when the data that has already been sent is finished

printing.

����������������������������������������������������������������������������

H P S C R E E N  3.2                                    page 8

����������������������������������������������������������������������������

4. CONTROLLING HPSCREEN

The data sent to the printer depends on the configuration of

HPSCREEN.  To control the configuration, HPSCREEN provides a "Hot

Key" menu.  The possibility to configure HPSCREEN allows you to

design your hardcopy.

4.1.  Main Menu

The menu for controlling HPSCREEN can be invoked by simultaneously

pressing the keys

<ALT> and <+> (plus key in the numeric keyblock)

If you have defined a different keystroke by using option

/HOTKEY: press

<ALT> and <Your Choice>

You may not call the HPSCREEN menu during the period when your

software draws its screen contents. Wait until your screen is drawn.

It is recommended to invoke the menu only at the DOS level.

���������������� HPSCREEN  Release 3.20 �������������Ŀ

�HPSCREEN        Active                               �

�Printer         LaserJet Series III, IIIP, IIID      �

�Printer Output  LPT1:                                �

�Display Mode    Automatic                            �

�Paper Size      A4                                   �

�Aspect Ratio    No Rescaling  Pixel 1:1              �

�Print Size      Default                              �

�Orientation     Portrait                             �

�Background      Normal                               �

�Color           Disabled                             �

�Gamma Correct.  Gamma 1  No Correction               �

�Top Offset      Default                              �

�Left Offset     Default                              �

�Spacing         Default                              �

�Symbolset       Default                              �

�Form Feed       Yes                                  �

�Copies          [ 1] Copies                          �

�������������������������������������������������������

## move bar   Select by #��   Exit window by Esc

S/L Save/Load Configuration

The screendump can be canceled during operation by pressing Esc

If you want to change the settings, select the corresponding

submenus.  Use the cursor keys to move the highlighted bar up and

down.  The marked option is selected by pressing <Enter>.  The

selected submenu will then appear.

����������������������������������������������������������������������������

H P S C R E E N  3.2                                    page 9

����������������������������������������������������������������������������

4.2. Submenu HPSCREEN

Option         �Explanation

��������������������������������������������������������������������

Active         �HPSCREEN is activated

�

Inactive       �HPSCREEN is deactivated and remains in memory

4.3. Submenu Printer

Option                  �Explanation

��������������������������������������������������������������������

LaserJet 4, 4D, 4Si     �HPSCREEN takes into account the various

LaserJet III, IIIP IIID �HPSCREEN takes into account the various

LaserJet II, IIP, IID   �features of the printers. For this reason

LaserJet 2000           �not all functions are applicable or make

LaserJet, 500, +        �sense for every printer (i.e. color printing

DeskJet, DeskJet+       �with LaserJet).

DeskJet 500             �

DeskJet 500c            �

DeskJet 1200c           �

PaintJet                �

PaintJet XL             �

PaintJet XL 300         �

QuietJet                �

QuietJet+               �

ThinkJet                �

RuggedWriter            �

4.4. Submenu Printer Output

Option         �Explanation

��������������������������������������������������������������������

LPT1:          �HPSCREEN will be only active on the selected

LPT2:          �printer port.

LPT3:          �

COM1:          �

COM2:          �

COM3:          �

File:          �HPSCREEN will send all its output to a named file

�instead of a selcted printer port.

�If you select this option you will be prompted

�to enter a filename. The filename must be a

�valid DOS filename.  HPSCREEN will beep when

�the screendump is completed.

�

�After the file has been created, the file can be

�printed by typing in at the DOS level:

�

�COPY /B FILENAME PRN

�

�You cannot save the HPSCREEN menu itself into a file.

�

����������������������������������������������������������������������������

H P S C R E E N  3.2                                    page 10

����������������������������������������������������������������������������

4.5. Submenu Display Mode

Most of the display adaptors can identify different display modes,

text modes and graphics modes e.g..  If you have a VGA display

adaptor, HPSCREEN automatically detects the right display mode for

generating the printout.  Usually this also works for the other

display adaptors.

But for technical reasons there are a few cases where HPSCREEN is not

able to find the right display mode.  In these cases, you can enter

the correct mode manually.

If your printout is wrong because of the above reason it is necessary

to leave your application program, and call the HPSCREEN menu at the

DOS level, then change the display mode to the right one, then

restart your application program and to start the screendump again.

Please refer to section 7.  There are the different display modes

explained.

Option         �Explanation

��������������������������������������������������������������������

Automatic      �HPSCREEN will automatically find (if possible)

�which display mode is active.

�

Other          �HPSCREEN will use the selected mode for the

�screendump

�

�

�Example 1:

�

�EGA user and mode 16 (640 x 350, 16 colors)

�

�Call the HPSCREEN menu at the DOS level!

�Select mode 16

�Leave the HPSCREEN menu

�Start your application program

�Start the screendump

�Leave your application program

�Call the HPSCREEN menu at the DOS level!

�Select "Automatic" to reset HPSCREEN

�

�Example 2:

�

�Hercules user and display page 1

�

�Call the HPSCREEN menu at the DOS level!

�Select mode 5, page 1

�Leave the HPSCREEN menu

�Start your application program

�Start the screendump

�Leave your application program

�Call the HPSCREEN menu at the DOS level!

�Select "Automatic" to reset HPSCREEN

�

����������������������������������������������������������������������������

H P S C R E E N  3.2                                    page 11

����������������������������������������������������������������������������

4.6. Submenu Paper Size

Specify the paper size present in the HP printer. This option

is only valid for HP LaserJet 2000, QuietJet+ and PaintJet XL.

Option         �Explanation

��������������������������������������������������������������������

A4             �A4 paper size

�

A3             �A3 Paper size

�Orientation of the paper:

�

�LaserJet 2000   QuietJet+

�PaintJet XL

�����Ŀ          ������Ŀ

��    �          �      �

��    �          ��������

�������

4.7. Submenu Aspect Ratio

All graphics consist of a pattern of pixels.  The pixel size of the

screen (except 3 VGA modes) are different compared to the pixel size

of the HP printers.  If you convert each pixel one to one, you obtain

a different size and shape of the graphic.  As a result, graphics in

the pixel-for-pixel (Not Rescaled) mode will be shown about 20%

smaller (in the vertical axis) on the HP printer.

Option         �Explanation

��������������������������������������������������������������������

No Rescaling   �The graphics pixels are printed one to one.

(Pixel 1:1)    �Circles may be distorted into ellipses.

�

�

Rescaling      �The varying graphics resolutions of the

�screen and the printer are compensated.

�Circles remain circles.  This will be done by

�duplicating dots.  It may happen that patterns

�are changed by this method.

����������������������������������������������������������������������������

H P S C R E E N  3.2                                    page 12

����������������������������������������������������������������������������

4.8. Submenu Print Size

Use the submenu "Print Size" when you want to reduce or enlarge the

size of your printout.  The minimum value is 30 mm.  The maximum

value depends on the paper size, orientation and the printer.  All

values are in mm.  You can define either the "Print Width" or either

the "Print Height".  If you have selected e.g.  "Print Width" the

"Print Height" is calculated automatically.  You can not define both

values independently.

The default value depends on the screen mode and the printer you

have selected.

Option         �Explanation

��������������������������������������������������������������������

Default        �HPSCREEN selects a default size.

�

Print Width    �You are asked for the print width.

�

Print Height   �You are prompted for the print height.

4.9. Submenu Orientation

Use Orientation to orient text and graphics on the page.

���������Ŀ

�   **    � Screen

�  ****   �

�����������

Option         �Explanation

��������������������������������������������������������������������

Portrait 0�    ������Ŀ     Text and graphics are printed

�� **  �     in portrait orientation.

��**** �

��     �

��������

�

Landscape 90�  ����������Ŀ Text (if possible) and graphics

��   **    � are printed in landscape orientation.

��  ****   �

������������

�

Portrait 180�  ������Ŀ     Only graphics are printed in portrait

��     �     180� orientation.

�� ****�

��  ** �

��������

�

Landscape 270� ����������Ŀ Only graphics are printed in landscape

��   ****  � 270� orientation.

��    **   �

������������

����������������������������������������������������������������������������

H P S C R E E N  3.2                                    page 13

����������������������������������������������������������������������������

4.10. Submenu Background

Use background "Reversed" to reverse the foreground and background

colors on your printed output compared to the colors displayed on

your monitor.  For example, if your monitor displays white lines on a

black screen, use "Reversed" to print black lines on a white page.

Use Background color "Normal" to print the screen as you see it.

Background "Reversed/Normal" affects graphics output only.  All other

colors remain unchanged.

The background of text modes cannot be printed.

���������Ŀ

���������۳ Screen picture

���������۳

�����������

Option         �Explanation

��������������������������������������������������������������������

Reversed       �The darkest screen color is reversed with the

�lightest screen color.

����������Ŀ

�� � � � � � printout

�� � � � � �

������������

�

Normal         �The screen is printed with the colors as you see them

����������Ŀ

����������۳ printout

����������۳

������������

����������������������������������������������������������������������������

H P S C R E E N  3.2                                    page 14

����������������������������������������������������������������������������

4.11. Submenu Color

This submenu is only valid for HP PaintJet and PaintJet XL.  The

other printers are black and white printers, they cannot print

colors.

Option         �Explanation

��������������������������������������������������������������������

Disabled       �Select "Color Disabled" if you want black and

�white output.

�

Enabled        �Select "Color Enabled" if you want color output.

�

User Defined   �The colors of the following display modes can be

�redefined.

�

�EGA and    Colors   Vertical    Horizontal

�VGA Mode            Resolution  Resolution

�    13      16      200         320

�    14      16      200         640

�    16      16      350         640

�

�These display modes have 16 different logical colors

�(numbers). Logical colors, because they will be

�assigned to physical (visual) colors later.

�HPSCREEN uses a default assignment of logical

�and physical colors.

�In case HPSCREEN can not read the right colors

�for technical reasons (EGA problem), the user has the

�possibility to select the colors with the help of

�this menu. The PaintJet printer has a possible

�330 different colors. You can view the colors by

�using the following steps:

�

�PaintJet:    Start the printer and at the same time

�(Selftest)   press the key "Linefeed". This starts

�             the selftest which prints the 330 colors.

�

�PaintJet XL: Start the program:

�             PALETTE.EXE #��

�             This program has the same function as

�             the PaintJet selftest.

�

�The colors are divided into rows and columns, and are

�defined by two numbers.

�

�Example:

�If you want to assign to the screen color 3 the

�printer color yellow (15,4) you have to select

�color 3:

�The cursor moves to the field "Row".

�Press 15 #��

�The cursor moves to the field "Column".

�Enter 4  #��

�

�As a result, the screen color 3 will be printed

�yellow on the PaintJet printer.

�

����������������������������������������������������������������������������

H P S C R E E N  3.2                                    page 15

����������������������������������������������������������������������������

4.12. Submenu Gamma Correction

Color perception is sensitive to intensity, which is non-linear

by nature. Perceptual correctness can be significantly improved

by an approximation technique called "gamma correction".

This option is useful for vga mode 13 (256 colors) only.

Option                   �Explanation

������������������������������������������������������������������������������

Gamma 1  No Correction   �No correction. For plain colors.

�

Gamma 2  (lighter)       �The non-linearity will be compensated.

Gamme 3  (lighter)       �The printed image becomes lighter.

�Depending on your image and your paper

�gamma 2 resp. 3 improves your printout.

�

����������������������������������������������������������������������������

H P S C R E E N  3.2                                    page 16

����������������������������������������������������������������������������

4.13. Submenu Top Offset

Use the submenu "Top Offset" to set the vertical offset from the left

top corner of the page to the origin of your printout.  The minimum

value depends on the orientation and the printer.  All values are in

mm. This menu is not available for all printers.

Option         �Explanation

��������������������������������������������������������������������

Default        �HPSCREEN selects a default value.

�

Top Offset     �You are prompted for the top offset.

4.14. Submenu Left Offset

Use the submenu "Left Offset" to set the horizontal offset from the

left top corner of the page to the origin of your printout.  The

minimum value depends on the orientation and the printer.  All

values are in mm. This menu is not available for all printers.

Option         �Explanation

��������������������������������������������������������������������

Default        �HPSCREEN selects a default value.

�

Left Offset    �You are prompted for the left offset.

4.15. Submenu Hardcopy Spacing

This submenu is important for LaserJet printers only.

The LaserJet printers print with 10 cpi, meaning that only 77

characters per line can be printed.  You have the option, with 10.5

cpi, of slightly lessening characters spacing so that 80 characters

can be printed per line.

Option         �Explanation

��������������������������������������������������������������������

Default        �The hardcopy is printed using the current

�character spacing.

�

10.5 cpi       �Any settings in the printer are retained.

�Proportional typefaces are then printed in

�10.5 cpi spacing.

Limitations:

The line draw symbols have been conceived for character spacing 10

cpi.  Because of this, displacements may arise if other cpi values

are used.

����������������������������������������������������������������������������

H P S C R E E N  3.2                                    page 17

����������������������������������������������������������������������������

4.16. Submenu Symbolset

Usually your text screen is displayed in the PC8 symbolset.

When you start the screendump your printer may be configured to any

other symbolset. In this case some of the characters are printed out

wrongly. To avoid this, select symbolset "PC8". Then the printer is

programmed to the PC8 symbolset.

Option         �Explanation

��������������������������������������������������������������������

Default        �The symbolset in the printer is not changed.

PC8            �The symbolset is turned to PC8

PC8/DKN        �The symbolset is turned to PC8 / DKN

�              (PC8 symbolset for Denmark and Norway)

Roman8         �The symbolset is turned to Roman8

4.17. Submenu Font

Use submenu "Font" when you want a text hardcopy with a desired

font. HPSCREEN will print the text on your screen using the

selected font

Option         �Explanation

��������������������������������������������������������������������

Default        �The font in the printer is not changed.

Courier 12pt   �The font Courier 12pt 10cpi is selected.

Courier 10pt   �The font Courier 10pt 12cpi is selected.

4.18. Submenu Hardcopy Eject

Use Hardcopy Eject after printing to advance paper to a clean page or

to continue printing on the same page.  Using Hardcopy Eject No

allows you to print multiple screens on one page.

Option         �Explanation

��������������������������������������������������������������������

Yes            �Paper will be ejected.

Yes Network    �A network command will be used to indicate

�end of file.

No             �Paper remains in the LaserJet.

4.19. Submenu Hardcopy Copies

Use the menu "Hardcopy Copies" to specify how many copies of each

page you want your printer to print. After you select "Hardcopy

Copies" you are asked for a number. The maximum value is "99".

"Hardcopy Copies" turns your laser printer into a combination of

printer and copy machine. Regardless of how long it takes to

generate the first page of printed output, subsequent copies of

the same page will be printed at the printer's maximum speed.

����������������������������������������������������������������������������

H P S C R E E N  3.2                                    page 18

����������������������������������������������������������������������������

4.20. Save Configuration

HPSCREEN is configured when your specific settings have been

assigned to all options.

With "Save Configuration" you can store the current settings.

There is no limitation to the number of configuration files

you can save.

After you have selected the "Save Configuration" menu, HPSCREEN

will prompt you for the filename.

!

It may happen that the configuration will be saved only when the

HPSCREEN menu has been exited.

Example:

You want to save a configuration file with the option settings

listed below.

���������������� HPSCREEN  Release 3.20 �������������Ŀ

�HPSCREEN        Active                               �

�Printer         PaintJet XL                          �

�Printer Output  LPT1:                                �

�Display Mode    Automatic                            �

�Paper Size      A4                                   �

�Aspect Ratio    Rescaling                            �

�Print Size      Default                              �

�Orientation     Portrait                             �

�Background      Normal                               �

�Color           Enabeld                              �

�Gamma Correct.  Gamma 1  No Correction               �

�Top Offset      Default                              �

�Left Offset     Default                              �

�Spacing         Default                              �

�Symbolset       Default                              �

�Form Feed       Yes                                  �

�Copies          [ 1] Copies                          �

�������������������������������������������������������

Press the <S> key

Type in the filename; e.g.: PAINT.CFG

Return to the main menu

����������������������������������������������������������������������������

H P S C R E E N  3.2                                    page 19

����������������������������������������������������������������������������

4.21. Load Configuration

With "Load Configuration" you can load any presaved configuration.

After you have selected the "Load Configuration" menu, HPSCREEN

will prompt you for the filename.

!

It may happen that the configuration will be loaded only when the

HPSCREEN menu has been exited.

Example:

You want to restore the above saved configuration PAINT.CFG

Press the <L> key

Type in the filename PAINT.CFG

Return to main menu

4.22. Batch Processing with HPSCREEN

If you have a graphics program and you want to make color

screendumps on a PaintJet printer, you can create the following

batch file to start your application.

.

.

HPSCREEN PAINT.CFG    The configuration saved in PAINT.CFG is loaded

Application Name      start application

����������������������������������������������������������������������������

H P S C R E E N  3.2                                    page 20

����������������������������������������������������������������������������

5. Display Specials

5.1. General

There are a number of compatible display adaptors on the market.

Not all compatibles are "true compatible".  Therefore special modes

of some display adaptors are not supported e.g.  (576 x 768 dots).

5.2. VGA

Some display adaptors are not 100% compatible to the IBM VGA

standard. There problems with the popup menu.

5.3. EGA and CGA

Most of the display adaptors can identify different display modes,

e.g. text modes and graphics modes. If you have a VGA display adaptor

HPSCREEN detects automatically the right display mode for generating

the printout. Usually this works also for the other display adaptors.

For technical reasons there are a few cases where HPSCREEN is not

able to identify the correct display mode.  In such cases, you can

manually enter the correct HPSCREEN mode.

5.4. Hercules

The Hercules Graphics Card can handle two display pages (0 and 1).

For technical reasons it is not possible to identify which page will

be displayed.  If you do not get the expected result on your

printer, you have to enter which page is active on the HPSCREEN menu.

Select "mode 4: graphics 720 x 348, page 0" to print page 0 and

"mode 5: graphics 720 x 348, page 1" to print page 1.

����������������������������������������������������������������������������

H P S C R E E N  3.2                                    page 21

����������������������������������������������������������������������������

6.  PRINTER SPECIALS

6.1. General

- You can not print landscape text and graphics on the same page.

- You can not use the "Print Width" and "Print Height" commands

for text screendumps.

- The default print size varies with the resolution of the printer.

- A fullscale printout takes a lot of time on a LaserJet printer.

- Cartridges are not supported

- If you get cancelled printouts using a parallel port, the following

DOS command may be helpful:

MODE LPT1:,,P #��     resp. MODE LPT1: retry=b

6.2. LaserJet

- No PC8 Symbolset.  Therefore HPSCREEN uses characters like (|-+) to

emulate the linedrawsymbols.

- Due to the 56 K Byte memory limitation it is not possible to print

graphics printouts bigger than about 32 cm�. The printer will show

printer error 20 (overflow).

6.3. LaserJet+ and LaserJet 500+

- No PC8 Symbolset.  Therefore HPSCREEN uses characters like (|-+) to

emulate the linedrawsymbols.

6.3. DeskJet

- Landscape Text

If you want to print text in landscape orientation on a DeskJet

with the K-Cartridge then you have to select DeskJet+ in the

printer menu.

6.4. PaintJet

- No Landscape Text

- If you define a left offset in a text hardcopy you will get

an additional linefeed.

6.5. QuietJet

- No Landscape Text

- No Left Offset

- The alternate mode is not supported.

����������������������������������������������������������������������������

H P S C R E E N  3.2                                    page 22

����������������������������������������������������������������������������

6.6. ThinkJet

- No PC8 Symbolset.  Therefore HPSCREEN uses characters like (|-+)

to emulate the linedrawsymbols.

- The alternate mode is not supported.

- No Left Offset

- No Top Offset

- The maximum print width is 165mm.

6.7. Ruggedwriter

- The Ruggedwriter does not support text in landscape orientation.

- The alternate mode is not supported.

7. TECHNICAL INFORMATION

7.1. General

HPSCREEN consists of two parts, the screendump program and the

"hot key" menu program to configure the screendump portion.

7.2. Display Modes

All standard display modes are listed below. Although a lot of

modes are available, not all of them are usual. Normally the

application programs use the modes which offer the highest

resolution. To help to identify the correct mode the usual

modes are marked.

VGA  Video Graphics Adaptor             is available also on

other display adaptors

��������������������������������������������������������������

Mode  0: Text 40 x 25   black & white   VGA, EGA, MCGA, CGA

Mode  1: Text 40 x 25       16 colors   VGA, EGA, MCGA, CGA

Mode  2: Text 80 x 25   black & white   VGA, EGA, MCGA, CGA

Mode  3: Text 80 x 25       16 colors   VGA, EGA, MCGA, CGA

Mode  4: Graphic 320 x 200, 4  colors   VGA, EGA, MCGA, CGA

Mode  5: Graphic 320 x 200, 2  colors   VGA, EGA, MCGA, CGA

Mode  6: Graphic 640 x 200, 2  colors   VGA, EGA, MCGA, CGA

Mode  7: Text 80 x 25       Monochrome  VGA, EGA,       MDA

Mode 13: Graphic 320 x 200, 16 colors   VGA, EGA,

Mode 14: Graphic 640 x 200, 16 colors   VGA, EGA,

Mode 15: Graphic 640 x 350, Monochrome  VGA, EGA,

Mode 16: Graphic 640 x 350, 16 colors   VGA, EGA,

Mode 17: Graphic 640 x 480, 2  colors   VGA,      MCGA

Mode 18: Graphic 640 x 480, 16 colors   VGA

Mode 19: Graphic 320 x 200, 256 colors  VGA,      MCGA

����������������������������������������������������������������������������

H P S C R E E N  3.2                                    page 23

����������������������������������������������������������������������������

Usual modes     Usual modes

if you have a   if you have a

VGA  Video Graphics Adaptor             color display   monochrome

display

��������������������������������������

Mode  0: Text 40 x 25   black & white

Mode  1: Text 40 x 25       16 colors

Mode  2: Text 80 x 25   black & white

Mode  3: Text 80 x 25       16 colors   text

Mode  4: Graphic 320 x 200, 4  colors

Mode  5: Graphic 320 x 200, 2  colors

Mode  6: Graphic 640 x 200, 2  colors

Mode  7: Text 80 x 25       Monochrome                  text

Mode 13: Graphic 320 x 200, 16 colors

Mode 14: Graphic 640 x 200, 16 colors

Mode 15: Graphic 640 x 350, Monochrome                  graphics

Mode 16: Graphic 640 x 350, 16 colors

Mode 17: Graphic 640 x 480, 2  colors

Mode 18: Graphic 640 x 480, 16 colors   graphics

Mode 19: Graphic 320 x 200, 256 colors

Usual modes     Usual modes

if you have a   if you have a

EGA  Enhanced Graphics Adaptor          color display   monochrome

�������������������������������������                   display

Mode  0: Text 40 x 25   black & white

Mode  1: Text 40 x 25       16 colors

Mode  2: Text 80 x 25   black & white

Mode  3: Text 80 x 25       16 colors   text

Mode  4: Graphic 320 x 200, 4  colors

Mode  5: Graphic 320 x 200, 2  colors

Mode  6: Graphic 640 x 200, 2  colors

Mode  7: Text 80 x 25       Monochrome                  text

Mode 13: Graphic 320 x 200, 16 colors

Mode 14: Graphic 640 x 200, 16 colors

Mode 15: Graphic 640 x 350, Monochrome                  graphics

Mode 16: Graphic 640 x 350, 16 colors   graphics

Usual modes     Usual modes

if you have a   if you have a

MCGA  Multi Color Graphics Card         color display   monochrome

��������������������������������������                  display

Mode  0: Text 40 x 25   black & white

Mode  1: Text 40 x 25       16 colors

Mode  2: Text 80 x 25   black & white                   text

Mode  3: Text 80 x 25       16 colors   text

Mode  4: Graphic 320 x 200,  4 colors   graphics

Mode  5: Graphic 320 x 200,  2 colors

Mode  6: Graphic 640 x 200,  2 colors   graphics

Mode 17: Graphic 640 x 480, Monochrome                  graphics

Mode 19: Graphic 320 x 200, 256 colors

����������������������������������������������������������������������������

H P S C R E E N  3.2                                    page 24

����������������������������������������������������������������������������

Usual modes     Usual modes

if you have a   if you have a

CGA  Color Graphics Card                color display   monochrome

�������������������������������������                   display

Mode  0: Text 40 x 25   black & white

Mode  1: Text 40 x 25       16 colors

Mode  2: Text 80 x 25   black & white

Mode  3: Text 80 x 25       16 colors   text

Mode  4: Graphic 320 x 200, 4  colors   graphics

Mode  5: Graphic 320 x 200, 2  colors

Mode  6: Graphic 640 x 200, 2  colors   graphics

Only

monochrome

HGC  Hercules Graphics Card                             display

������������������������������������

Mode  4: Graphic 720 x 348, Page 1                      graphics

Mode  5: Graphic 720 x 348, Page 2                      graphics

Mode  7: Text 80 x 25      Monochrome                   text

Only

monochrome

MDA  Monochrome Display Adaptor                         display

������������������������������������

Mode  7: Text 80 x 25   Monochrome                      text

Usual modes     Usual modes

if you have a   if you have a

MMC  Multi Mode Card                    color display   monochrome

�������������������������������������                   display

Mode  0: Text 40 x 25   black & white

Mode  1: Text 40 x 25       16 colors

Mode  2: Text 80 x 25   black & white

Mode  3: Text 80 x 25       16 colors   text

Mode  4: Graphic 320 x 200, 4  colors   graphics

Mode  5: Graphic 320 x 200, 2  colors   graphics

Mode  6: Graphic 640 x 200, 2  colors

Mode  7: Text 80 x 25       Monochrom                   text

Mode 13: Graphic 640 x 400,  2 colors

Mode 14: Graphic 320 x 400,  4 colors

Mode 15: Graphic 320 x 400,  2 colors

����������������������������������������������������������������������������

H P S C R E E N  3.2                                    page 25

����������������������������������������������������������������������������

7.3. Ordered Dithering

HPSCREEN prints different colors by using the "Ordered Dithering"

algorithm.  This algorithm allows to print colors in gray shades

on black and white printers and to print color shades on color

printers. This algorithm is fast and does not slow down the

resolution of printer.

7.4. Funny Patterns

Some applications mix screen colors to generate more than the true

screen colors.  HPSCREEN uses the above mentioned algorithm to

generate more than the true printer colors which mixes also printer

colors to create different color shades.  As a result you will get

unpredictable patterns (funny patterns).

8. PALETTE

The PaintJet is able to print its 330 different colors using

the selftest. You start the selftest by starting the printer and

at the same time pressing the key <LF> (Linefeed).

Because the PaintJet XL does not provide this feature, the

program PALETTE is provided on the HPSCREEN disk.  This utility

produces the same output on the PaintJet, PaintJet XL as the selftest

mentioned above.

!!

This utility is written for the PaintJet /XL  ONLY!  If you use

PALETTE with other printers you will get strange output on the

printer or your system may hang up.

9. Changing the hotkey

If necessary you can change the hotkey <Alt><+> (+ on the

numeric keyboard) into another <Alt><YOUR CHOICE> keystroke

combination.

To use the new keystroke combination start HPSCREEN by

HPSCREEN /HOTKEY:Scancode

e.g.

HPSCREEN /HOTKEY:35

The menu can be activated by <Alt><H>.

The following table shows you some scancodes.

�����������������������������������������������������������������������������

H P S C R E E N  3.2                                    page 26

����������������������������������������������������������������������������

Scancodes for some keystrokes   (english keyboard layout)

Q   W   E   R   T   Y   U   I   O   P   [   ]

16  17  18  19  20  21  22  23  24  25  26  27

A   S   D   F   G   H   J   K   L   ;   .   \

30  31  32  33  34  35  36  37  38  39  40  43

Z   X   C   V   B   N   M   ,   .   /

44  45  46  47  48  49  50  51  52  53

The scancode layout, espacially for the special keys like

F1, F2 a.s.o.,  depends on the computer. Therefore you have

try some numbers to find the best solution.

�����������������������������������������������������������������������������