MS-5129, MS-5136, MS-5137 and MS-5143 AI78 BIOS Release

-------------------------------------------------------

1.  This is an AMI BIOS release.  AI76 and AI77 BIOSes are not formally

released.

2.  This BIOS fixes the following problems of the previous versions:

-  In Windows 95, if the SUSPEND button is pressed twice, system will hang.

-  In Windows 95, even if the Power Management is enabled, SUSPEND

button will not appear.

-  In Windows 95, 3COM 3C509B ISA PnP Network Card will have a warning (!)

message, and will not function properly.

-  When booting with 3COM 3C590 PCI network card boot ROM, the message

"Press H to boot from hard disk" will not appear.

-  Intel P54C-200 MHz CPU will sometimes be detected as 210 MHz during

boot-up.

-  With on-board 256K cache, adding a 256K cache module will cause the

system to hang during boot-up.

-  After performing FDISK in Windows 95, DOS FDISK will recognize the

primary partition as non-DOS and the partition size will be 2MB

lesser.

-  These problems have been fixed in AI78.

3.  NEW FUNCTIONS:

-  The new AMD-K5/PR166 CPU is now supported.

-  * The new AMD-K6/PR2-166 to PR2-233 CPUs are now supported.

-  * The new Intel P55C-MMX/233 CPU is now supported.

-  "Boot from CD-ROM" and "NT 4.0 Auto Installation" functions are now

supported.

* NOTE:  Please take note that these CPUs require hardware modifications.

Kindly check with your Sales Executive.

** Please refer to FLASH.DOC for programming notes.