MS-5164 W564MS30 BIOS Release

-----------------------------

1.  This is an AWARD BIOS ver 3.0 release.

2.  This BIOS fixes the following problems of the previous version:

-  The default value for Pipelined Function is changed to Disabled for

better compatibility with DVD ROM.

-  The default value for DRAM Refresh Period is changed to 15us to

solve compatiblity problems with Mitsubishi memory.

-  The default value for Primary Frame Buffer is changed to Disabled

and VGA Frame Buffer is changed to Disabled to solve the problem

of screen corruption when running Netware 3.12 using Cyrix CPU.

-  When Power Button is set to Disabled: in Windows 95 DOS mode, if

the power switch is pressed once, and EXIT is typed to run Windows

95, the system will power off.

-  The date and time will be cleared, after the system is powered off

for a period of time.

-  All the above problems have been fixed in this version.

** Please refer to FLASH.DOC for programming notes.