MS-5129, MS-5136, MS-5137 WI7A BIOS Release

-------------------------------------------

1.  This is an AWARD BIOS release.  WI79 is not formally released.

2.  This BIOS fixes the following problems of WI78:

-  With Intel 150MHz, the system will sometimes hang during warm boot.

-  Using single sided + double sided EDO memory with 133MHz CPU, the

system will sometimes hang during warm boot.

-  Toshiba 8x CD-ROM will have a warning (!) in Windows 95.

-  With 2 WD hard disks as master and slave, obly the master hard disk can

be detected.

-  All the above problems have been fixed in WI7A.

3.  NEW FUNCTIONS:

A.  In ADVANCED CHIPSET SETUP, an option called "CD-ROM OPTION" has been

added.  This has to be set for some types of CD-ROM which will show

a warning (!) in Windows 95.

B.  In ADVANCED CMOS SETUP, SCSI has been added as an option for "BOOT

SEQUENCE".

** Please refer to FLASH.DOC for programming notes.