Flash BIOS upgrade for the UM8810 486 PCI Motherboard.

**************************************************************************

WARNING: Use of this BIOS upgrade with anything other than a UM8810

motherboard will cause serious hardware problems.

**************************************************************************

1: Unzip the BIOS upgrade to disk using the following command:

PKUNZIP 8810V21.ZIP A:\

2: Reboot machine.

3: When system starts to boot from hard drive, hold down the Left Shift key.

4: Type 'A:' and hit <ENTER>

5: Type 'flash' and hit <ENTER>

6: Select 'Load file to buffer' and hit <ENTER>

7: Type '881_V21.ROM' and hit <ENTER>

8: Hit <Enter>

9: Select 'Program Flash ROM' and hit <ENTER>

10: The BIOS will now be programmed.  When the Program function is done,

your machine will reboot itself.

11: You will now need to go into the BIOS setup and setup hard drives,

floppy drives, serial ports, etc.

Follow these steps so that the BIOS can see system componites.

1. Hit the DEL key when prompted

Go to standard

2. Reset the floppy drives

Go to fix disk setup

3. Set the Hard drive settings

Go to the Advance Setup

4. Go to the VLB Mode. set it to enable

5. Go to 32 bit access to ON

6. go to Multi Transfer to 16

7. Set the PCI setting to disable