Rom capture utilities for PC and AT

This is a set of utilities that allows the user to capture the contents

of the rom sets in either the PC or the AT.  It allows the content of the rom

to be written to a file called xtrom for the XT or atrom for the AT.  A

utility called romsplit is provided for the AT that allows the atrom file to

be split into two files that represent the even and odd bytes and which are

called tempeven and tempodd, respectively.  Note that this is not necessary

for the XT.  Another utility called lsplit is provided to allow the large

files produced by romsav to be split into files that will fit into the desired

destination eproms.  For prom burners that accept data format in the Motorola

S record format, a utility called binex is provided that converts a binary

file into standard S record format.  A typical session is described below for

an AT.	Original Author UNKNOWN, more routines added by Allan Teo.

1.   Run the command "romsavat".  This will produce a file called "atrom".

1.1  FOR THE TOSHIBA t1100+ DELETE THE FIRST 2000h(8192d) BYTES

SINCE IT OCCIPIES A 27256 EPROM (32K)

2.   Run the command "romsplit" (copy "atrom" to a new file temp).  This

will produce two files called "tempeven" and "tempodd".

3.   If the destination eproms are 27256 then it is not necessary to split

the files into smaller ones, otherwise run the command "lsplit <epromsize>

tempeven", where <epromsize> is the decimal size of the destination file in

bytes (e.g. "lsplit 16384 tempeven" to make tempeven into two files called

"F0" and "F1" that will fit into 27128 eproms.  Rename the files to some

new name so that they will not be overwritten at the next step.  Now run

the command "lsplit <epromsize> tempodd" to produce files for the odd byte

eproms.

3.0.1 You can run APPEND to get LSPLITed ROMS back to the original config.

EG: Append F0 F1 tempevn to get back tempevn

RUN UNSPLIT to get back ATROM after recovering tempevn + tempodd

3.1 If you are using roms that come only in 2 27128 eproms (EG: AT PHEONIX),

and you had run ROMSAVAT you will still get a 65536 byte file.

then run UNSPLIT after running lsplit 16384 tempevn etc

producing the F0 and F1 files for both even and odd eproms.

F0 should all be FFs or some garbage and F1 is CODE.

Copy the EVEN F1 to tempevn and the ODD F1 to tempodd.

run unsplit. It will produce a file call ROM.

You can then debug and change it. No point wasting it in 2 27256 eproms.

3.2 Chksum should then be run to recalculate the chksum and produce

a new file you specify, say TEMP. Then you can split it again.

3.3 if the rom file is 65536 bytes,  then in DEBUG, the rest of the code

should continue at CS+1000:0 to the end where you can see the date.

4.  If you are producing S records run "binex <source> <destination>" on

each file that you wish to convert to S record format.

5.   Feed them to a prom burner and keep track of which eproms were even

and odd and the order in which they were burned to allow you to install

them properly in the target system.

EPROMS:		in bytes ; if you are new WATCH OUT FOR PROGRAMMING VOLTAGE!

27512 = 65536

27256 = 32768

27128 = 16384

2764  = 8192

romsavxt will save = 40960 bytes

romsavat will save = 65536   "

chksum.exe works on AT and XT roms that add up all bytes in the rom ignoring

the carry, after which the result in the 1 byte register

(EG:AL or a BL) must equal 0.

:: Example Saving Phoenix Roms or similar clone roms.

Steps: ROMSAVAT

copy atrom temp

romsplit

lsplit 16384 tempevn

copy f1 tempevn

lsplit 16384 tempodd

copy f1 tempodd

unsplit			;; produce a 32768 file

debug ROM ;; make changes if wanted

chksum rom temp

romsplit

BURN tempevn and tempodd in 27128 chips

OR

append f0 tempevn newtempe

copy newtempe tempevn

append f0 tempodd newtempo

copy newtempo tempodd

to BURN TEMPEVN and TEMPODD in 27256 eproms

:: Example Saving the Real Thing and put it into a Taiwanese CLONE.

steps:

ROMSAVAT

debug atrom  ;; make changes if needed

chksum atrom temp

romsplit

You can now BURN tempevn and tempodd in 27256 chips

OR

lsplit 16384 tempevn - produce f0 and f1

burn f0,f1 into 27128 eproms, f0 eprom goes into first socket, f1

into SECOND.

lsplit 16384 tempodd - produce f0 and f1

burn f0,f1 into 27128 eproms, f0 eprom goes into THIRD socket, f1 goes

into FOURTH

:::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::

The Next step describes how to CHANGE THE FIXED DRIVE TABLES in	 an AT.

:::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::

YOU MUST HAVE SYMDEB from the MASM4.0 package to use this

If you have BASICA or GWBASIC execute the command

A>basica hdat

FOLLOW the instructions in UPPERCASE

-----------------------------------------------------------------------

THE ABOVE PROGRAMS WERE COMPILED WITH MICROSOFT C

Remember to use a FAR pointer if you don't have large mem model

(Large mem model used only in romsavat and romsavxt, all others

can be compiled using the small model)

** ORIGINAL AUTHOR UNKNOWN **

hdat.bas, all batch files Chksum.c, unsplit.c and append.c

added by Allan Teo, 76566,2357