PROGRAM:     ROMU.COM

DESCRIPTION: A simple utility to create and maintain AT ROM image

files including disk table and checksum changes.

CONTACT:     Keith P. Graham at PC Rockland BBS (914) 353-2157

USAGE:       ROMU.COM is initiated by entering ROMU at the dos

prompt. ROMU has 3 menus.

MENU 1. This menu provides several choices for a source of

ROM data. The source can be the current AT ROM, or

a file or files created by ROMU or similar utility.

The source of data can be either 32K (27128 type ROMS)

or 64k (27256 type ROMS). If loading information from

a file, the data can be a complete BIOS image or a

split image which corresponds to the high and low ROM

chips.

MENU 2. After a source is selected it is possible to display

and alter the disk table, calculate the checksum or

force the checksum to 0. A 32K ROM image can be

be converted to 64K by padding the low 32k with zeroes.

The file can be written out as either a BIOS image

or two split rom images.

MENU 3. This menu is shown when the the user chooses to display

and modify the disk table entries. The normal disk

table address in an AT is fixed at f000:e401. ROMU

makes the assumption that this address holds true for

all AT roms. IBM AT's have only 15 table entries which

can be changed. Some AT clones and 386 clones have up

to 64 table entries. ROMU will display all 64 entries

even if they are not valid. The invalid entries will

have odd characters while the valid entries will have

only decimal characters. Do not change entires beyond

the known limit of your bios.

When changing an entry you will be shown the current

value and asked for a replacement. If you press the

return without making an entry the existing entry will

be used by default.

When you have finished making all entries for a disk type

you will be shown the table entry and asked if it is

acceptable. If you enter "Y" the bios image will be

updated.