Date: 02/15/1991    Page: 1 of 8

AMI-BIOS CHECK-POINT, (C)1991 American Megatrends Inc.,All Rights Reserved

1346 Oakbrook Dr. #120. GA-30093. Phone:(404)-263-8181, Fax:(404)-263-9381

CHECK-POINT	DESCRIPTION OF CHECK-POINT

01	Processor register test about to start,

and NMI to be disabled.

02	NMI is DIsabled.

Power on delay starting.

03	Power on delay complete.

Any initialization before keyboard BAT is in progress.

04	Any initialization before keyboard BAT is complete.

Reading keyboard SYS bit, to check soft reset/ power-on.

05	Soft reset/ power-on determined.

Going to enable ROM. i.e. disable shadow RAM/Cache if any.

06	ROM is enabled. Calculating ROM BIOS checksum, and waiting

for KB controller input buffer to be free.

07	ROM BIOS checksum passed, KB controller I/B free.

Going to issue the BAT command to keyboard controller.

08	BAT command to keyboard controller is issued.

Going to verify the BAT command.

09	Keyboard controller BAT result verified.

Keyboard command byte to be written next.

0A	Keyboard command byte code is issued.

Going to write command byte data.

0B	Keyboard controller command byte is written.

Going to issue Pin-23,24 blocking/unblocking command.

0C	Pin-23,24 of keyboard controller is blocked/ unblocked.

NOP command of keyboard controller to be issued next.

0D	NOP command processing is done.

CMOS shutdown register test to be done next.

0E	CMOS shutdown register R/W test passed.

Going to calculate CMOS checksum, and update DIAG byte.

0F	CMOS checksum calculation is done, DIAG byte written.

CMOS init. to begin (If "INIT CMOS IN EVERY BOOT IS SET").

10	CMOS initialization done (if any).

CMOS status register about to init for Date and Time.

11	CMOS Status register initialised.

Going to disable DMA and Interrupt controllers.

Date: 02/15/1991    Page: 2 of 8

AMI-BIOS CHECK-POINT, (C)1991 American Megatrends Inc.,All Rights Reserved

1346 Oakbrook Dr. #120. GA-30093. Phone:(404)-263-8181, Fax:(404)-263-9381

CHECK-POINT	DESCRIPTION OF CHECK-POINT

12	DMA controller #1,#2, interrupt controller #1,#2 disabled.

About to disable Video display and init port-B.

13	Video display is disabled and port-B is initialized.

Chipset init/ auto memory detection about to begin.

14	Chipset initialization/ auto memory detection over.

8254 timer test about to start.

15	CH-2 timer test halfway.

8254 CH-2 timer test to be complete.

16	Ch-2 timer test over.

8254 CH-1 timer test to be complete.

17	CH-1 timer test over.

8254 CH-0 timer test to be complete.

18	CH-0 timer test over.

About to start memory refresh.

19	Memory Refresh started.

Memory Refresh test to be done next.

1A	Memory Refresh line is toggling.

Going to check 15 micro second ON/OFF time.

1B	Memory Refresh period 30 micro second test complete.

Base 64K memory test about to start.

20	Base 64k memory test started.

Address line test to be done next.

21	Address line test passed.

Going to do toggle parity.

22	Toggle parity over.

Going for sequential data R/W test.

23	Base 64k sequential data R/W test passed.

Any setup before Interrupt vector init about to start.

24	Setup required before vector initialzation complete.

Interrupt vector initialization about to begin.

25	Interrupt vector initialization done.

Going to read I/O port of 8042 for turbo switch (if any).

26	I/O port of 8042 is read.

Going to initialize global data for turbo switch.

Date: 02/15/1991    Page: 3 of 8

AMI-BIOS CHECK-POINT, (C)1991 American Megatrends Inc.,All Rights Reserved

1346 Oakbrook Dr. #120. GA-30093. Phone:(404)-263-8181, Fax:(404)-263-9381

CHECK-POINT	DESCRIPTION OF CHECK-POINT

27	Global data initialization is over.

Any initialization after interrupt vector to be done next.

28	Initialization after interrupt vector is complete.

Going for monochrome mode setting.

29	Monochrome mode setting is done.

Going for Color mode setting.

2A	Color mode setting is done.

About to go for toggle parity before optional rom test.

2B	Toggle parity over. About to give control for any

setup required before optional video ROM check.

2C	Processing before video ROM control is done.

About to look for optional video ROM and give control.

2D	Optional video ROM control is done. About to give control

to do any procesing after video ROM returns control.

2E	Return from processing after the video ROM control.

If EGA/VGA not found then do display memory R/W test.

2F	EGA/VGA not found.

Display memory R/W test about to begin.

30	Display memory R/W test passed.

About to look for the retrace checking.

31	Display memory R/W test or retrace checking failed.

About to do alternate Display memory R/W test.

32	Alternate Display memory R/W test passed.

About to look for the alternate display retrace checking.

33	Video display checking over. Verification of display

type with switch setting and actual card to begin.

34	Verification of display adapter done.

Display mode to be set next.

35	Display mode set complete.

BIOS ROM data area about to be checked.

36	BIOS ROM data area check over.

Going to set cursor for power on message.

37	Cursor setting for power on message id complete.

Going to display the power on message.

Date: 02/15/1991    Page: 4 of 8

AMI-BIOS CHECK-POINT, (C)1991 American Megatrends Inc.,All Rights Reserved

1346 Oakbrook Dr. #120. GA-30093. Phone:(404)-263-8181, Fax:(404)-263-9381

CHECK-POINT	DESCRIPTION OF CHECK-POINT

38	Power on message display complete.

Going to read new cursor positon.

39	New cursor position read and saved.

Going to display the reference string.

3A	Reference string display is over.

Going to display the Hit <ESC> message.

3B	Hit <ESC> message displayed.

Virtual mode memory test about to start.

40	Preperation for virtual mode test started.

Going to verify from video memory.

41	Returned after verifying from display memory.

Going to prepare the descriptor tables.

42	Descriptor tables prepared.

Goin to enter in virtual mode for memory test.

43	Entered in the virtual mode.

Going to enable interrupts for diagnostics mode.

44	Interrupts enabled (if diagnostics switch is on).

Going to initialize data to check memory wrap around at 0:0.

45	Data initialized. Going to check for memory wrap around at 0:0

and finding the total system memory size.

46	Memory wrap around test done. Memory size calculation over.

About to go for writing patterns to test memory.

47	Pattern to be tested written in extended memory.

Going to write patterns in base 640k memory.

48	Patterns written in base memory.

Going to findout amount of memory below 1M memory.

49	Amount of memory below 1M found and verified.

Going to findout amount of memory above 1M memory.

4A	Amount of memory above 1M found and verified.

Going for BIOS ROM data area check.

4B	BIOS ROM data area check over. Going to check <ESC>

and to clear memory below 1M for soft reset.

4C	Memory below 1M cleared.	(SOFT RESET)

Going to clear memory above 1M.

Date: 02/15/1991    Page: 5 of 8

AMI-BIOS CHECK-POINT, (C)1991 American Megatrends Inc.,All Rights Reserved

1346 Oakbrook Dr. #120. GA-30093. Phone:(404)-263-8181, Fax:(404)-263-9381

CHECK-POINT	DESCRIPTION OF CHECK-POINT

4D	Memory above 1M cleared.	(SOFT RESET)

Going to save the memory size.

4E	Memory test started.		(NO SOFT RESET)

About to display the first 64k memory test.

4F	Memory size display started. This will be updated during

memory test. Going for sequential and random memory test.

50	Memory test below 1M complete.

Going to adjust memory size for relocation/ shadow.

51	Memory size adjusted due to relocation/ shadow.

Memory test above 1M to follow.

52	Memory test above 1M complete.

Going to prepare to go back to real mode.

53	CPU registers are saved including memory size.

Going to enter in real mode.

54	Shutdown successfull, CPU in real mode. Going to restore

registers saved during preparation for shutdown.

55	Registers restored.

Going to disable gate A20 address line.

56	A20 address line disable successful.

BIOS ROM data area about to be checked.

57	BIOS ROM data area check halfway.

BIOS ROM data area check to be complete.

58	BIOS ROM data area check over.

Going to clear Hit <ESC> message.

59	Hit <ESC> message cleared. <WAIT...> message displayed.

About to start DMA and interrupt controller test.

60	DMA page register test passed.

About to verify from display memory.

61	Display memory verification over.

About to go for DMA #1 base register test.

62	DMA #1 base register test passed.

About to go for DMA #2 base register test.

63	DMA #2 base register test passed.

About to go for BIOS ROM data area check.

Date: 02/15/1991    Page: 6 of 8

AMI-BIOS CHECK-POINT, (C)1991 American Megatrends Inc.,All Rights Reserved

1346 Oakbrook Dr. #120. GA-30093. Phone:(404)-263-8181, Fax:(404)-263-9381

CHECK-POINT	DESCRIPTION OF CHECK-POINT

64	BIOS ROM data area check halfway.

BIOS ROM data area check to be complete.

65	BIOS ROM data area check over.

About to program DMA unit 1 and 2.

66	DMA unit 1 and 2 programming over.

About to initialize 8259 interrupt controller.

67	8259 initialization over.

About to start keyboard test.

80	Keyboard test started. clearing output buffer, checking

for stuck key, About to issue keyboard reset command.

81	Keyboard reset error/stuck key found. About to

issue keyboard controller interface test command.

82	Keyboard controller interface test over.

About to write command byte and init circular buffer.

83	Command byte written, Global data init done.

About to check for lock-key.

84	Lock-key checking over.

About to check for memory size mismatch with cmos.

85	Memory size check done. About to display soft error

and check for pasasword	or bypass setup.

86	Password checked.

About to do pogramming before setup.

87	Programming before setup complete.

Going to cmos setup program.

88	Returned from cmos setup program and screen is cleared.

About to do programming after setup.

89	Programming after setup complete.

Going to display power on screen message.

8A	First screen message displayed. About to display <WAIT...>

message

8B	<WAIT...> message displayed.

About to do Main and Video BIOS shadow.

8C	Main and Video BIOS shadow successful.

Setup options programming after cmos setup about to start.

Date: 02/15/1991    Page: 7 of 8

AMI-BIOS CHECK-POINT, (C)1991 American Megatrends Inc.,All Rights Reserved

1346 Oakbrook Dr. #120. GA-30093. Phone:(404)-263-8181, Fax:(404)-263-9381

CHECK-POINT	DESCRIPTION OF CHECK-POINT

8D	Setup options are programmed, mouse check and init

to be done next.

8E	Mouse check and initialisation complete.

Going for hard disk, floppy reset.

8F	Floppy check returns that floppy is to be initialized.

Floppy setup to follow.

90	Floppy setup is over.

Test for hard disk presence to be done.

91	Hard disk presence test over.

Hard disk setup to follow.

92	Hard disk setup complete.

About to go for BIOS ROM data area check.

93	BIOS ROM data area check halfway.

BIOS ROM data area check to be complete.

94	BIOS ROM data area check over.

Going to set base and extended memory size.

95	Memory size adjusted due to mouse support, hdisk type-47.

Going to verify from display memory.

96	Returned after verifying from display memory.

Going to do any init before C800 optional ROM control

97	Any init before C800 optional ROM control is over.

Optional ROM check and control will be done next.

98	Optional ROM control is done. About to give control to do

any required procesing after optional ROM returns control.

99	Any initialization required after optional ROM test over.

Going to setup timer data area and printer base address.

9A	Return after setting timer and printer base address.

Going to set the RS-232 base address.

9B	Returned after RS-232 base address.

Going to do any initialization before Co-processor test

9C	Required initialization before co-processor is over.

Going to initialize the coprocessor next.

9D	Coprocesor initialized.

Going to do any initialization after Co-processor test.

Date: 02/15/1991    Page: 8 of 8

AMI-BIOS CHECK-POINT, (C)1991 American Megatrends Inc.,All Rights Reserved

1346 Oakbrook Dr. #120. GA-30093. Phone:(404)-263-8181, Fax:(404)-263-9381

CHECK-POINT	DESCRIPTION OF CHECK-POINT

9E	Initialization after co-processor test is complete.

Going to check extd keyboard, keyboard ID and num-lock.

9F	Extd keyboard check is done, ID flag set. num-lock on/off.

Keyboard ID command to be issued.

A0	Keyboard ID command issued.

Keyboard ID flag to be reset.

A1	Keyboard ID flag reset.

Cache memory test to follow.

A2	Cache memory test over.

Going to display any soft errors.

A3	Soft error display complete.

Going to set the keyboard typematic rate.

A4	Keyboard typematic rate set.

Goin to program memory wait states.

A5	Memory wait states programming over.

Screen to be cleared next.

A6	Screen cleared.

Going to enable parity and NMI.

A7	NMI and parity enabled.	Going to do any initialization

required before giving control to optional ROM at E000.

A8	Initialization before E000 ROM control over.

E000 ROM to get control next.

A9	Returned from E000 ROM control.	Going to do any

initialization required	after E000 optional ROM control.

AA	Initialization after E000 optional ROM control is over.

Going to display the system configuration.

00	System configuration is displayed.

Going to give control to INT 19h boot loader.