STANDARD MICROSYSTEMS CORPORATION

IBM LAN Server v2.0 and v3.0 OS/2 Requester

For EtherCard Elite32 Ultra Ethernet LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1994 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  How to use EZStart to copy the IBM LAN server OS/2 requester

from the SuperDisk to your PC.

o  How to install the IBM LAN server OS/2 requester.

COPYING THE DRIVER FILES

From the EZStart Driver Installation screen, select 'Copy Files'

to copy all the files used by this driver from the SuperDisk to

the Destination Directory recommended by EZStart, or to a different

directory that you specify.

The EZStart default Destination Directory is a:\NDIS.  Please make

a backup copy of the SuperDisk before EZStart copies files to the

SuperDisk to create an "option" diskette.

INSTALLING THE DRIVER

1. Insert the SMC or IBM LAN Adapter Protocol Support (LAPS)

Version 1.00 diskette into your computer's disk drive.

2. Type: LAPS

3. Press the Enter key.

The IBM logo appears.

4. Select the INSTALL button.

5. Specify the target drive (usually C:\) and press Enter or

select OK. Files will be copied to target drive, then

select OK.

6. Select the INSTALL button a second time to load the SMC

drivers and press ENTER.

7. Place the SuperDisk in drive a:.

8. Change "Install Additional Network Driver" panel to read

"a:\NDIS", then select OK.

9. Press OK for each driver.

10. Select the Configure button.

11. Select "Configure LAN transports" option, then select Continue.

The Configure Workstation screen appears.

12. From the Network Adapters window, select the "SMC Ethernet

Adapter for OS/2" and select the Add button.

This adapter name appears in the Current Configuration window.

13. From the Protocols window, select "IBM IEEE 802.2" followed by

the Add button. Then select "IBM OS/2 NETBIOS" followed by Add.

(You may also need to select the "IBM NetWare Requester" option

and Add, depending on your network.)

Each time you select the Add button, the selected option appears

in the Current Configuration window, below the chosen driver.

14. Select OK.

15. Select Exit.

The CONFIG.SYS Update screen appears.

16. Specify the drive where the CONFIG.SYS file resides.

17. Select the Continue button.

18. Select OK.

19. Select Exit.

20. Reboot your computer. Use the OS/2 Shutdown process to exit from

the program in an orderly fashion.

21. Reboot your computer. Enter OS/2, and insert the OS/2 Requester

Install diskette into your computer's disk drive (a:\).

22. From the OS/2 prompt, type: LANINST.

23. Press ENTER.

24. Select OK at the IBM logo, then select OK again.

25. Select the "Advanced" option.

The Installation Tasks panel appears.

26. Select "Install or Configure this workstation", then select OK.

27. Select "Install or remove a component", then select OK.

28. From the displayed list, highlight all other components you

want to install.  Use the arrow keys to choose each component

and press the space bar to highlight it.  Select the Install

button and the word "Installed" appears in the Status column.

After you select all the components you need for this

installation, select the OK button.

29. Select the "Configure a component" option and select OK.

30. Select a component to configure from the list and select the

Configure button (Default is "Requester").

31. Specify the requester name and the domain name for that

component and select OK.

32. If you want the server to start automatically when one of

its workstations is started, select YES.

33. Select the requester services you require (either

"Messenger" or "Message pop-up") and select OK.

The Configure screen reappears.

34. Select OK. At "Apply the Changes" select OK and input the

discs as required. At the file backup panel select OK.

35. Select the "Configuration complete" option.

36. Select the LAN adapter to which this configuration applies.

If there is only one adapter in this computer, select

"Adapter 0" followed by OK.

37. Select OK when the Configure screen appears.

38. Select the "Apply the changes" option followed by OK.

39. Follow the screen prompts to copy all files from the

Requester diskettes to your computer's hard disk.

This completes the installation procedure.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and

EtherCard Elite32 Ultra, Elite Series, EliteView, EZStart,

SuperDisk, and UltraChip are trademarks of Standard Microsystems

Corporation.  Other products and company names are registered

trademarks or trademarks of their respective companies.