STANDARD MICROSYSTEMS CORPORATION

NDIS Driver

For EtherCard Elite32 Ultra Ethernet LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1994 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  An overview of the NDIS driver.

o  The files supplied with the NDIS drivers.

o  How to configure the SMC NDIS drivers.

o  The PROTOCOL.INI parameters.

o  Error messages.

OVERVIEW

The NDIS driver is designed to work with the SMC EtherCard

Elite32 Ultra Ethernet LAN adapters. It conforms to the

Network Driver Interface Specification (NDIS) version 2.0.1 defined

by Microsoft/3Com. The driver has been tested with Microsoft LAN

Manager, 3Com 3+Open, IBM LAN Server, IBM PC LAN Program,

DEC Pathworks, and other NDIS-compatible network operating systems

and protocol stacks.

SUPPLIED FILES

FILE               DESCRIPTION

____               ____________

NDIS.DOC           Installation Guide (this File)

SMC8232.OS2        OS/2 NDIS MAC driver

SMC8232.DOS        DOS NDIS MAC driver

PROTOCOL.INI       PROTOCOL.INI file sample

SMCDOSE.NIF        NIF file for adapters using DOS and IBM

LAN Server

SMCOS2E.NIF        NIF file for adapters using OS/2 and IBM

LAN Server

Follow the instructions in your SMC user guide to install the

adapter in your PC.

CONFIGURATION

The SMC NDIS drivers may be used to install one or two adapter

cards. When installing a LAN software, select either one or two SMC

adapters. When the LAN setup is done, examine the CONFIG.SYS and

PROTOCOL.INI files.

1. Copy the desired SMC NDIS driver from your SMC SuperDisk.

2. The CONFIG.SYS file should have only one DEVICE= line that

contains the SMC8232.XXX driver for DOS/OS2. For example:

DEVICE=C:\LANMAN\DRIVERS\ETHERNET\SMC8232\SMC8232.DOS

or

DEVICE=C:\LSP\SMC8232.DOS

or

DEVICE=C:\IBMCOM\MACS\SMC8232.OS2

3. The PROTOCOL.INI file should specify one or two SMC8232$ entries.

a. For the first adapter:

[SMC8232_NIF]

DriverName = SMC8232$

SlotNumber = 0x280

For the second adapter

[SMC8232_NIF]

DriverName = SMC8232$

SlotNumber = 0x300

Make sure the SlotNumber value does not conflict with the

value provided for the first adapter, and that it matches

the second adapter setting. Always include a SlotNumber

statement with the DriverName.

4. Modify adapter values so the two adapters do not use the same

values for I/O Base Address, RAM Address, and IRQ. Refer to your

SMC user's guide for information about these parameters.

PROTOCOL.INI PARAMETERS

Several parameters are typically specified in the SMC8232 section of

the PROTOCOL.INI file. The only parameter that must be specified

is the 'DriverName'. All others have default values supplied by the

driver and can be omitted from the PROTOCOL.INI file if the

installed adapter matches the default configuration.

Your SMC EtherCard Elite32 Ultra Ethernet adapter ignores I/O Base

Address, IRQ, and RAM Address and instead uses the values in the

POS registers, which are set up when the adapter is installed.

Below is a list of parameters and their definitions commonly found

in the PROTOCOL.INI file.

SlotNumber   Specifies the number of the slot containing the

adapter in an EISA computer. This parameter is a

decimal number in the range of 1 - 10 and is required

for all EISA adapters.

NetAddress   Overrides the factory set network address of the

network adapter. The value of this parameter is a

hexadecimal string of 12 digits between double

quotation marks. The address must be unique among

all network addresses on the network.

ERROR MESSAGES

"Could not find an adapter [at slot ___] [at I/O base address ___]"

The driver could not find an adapter at the specified location.

"Cannot access adapter RAM properly"

The memory test for adapter memory failed.

"Adapter hardware failure detected (self-test)"

There was an unidentified failure during adapter initialization.

"Adapter hardware failure detected during initialization"

The hardware did not respond correctly to initialization

commands.

"Other system RAM already occupies adapter RAM address space"

There is a RAM conflict with the adapter RAM.

"Other system ROM already occupies adapter RAM address space"

There is a rom conflict with adapter RAM.

"RAM base address chosen for driver does not match adapter

configuration"

The RAM base address must be specified by EZStart. The definition

in the configuration file must be removed.

"IRQ chosen for driver does not match adapter configuration"

The IRQ value must be specified by EZStart. The definition in the

configuration file must be removed.

"RAM size chosen for driver does not match adapter configuration"

The RAM size is stored on the adapter and is fixed in size. The

definition in the configuration file must be removed.

"I/O base address chosen for driver not valid"

The adapter I/O base address must be set between 200h-ec0h on a 20h

boundary.

"RAM base address chosen for driver not valid (out of range)"

The RAM base address must be between a000h-c000h.

"IRQ chosen for driver not valid (out of range)"

IRQ values must be between 2-15.

"RAM size chosen for driver not valid (out of range)"

RAM size must be either 200h (8k) or 400h (16k).

"Adapter configuration information corrupted; run software setup

program"

The onboard EEROM contents have been corrupted. Please run EZStart

to verify and correct.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and

EtherCard Elite32 Ultra, EliteSeries, EliteView, EZStart,

SuperDisk, and UltraChip are trademarks of Standard Microsystems

Corporation. Other products and company names are registered

trademarks or trademarks of their respective companies.