STANDARD MICROSYSTEMS CORPORATION

SuperDisk

For EtherCard Elite32 Ultra Ethernet LAN Adapters

This software is licensed by SMC for use by its customers only.

Copyright (c) 1994 Standard Microsystems Corporation.

All rights reserved.

Version 3.0   31 October 1994

Compressed Driver File SMCC30.EXE

DISK INFORMATION

This SMC SuperDisk is designed to be used with SMC's EZStart Program,

which is on this SuperDisk.  EZStart configures and tests your adapter,

copies the files you need from this disk to your PC, and displays or

prints appropriate instruction files that are located on this disk.

SYSTEM REQUIREMENTS

To use EZStart, your system must meet the following minimum requirements:

o  MS-DOS or PC-DOS operating system, version 3.11 or later, which

is required to run the EZStart program

o  To take advantage of EZStart's graphics features, a VGA or better

monitor; however, EZStart can operate in text mode on any monitor

o  A minimum amount of available hard disk space (typically 50KB)

for installing a single driver from the SuperDisk using EZStart

OR

At least 1.44MB of available hard disk space if you you intend

extract all of the drivers from the SuperDisk.

Note: Refer to the "Installing A Network Driver" section later in

this document or the User Manual for more information on

copying driver files from the SuperDisk.

USING EZSTART

EZStart is an all-purpose utility program that simplifies the

following tasks:

Install the network driver

Install and configure PC Agent/SNMP software, and view

network management information it collects

In addition, EZStart makes it easy to perform optional installation

and diagnostic procedures:

Test two or more adapters communicating with each other

over your actual network

View the configurations of all installed Elite Ultra adapters

LOADING EZSTART

EZStart can run from the diskette, or for faster operation it can be

copied onto a hard disk and run from there.

Note: To run EZStart from the hard disk, copy all the files from

the SuperDisk to your computer's hard disk.  However, make sure

that the compressed driver file, SMCC30.EXE, and EZSTART.EXE

reside in the same directory.

To load EZStart, from the DOS command line, type:

[path]ezstart<Enter>

To load EZStart in Text Mode (instead of Graphics Mode), type

[path]ezstart/T<Enter>

To load EZStart from Microsoft Windows, select the File menu in

Program Manager.  When the Run dialog box appears, type:

[path]ezstart

in the Command Line box, then select the OK button.

INSTALLING A NETWORK DRIVER

EZStart accesses network driver files through the compressed

driver file, SMCC30.EXE.  The file name, SMCC30.EXE, specifies

the family of adapters supported ("C" denotes 82M32 EtherCard

Elite32 Ultra), and the version of the SuperDisk ("30" denotes

3.0).

EZStart copies the network driver and any other supporting files

from the compressed driver file to a location compatible with

your network operating system.  If your network operating system

requires an installation or options floppy diskette, or your PC

has a single floppy drive, you can build the installation or

options diskette on the SuperDisk by following these steps:

Make a backup copy of your original SuperDisk

Load EZStart

When prompted for the Destination Directory by the EZStart

Driver Installation window, specify the floppy drive which

contains the SuperDisk (e.g., a:\)

Note: If you prefer, you can extract all of the network drivers

from the SuperDisk by:

Changing your default directory to the floppy drive where

the SuperDisk resides

Typing SMCC30 <Destination Directory> at the DOS prompt

(where <Destination Directory> is the path where you want

the extracted SuperDisk contents to reside)

Note: There must be at least 1.44MB of available hard disk space

Pressing Enter

The following list describes some of the files contained in

SMCC30.EXE:

File                       Contents

README.DOC     This file.

!SMC8010.CFG   EISA configuration files for the EtherCard

Elite32 Ultra Ethernet LAN Adapters (82M32)

EISA.DOC       Information about the EISA configuration file

Drivers for many LAN operating systems and software utilities are

also available through EZStart:

Utilities/     Contents

Drivers

DOS ODI        Open data-link interface driver to be used with

ODI protocol stacks.

EZSTART        Documentation for the EZStart Program.

MS LAN         (.DOS and .OS2) Microsoft LAN Manager drivers.

MANAGER

NDIS           DOS and OS/2 NDIS drivers to be used with NDIS-

compliant network operating systems and protocol

stacks. Subdirectories contain documentation and

.INI files for specific network operating systems.

NETWARE 286    Documentation for drivers for Novell NetWare 286.

NETWARE 386    Server driver for Novell NetWare 386 version 3.1.

Novell NetWare Client for Microsoft Windows NT.

NETWARE        Driver for Novell NetWare 386 version 4.0.

386 4.0

OS2 ODI        Driver for OS/2 nodes in Novell NetWare

environments.

PCAGENT        Documentation and driver for SMC's PC Agent/SNMP

network-management utility.

WINDOWS NT     Driver for Microsoft Windows NT.

In addition to the driver or utility program itself, each

driver has the following associated files:

o  An INSTALL.DOC or other .DOC file that explains how to install

and use the software; and

o  A RELEASE.DOC file that states the current version, release date,

and any improvements made in this version of the software.

SUPPORTED ADAPTER

The software on this SuperDisk operates with the EtherCard Elite32

Ultra Ethernet LAN Adapters.

BOOT ROMs FOR SMC ETHERNET ADAPTERS

Model Number      Adapters Supported     Protocols Supported

SMC-EROM          EtherCard Elite32      Native NetWare Protocol,

Ultra (82M32)          RPL (Remote Program

Load) Boot, and TCP/IP

Protocols

TECHNICAL SUPPORT

For technical support, first contact your place of purchase. If you

need additional assistance, call our Technical Support departments:

New York (8:30 am - 6:00 pm Eastern Time)

(800) 992-4762  (from U.S.A. and Canada)

(516) 435-6250

(516) 434-9314  (Fax)

California (7:00 am - 5:00 pm Pacific Time)

(800) 638-5323  (from U.S.A. and Canada)

(714) 707-5607

(714) 707-2491  (Fax)

SMC offers Bulletin Board Services (BBS) from which you can obtain

the latest information on adapters and drivers for SMC products via

modem:

California    (714) 707-2481

9600 baud, 8 data bits, no parity, 1 stop bit

New York      (516) 434-3162

9600 baud, 8 data bits, no parity, 1 stop bit

Germany       49 (89) 928806-50

2400 (V.22bis) or 4800 (V.32bis) baud, 8 data

bits, no parity, 1 stop bit

France        33 (1) 39.73.57.00

1200 to 14K baud, 8 data bits, no parity, 1 stop bit

United        44 (0) 932 350681

Kingdom       2400 or 19.2 (HST only), 8 data bits, no parity,

1 stop bit

INTERNET Support:  SMC Technical Support can also be obtained through

the INTERNET. Our address is:  techsupt@ccmail.west.smc.com

CompuServe Support:  For information and drivers for SMC network

products, you can also access the SMC Forum through CompuServe. Log

into CompuServe, and at the "!" prompt type "GO SMC".

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and

EtherCard Elite32 Ultra, EliteSeries, EliteView, EZStart,

SuperDisk, and UltraChip are trademarks of Standard Microsystems

Corporation. Other products and company names are registered

trademarks or trademarks of their respective companies.