����������������������������������������������������������������ͻ

�    Intel Above Board ISA Installation Problems and Solutions   �

����������������������������������������������������������������ͼ

Technical installation documents are also available through our 24-hour

FaxBack service.  If you have a touch-tone phone and a fax machine, you

can order a catalog of all Above Board and Matched Memory Classic documents

or an individual document.  The FaxBack number is (800) 525-3019.

COMPATIBILITY WITH OTHER INTEL MEMORY BOARDS

����������������������������������������������������������

Your Intel Above Board ISA is fully compatible with all other

Intel memory boards.  However, if your system has a mixture of

Above Board ISAs and other Intel memory boards, expanded memory

can be supplied ONLY by the Above Board ISAs OR by the other

memory boards, but not by both.

INSTALLING IN AN IBM XT

����������������������������������������������������������

If you wish to install your Above Board ISA into an IBM XT, you

should not set the switches so that the board uses I/O addresses

210-217 (as board number 4).  Refer to your Installation Guide for

information on setting the switches.

HWSETUP OPTIONS

����������������������������������������������������������

If you wish to run the HWSETUP program and bypass the automatic

configuration which occurs when the program starts, add "-A"

after the HWSETUP command, as follows:

HWSETUP -A

INSTALLING WITH AN IBM TOKEN RING ADAPTER

����������������������������������������������������������

A memory conflict can occur when expanded memory is installed in

a system with an IBM Token Ring board.  The Token Ring board and

the Expanded Memory Manager (EMM) use some reserved memory in the

range from C000 to DFFF in ATs and up to EFFF in PCs.

Follow the directions below to eliminate a reserved memory conflict.

For IBM AT or compatibles:

1.  After running SWSETUP, edit the CONFIG.SYS and add EP=C800,CC00

to the end of the ISAEMM.SYS command line, as follows:

DEVICE=ISAEMM.SYS <other parameters> EP=C800,CC00

This parameter assumes the Token Ring board switches are set for

the default ROM address of CC00.

2.  Set the Shared RAM address for the Token Ring to C800:

If CONFIG.SYS has the following drivers, edit CONFIG.SYS.

With DXMC0MOD.SYS, use:  Device=DXMC0MOD.SYS ,C800

OR

With DXMC1MOD.SYS, use:  Device=DXMC1MOD.SYS ,C800

(Don't add a space after the comma, otherwise the computer

will report an Invalid Early Token Passing Parameter error.)

If AUTOEXEC.BAT has TOKREUI, edit AUTOEXEC.BAT and use:

TOKREUI ,C800

If using the Banyan LAN software, use the PCCONFIG program to

change the Shared RAM address to C800.

For further information on resetting the Shared RAM address, see

your Token Ring manual.

For IBM PC, XT or compatibles:

1.  Add EP=CC00,D800 to the end of the ISAEMM.SYS driver line in the

CONFIG.SYS file, as follows:

DEVICE=ISAEMM.SYS <other parameters> EP=CC00,D800

This parameter assumes the Token Ring ROM address is set for the

default address CC00 and the Shared RAM address is set for the

default address D800.

16-BIT VIDEO BOARD COMPATIBILITY

����������������������������������������������������������

If your Intel Above Board ISA supplies expanded memory, there could

be a conflict with 16-bit VGA boards in the address range C000-DFFF.

The solution is to set the video board to work in "8-bit" mode (This

will not degrade or slow down the VGA if a cache is provided), and

exclude the video board reserved memory address on the ISAEMM.SYS

line in the CONFIG.SYS file.  For example, if you have a VGA

video card with a 32K ROM starting at C000, your ISAEMM.SYS line

would be:

DEVICE=ISAEMM.SYS <other parameters> EP=C000-C7FF

EXPANDED MEMORY CONFLICTS

����������������������������������������������������������

The Above Board ISA Expanded Memory Manager (ISAEMM.SYS) can conflict

with another board in the way they use reserved memory.

ISAEMM.SYS searches reserved memory from C000 to DFFF in an AT-type

computer and from C000 to EFFF in a PC-type computer to find unused

pages.  ISAEMM needs at least four 16K-byte pages of contiguous memory,

but will use as many free pages as it finds.

If your computer contains a ROM that isn't active at start-up (such as

IBM Token Ring), ISAEMM may assume the page is free for its own use and

a conflict will occur.  In this case, you must exclude the conflicting

page on the ISAEMM command line.

To exclude a page, use the EP parameter to specify the page or range

of pages that ISAEMM should not use.  For example, if you have a VGA

video card using up to C800, your ISAEMM.SYS line would be:

DEVICE=ISAEMM.SYS <other parameters> EP=C000-C7FF

This excludes the range from C000 up to C800.

MICROSOFT WINDOWS 3.0 and 3.1

����������������������������������������������������������

All Intel Memory boards are compatible with Microsoft Windows 3.0

and 3.1.

For 386 enhanced mode in a 386-based machine, set your Above Board

ISA for all extended memory.  A minimum of 2MB of memory is needed

to run Windows in this mode.  Windows 3.0 provides expanded memory

for applications that run under Windows.  You can install

Microsoft's EMM386.SYS or EMM386.EXE to provide expanded memory for

your applications that need expanded memory but don't run under

Windows.

For standard mode in a 286 machine, you can set your Above Board

ISA for both extended and expanded memory if needed.  A minimum

of 256K of extended memory is required to run Windows in this mode.

Be sure ISAEMM.SYS is BEFORE HIMEM.SYS in the CONFIG.SYS.

For real mode in an 8086/8088 machine, set your Intel memory board

for expanded memory.

Quarterdeck QRAM

����������������������������������������������������������

The QRAM manual mentions an Above Board Plus upgrade.  The Above

Board 286 is the only Above Board that needs the upgrade to take

better advantage of QRAM's abilities.

ISAEMM requires a minimum 64K-byte page frame to supply expanded

memory for applications.  QRAM will use any extra 16K-byte pages

(beyond the 64K-byte page frame that ISAEMM uses) to load device

drivers and TSRs out of conventional memory.  The pages QRAM uses

are dedicated to what is loaded into them.  QRAM will report

"Nothing useful to do" if there are no extra pages.

If you have problems with a program that uses expanded memory,

remove QRAM to see if the problem persists.