CardSoft ™ 3.1

Software

Technical Reference

Copyright (c) 1992-1994, SystemSoft Corporation.  All Rights Reserved.

SystemSoft Corporation

313 Speen Street

Natick MA 01760

No part of this publication may be reproduced, transmitted, transcribed, stored in a retrieval system, or translated into any language or computer language, in any form, or by any means, electronic, mechanical, magnetic, optical, chemical, manual or otherwise, without the prior written permission of SystemSoft Corporation.

Disclaimer

SystemSoft Corporation provides this document and the programs "as is" without warranty of any kind, either expressed or implied, including, but not limited to, the implied warranties of merchantability or fitness for a particular purpose.

This document could contain technical inaccuracies or typographical errors.  Changes are periodically made to the information herein;  these changes will be incorporated in future revisions of this document.  SystemSoft Corporation is under no obligation to notify any person of the changes.

The following trademarks are used in this document:

CardID is a trademark of SystemSoft Corporation.

CardSoft is a trademark of SystemSoft Corporation.

Microsoft is a registered trademark and Windows is a trademark of Microsoft Corporation.

PCMCIA is a trademark of PCMCIA Corporation.

SystemSoft is a registered trademark of SystemSoft Corporation.

CardSoft 3.1

Software

Technical Reference

Document Revision: 0.1

May 1994

# Table of Contents

# µ

Preface	vii

Reference Documentation	vii

Chapter 1

CardSoft PCMCIA Software Suite

Socket Services	1-1

Card Services (CS.EXE)	1-1

Card Identification (CARDID.EXE)	1-1

SRAM Card Driver (MTSRAM.EXE)	1-1

IDE/ATA Support (ATADRV.EXE and ATAINIT.EXE)	1-1

Card Services Power Management Enabler (CS_APM.EXE)	1-1

Windows Support Drivers (SSVCD.386, SSVRDD.386, and SSCOMM.DRV)	1-2

CardSoft for Windows Dynamic Link Library (SSWINCS.DLL)	1-2

CardSoft Utilities and Commands	1-2

Installation Program (INSTALL.EXE)	1-2

Configuration Utility (CONFIG.EXE)	1-2

Resource Allocation Utility (CSALLOC.EXE)	1-2

Card Information Utility (CARDINFO.EXE)	1-2

Windows Support Installation Utility (SETUP.EXE)	1-2

CardBoot (CARDBOOT.ROM, ENDBOOTB.COM, ENDBOOTR.COM, and FLASHFMT.EXE)	1-3

Installing the CardBoot Option ROM	1-3

Chapter 2

Using the Configuration Utility

Configuration Utility Help	2-1

Running the Configuration Utility	2-1

Accessing Menu Items	2-2

Card Recognition/Configuration Process	2-2

Creating a New Card Library File	2-3

Opening a Card Library File	2-3

Opening a Second Card Library File	2-4

Viewing a Card Library File	2-4

Creating New Card Library Records	2-4

Viewing a Card Library Record	2-4

Deleting Records From a Card Library File	2-4

Copying Records From a Card Library File	2-4

Saving a Card Library File	2-5

Closing a Card Library File	2-5

Exiting the Configuration Utility	2-5

Editing Card Recognition Parameters	2-5

Recognition Algorithms	2-6

Defining the Card Recognition Configuration	2-7

Configuration Algorithms	2-7

Using Custom Code	2-7

Defining Specific Card Recognition Parameters	2-8

Defining Parameters for Executable Files	2-11

Chapter 3

Configuring Driver Parameters

DEVICEHIGH Line Parameters	3-1

Socket Services	3-2

Intel 82365SL PCIC (ss365sl.exe or sl365lp.exe)	3-2

IBM ThinkPad 720 (ssibm720.exe)	3-2

DataBook TCIC-2/N (ssdbook.exe)	3-3

Cirrus CL-PD67xx (sscirrus.exe)	3-5

Intel AT2092AA PPEC (ssppec.exe)	3-5

Ricoh RF5C266/RF5C366 (ssricoh.exe)	3-5

Vadem VG365/465/468 (ssvadem.exe)	3-5

VLSI VL82C146 (ssvlsi.exe)	3-5

Card Services (cs.exe)	3-6

CSALLOC	3-8

SRAM Driver (MTSRAM.EXE)	3-9

Memory Technology Driver (MTDDRV.EXE)	3-10

IDE/ATA Device Driver (atadrv.exe)	3-10

Configuration Parameters	3-10

DEVICEHIGH Line Parameters	3-10

Disk Partitioning Utility (atainit.exe)	3-11

ATAINIT Return Code	3-11

Flash File System Executables	3-11

Card-Specific Memory Technology Drivers	3-11

MTAA.EXE, MTAB.EXE, MTI1.EXE, MTI2P.EXE	3-11

Appendix A

CardID

The [Common] Section	A-1

The [Libraries] Section	A-3

Comment Record	A-3

Card Services Resource Record	A-3

Card Recognition and Configuration Record	A-4

Card Recognition Fields	A-5

Card Configuration Fields	A-7

Appendix B

Card Services Vendor-Specific API Definitions

Subcomponent IDs	B-1

RegisterSpecialClient	B-2

SpecialGetStatus	B-3

Suspend/Resume	B-4

Socket Suspend/Resume	B-5

Ring Resume	B-6

CardBoot	B-7

BeepControl	B-9

GetWindowInfo	B-10

Appendix C

Socket Services Vendor-Specific API Definitions

Subcomponent IDs	C-1

Vendor Specific Functions	C-1

Set/Reset DMA Mode	C-1

Get CardBoot Info	C-2

Re-Initialize Adapter After Resume	C-3

Set/Reset Ring Indicate Mode	C-3

Appendix D

SystemSoft CardSoft API for Windows

Component Descriptions	D-2

Card Services API Description	D-2

Socket Services API Description	D-4

Appendix E

Extensions to the Card Services Specification Provided by CardID

GetClientInfo Subfunctions	E-1

GetGCIExtensionsInfo (Info Subfunction 80h)	E-2

GetCardID (Info Subfunction 81h)	E-3

Reserved Functions	E-3

GetCardIDArgList (Subfunction 86h)	E-4

SetCardIDArgList (Subfunction 87h)	E-5

GetLastError (Subfunction 8Fh)	E-5

Appendix F

Card Services Vendor-Specific Callbacks

Socket Configuration (83h)	F-1

# Preface

This document is written for software developers who need detailed information on SystemSoft's CardSoft software suite.  The information contained within this document is intended for a technical audience.  For instructions on installing and using the CardSoft software, refer to the CardSoft 3.1 Software User's Guide.  For information on using the Flash File System, refer to the Flash File System User's Guide.

# Reference Documentation

The following are available from the PCMCIA Corporation:

PCMCIA PC Card Standard Release 2.10

PCMCIA Socket Services Release 2.10

PCMCIA Card Services 2.10

PCMCIA PC Card ATA Specification Release 1.01

Chapter 1

# CardSoftTM PCMCIATM Software Suite

SystemSoft's CardSoft PCMCIA solution provides OEMs with a complete software solution for integrating PCMCIA controllers and slots into their computers.  The CardSoft software suite provides the end user with a complete "plug and play" system software solution for both DOS and Windowsä 3.1.  This solution consists of the following drivers and utilities.  Please be aware that your particular configuration may not include all drivers and utilities.

## Socket Services

Socket Services provides a standard software interface to host controller chips (such as the Intel 82365SL PCIC, Intel AT2092AA PPEC, ASCII, Cirrus CL-PD67xx, Motorola ScatKat/LapKat, IBM ThinkPad 720, Ricoh RF5C266/RF5C366, Vadem VG365/VG465/VG468, VLSI VL82C145, and the DataBook TCIC-2/N) and isolates the socket hardware from higher level software.  Socket Services includes functions such as configuring a socket for an I/O or memory interface and controlling socket power voltages.  The Socket Services driver that you receive depends upon the host controller chip that you are supporting.

## Card Services (CS.EXE)

The Card Services driver manages competition for system resources and manages adapter and card resources and configuration.

## Card Identification (CARDID.EXE)

This client device driver detects the insertion and removal of PC cards, automatically determines the card type upon insertion, and then configures the card and slot/adapter.

## SRAM Card Driver (MTSRAM.EXE)

This SystemSoft device driver recognizes and supports SRAM cards.

## IDE/ATA Support (ATADRV.EXE and ATAINIT.EXE)

ATADRV.EXE is a block device driver that supports ATA Type II Flash Disk or ATA Type III Hard Disk PC cards.  The ATAINIT.EXE utility places a hard disk partition table onto a blank ATA drive.  This utility initializes the ATA drive in the same manner as FDISK does with a standard hard drive.

## Card Services Power Management Enabler (CS_APM.EXE)

CS_APM.EXE is a DOS-based background task that enables Card Services to process system power management Suspend/Resume requests.  When a Suspend request is initiated by system power managment software, CS_APM notifies Card Services, which then verifies that the system PCMCIA slots are idle, and can be powered down.  Card Services then passes this information back to CS_APM, which then notifies the power management software that the sockets can be powered off.  When a Resume request is received by CS_APM, it informs Card Services, which then powers the sockets on again.

## Windows Support Drivers (SSVCD.386, SSVRDD.386, and SSCOMM.DRV)

These drivers permit hot insertion/removal of communications I/O, memory, and removable drive cards within Windows.  They are provided only if you received CardSoft with Windows support.

## CardSoft for Windows Dynamic Link Library (SSWINCS.DLL)

This Windows dynamic link library (DLL) is also required to support hot insertion/removal of PC Cards within a Windows environment.  This DLL is provided only if you received CardSoft with Windows support.

## CardSoft Utilities and Commands

### Installation Program (INSTALL.EXE)

This easy-to-use program installs all or some of the CardSoft components onto the system.  Users have the option of a standard or custom installation.

### Configuration Utility (CONFIG.EXE)

The basic Configuration Utility enables you to

·	Define the general configuration settings for fax/modem, ATA, and network cards.

·	Quickly view and change the system resources that are made available to Card Services.

·	Select the PCMCIA events for which a system beep code will be emitted.

In addition, a Configuration Utility for OEM developers enables you to:

·	Create or modify card records that define configuration and recognition parameters for specific PC cards, and save these records in a Card Library.

·	Configure CardSoft to your specific system requirements by modifying the various parameters for the CardSoft executable files (making it unnecessary to configure CardSoft with command line/device line switches).

This version of the Configuration Utility enables OEMs to more easily define the configuration and recognition of new PC cards.

### Resource Allocation Utility (CSALLOC.EXE)

CSALLOC is a DOS utility that scans the system for available memory, I/O port, and IRQ resources, updates the file CSALLOC.INI with this information, then displays the list of available resources that can be used by Card Services.  In addition, keywords can be added to CSALLOC.INI to force CSALLOC to exclude resources from being scanned, or include resources that are not detectable (due to security features or hardware design).  Other keywords enable you to reserve resources that may be specifically needed by a particular PC card or other system component.

### Card Information Utility (CARDINFO.EXE)

CARDINFO is a DOS utility that scans the PCMCIA slots on the system and lists information about the cards in these slots.  It also lists any warnings or error messages that may have occurred during the configuration of the cards by the CardSoft drivers.

### Windows Support Installation Utility (SETUP.EXE)

This installation utility automatically installs the Windows support files and DLL file, and automatically configures your Windows environment for CardSoft by modifying the Windows SYSTEM.INI file.  This file is provided only if you received CardSoft with Windows support.

### CardBoot (CARDBOOT.ROM, ENDBOOTB.COM, ENDBOOTR.COM, and FLASHFMT.EXE)

CardBoot is a suite of components that provide boot capabilities to Intel Series II cards:

CARDBOOT.ROM is the Boot Extension ROM.  It redirects reads of Drive A to reads of the PC card.  It is a binary image of a ROM file that should be put into the system ROM and will be found during the standard BIOS extension ROM scan.

ENDBOOTB.COM and ENDBOOTR.COM turn off the Drive A redirection, making diskette Drive A accessible after the boot from the PC card is completed.  ENDBOOTB.COM is used if the boot code is included in the INT13 handler of a SystemSoft BIOS.  ENDBOOTR.COM is used if the boot code is included in extension ROM.

FLASHFMT.EXE is the utility for creating a bootable partition on a Series II Flash card.

For more information on these components, refer to the CardSoft 3.1 Software User's Guide or the Flash File System User's Guide.

### Installing the CardBoot Option ROM

To install the CardBoot Option ROM:

1.	Use a ROM locator program (such as SystemSoft's ROMLOC program) to combine the CARDBOOT.ROM file with the BIOS ROM image (this creates a file approximately 128K in size).  CardBoot can be located at any available address in the range C0000-EFFFF.

2.	Use a PROM programmer to burn the new file onto the appropriate type of ROM.

3.	Insert the new ROM into the system.

4.	At system boot, the BIOS POST will find CardBoot during its normal scan for Option ROMs, and will call the CardBoot initialization routine.

5.	Once CardBoot is activated, follow the instructions in the CardSoft 3.1 Software User's Guide.

Chapter 2

# Using the Configuration Utility

This chapter describes how to use the Configuration Utility to configure the system's PC card subsystem, and to create and manage card library records that contain the configuration settings and tuple information required for the card to work properly.

The basic Configuration Utility, config, enables end users to:

·	Define the general configuration settings for fax/modem, modem, ATA drive, and network cards.

·	Quickly view and change the system resources that are made available to Card Services.

·	Select the PCMCIA events for which system beep codes will be emitted.

For information on this version of the Configuration Utility, refer to the CardSoft 3.1 Software User's Guide.

In addition, a Configuration Utility for OEM developers, config /all, enables you to

·	Create or modify card records that define configuration and recognition parameters for specific PC Cards, and save these records in a Card Library.

·	Configure CardSoft to your specific system requirements by modifying the various parameters for the CardSoft executable files (making it unnecessary to configure CardSoft with command line/device line switches).

This version of the Configuration Utility enables OEMs to more easily define the configuration and recognition of new PC cards.

The Configuration Utility makes modifications to the CARDID.INI and CSALLOC.INI files (the initiator files for CardID and CSALLOC).  When modifications are made to any of the CardSoft components, the time and date of the filename is changed to reflect the change (the size of the file, however, never changes).

Note	Any time modifications are made to CARDID.INI or CSALLOC.INI, you must reboot the system for the changes to take effect.

## Configuration Utility Help

On-line help is available for many of the Configuration Utility fields.  To access on-line help for a particular field, position the cursor in the field (or highlight the field), then press F1 (or click on the <Help> button, if available).

## Running the Configuration Utility

The Configuration Utility is an easy-to-use program for changing the CardSoft components.  Its interface is a windowed type with pull down menus.  You have the option of using the keyboard, a pointing device, or both to maneuver around the screen.

To run the Configuration Utility, change to the directory containing the utility (i.e., cd cardsoft), then type config /all.  If you only want to edit the card configuration settings or run the Resource Allocation utility, type config.  Depending upon the method used to start the Configuration Utility, one of the following screens is displayed.  Notice that without the /all switch, the Edit pull down menu is not available.

µ §

µ §

The initial Configuration Utility screen consists of a menu bar at the top and a help text section at the bottom.  The middle of the screen is blank.  When a Card Library file is selected, the card records in that file are displayed on the left in the Current File list box.  A second Card Library can be opened in view/copy mode; the records in this file are displayed on the right in the Card Library list box.

### Accessing Menu Items

The method for accessing menu items is the same for each Configuration Utility.  Each menu bar item contains a pull down menu with various items to choose from.  To access a pull down menu, click the left mouse button on the desired item or press Alt + the highlighted key.  For example, to access the File menu, click on the word File or press <Alt-F> from the keyboard.  When the pull down menu appears, select the desired item by:

·	Clicking on it with the left mouse button

·	Pressing the down arrow key to highlight the item and then pressing <Enter>, or

·	Pressing the key that corresponds to the highlighted letter.

### Card Recognition/Configuration Process

This section describes the basic process for defining card recognition and configuration settings for a specific PC card from within the Configuration Utility.  To define these settings, you have to run the config /all version of the utility.  For specific information on each step in the process, and whether or not additional steps are required for the card being defined, refer to the appropriate sections in this manual.

Note	If you only need to edit the general configuration parameters (that is, those that apply to all cards of a particular type), use the config command, and, if necessary, refer to the CardSoft 3.1 Software User's Guide for instructions.

1.	Open an existing card library (File Þ Open), or create a new card library (File Þ New).

2.	Select an existing card library record and edit it (Edit Þ Edit Current), or create a new card library record (Edit Þ Create New Record).  Either method displays the Card Recognition screen.

3.	Specify the Record ID, Card Type, and Recognition Algorithm.

4.	Select <Edit Recognition Detail> and enter the appropriate information.

5.	Select <OK> to display the Card Configuration screen.

6.	Specify the Configuration Algorithm.

7.	To use custom configuration or removal code, select these items, then select <Specify Custom Configuration Code>/<Specify Custom Removal Code> to enter the file names.

8.	Select <Edit Recognition Detail> to display the Specify Configuration Parameters screen.

9.	Specify the types of CONFIG and CFTABLE_ENTRY tuples you want to edit by selecting the appropriate <Specify ..> items, then select the matching <Edit ..> items to specify the tuple values.

10.	Return to the main menu and save the new record in the card library (File Þ Save) or 
(File Þ Save As).

### Creating a New Card Library File

New Card Library files can be created through the Configuration Utility.  Once a new Card Library file is created, you can copy records into it from other Card Library files, or you can create new records and add them to the file.

To create a new Card Library file, select New from the File pull down menu.  The message "Untitled (Card Library)" is displayed above the Current File box on the left side of the main menu.  Refer to "Creating New Card Library Records" for information on how to create new records for the file, or refer to "Copying Records from a Card Library File" for information on how to copy records from an existing Card Library to the new Card Library.

### Opening a Card Library File

To open an existing Card Library file for an editing session, select Open from the File pull down menu.  The following dialog box appears.

| µ § | Type the file name here, then select <OK>. OR Select the drive/directory. Then select the file from the File List box, and select <OK>. (Use the TAB key to move from area to area.) |
| --- | --- |

After opening a Card Library file, the PC card records in the file are displayed in the Current File list box on the left side of the main menu.  You can then select the record you want to edit by clicking on the item, or by using the â key to highlight it.  Once the record is selected, you can edit/view it.

### Opening a Second Card Library File

You can also open a second Card Library file in order to copy records from it into the Card Library file displayed in the Current File list box.  The second Card Library is opened in read-only mode; records can be copied, but they cannot be modified.  To open a second Card Library file, select Open Card Library from the File pull down menu.  The Open File dialog box appears, enabling you to select the file you want to open.  The records in the second Card Library file are displayed in the Card Library List box on the right side of the main menu.

### Viewing a Card Library File

Card Library files can also be opened in view-only mode.  When you open a file in this mode, you cannot edit or copy the records in the file (all Edit commands on the Edit pull down menu are grayed out).

To open a file in view-only mode, select View from the File pull down menu.  The Open File dialog box is displayed, enabling you to select the file whose records you want to view.

### Creating New Card Library Records

To create new PC card configuration records in a Card Library file, open the Card Library file to which you want to add records, then select Create New Record from the Edit pull down menu.  Type in the name for the record (usually the card name) in the Record ID field.  Then refer to "Editing Card Recognition Parameters" and "Defining the Card Recognition Configuration" for information on how to configure the card for recognition by Card Services.

### Viewing a Card Library Record

Card Library records can be opened in view-only mode.  To open a record in view-only mode, highlight the record in the Current File list box, then select View Current Record.  The Card Recognition screen appears (showing the Card Type and Recognition Algorithm).  From the Card Recognition screen, you can click on <OK> to view the Configuration Algorithm setting on the Card Configuration screen.  For more information on Card Type, Recognition Algorithms, and Configuration Algorithms, refer to "Editing Card Recognition Parameters" and "Defining the Card Recognition Configuration".

### Deleting Records From a Card Library File

Records can be deleted from a Card Library file that is opened in read/write mode (but not in view-only mode).  To delete a record from a Card Library file, open the file using the Open command on the File pull down menu.  Then highlight the record you want to delete and select Delete From Library from the Edit pull down menu.  A confirmation box is displayed.  Select <Yes> to delete the file, or select <No> to cancel the delete operation.

### Copying Records From a Card Library File

Records can be copied from a Card Library file that is opened in read-only mode into a Card Library file opened in read/write mode.  To copy a Card Library file:

1.	Open the destination Card Library file using the Open command on the File pull down menu.  The records in this file are listed on the left side of the main screen.

2.	Open the source Card Library file using the Open Card Library command on the File pull down menu.  The records in this file are listed on the right side of the main screen.

3.	In the Card Library list box, highlight the record that you want to copy.  Then select Copy From Library from the Edit pull down menu.  The name of the record should now appear in the Current File list box.

### Saving a Card Library File

Once you have modified a Card Library file, you must save your changes.  To save the file under its original name, select Save from the File pull down menu.  To save the file as a new file, select Save As from the File pull down menu, then type the new file name in the File Name box that is displayed.

### Closing a Card Library File

To close a Card Library file that was opened in view-only mode (that is, the Card Library file displayed on the right side of the main menu), select Close Card Library from the File pull down menu.

### Exiting the Configuration Utility

To exit the Configuration Utility, select Exit from the File pull down menu.  If the Current File has been modified, a dialog box is displayed that enables you to save your changes or discard them before exiting.  Select <Yes> to save your changes, <No> to discard them, or <Cancel> to return to the main menu and continue your editing session.

## Editing Card Recognition Parameters

To enable Card Services to recognize an inserted PC card, you must define the recognition parameters for the card.  This involves:

·	Specifying the Card Type

·	Selecting the Recognition Algorithm

·	Selecting the Configuration Algorithm

In addition, you may want to set specific configuration parameters for the card, or you may want to use custom configuration code or custom removal code to configure the card.

To define the Card Recognition parameters for a card, select the card record from the Current File list box, then select Edit Current Record from the Edit pull down menu.  The Card Recognition screen is displayed.

| µ § |  | Type the record name in the Record ID field. This is the text that appears in the Current File list box. Select a card type by clicking on the appropriate type, or by highlighting it and pressing the space bar. If not listed, select "Other" and type the name in the Card Type Name field. |  |
| --- | --- | --- | --- |
| To edit/view the detail record for the selected Recognition Algorithm, select Edit (or View) Recognition Detail. | Select the Recognition Algorithm by clicking on the desired item, or by highlighting it and pressing the space bar. |  |  |

When you are done defining the Card Recognition information, click on <OK>.  The Card Configuration screen appears.  Refer to "Defining the Card Recognition Configuration" to continue.

### Recognition Algorithms

This section describes the available recognition algorithms.

#### CIS Manufacturer/Product Data

Looks up the Level 1 Version Tuple in the Card Information Structure (CIS) on the card.  If this recognition algorithm is selected, the following screen appears when you select Edit/View Recognition Detail.  It contains four fields.  To match any or all of the strings in these fields with the CIS information on the card, a trailing star can be used as a wildcard character at the end of each string.  If a string is omitted, it is not matched with the CIS when card recognition is attempted.  For more information on Level 1 Version Tuples, refer to the PCMCIA PC Card Standard Release 2.10, available from the PCMCIA Corporation.

µ §

#### CIS Literal Bytes

Enables you to load a file that specifies a starting offset, length, and number of bytes to match.  Select this option for defective CISs (that is, when you can't find the Level 1 Version Tuple).  This function is not currently supported.

#### Generic Recognition by Card Type

Recognizes modem and ATA card types generically.  When you select this option, no additional card recognition information needs to be defined (that is, no Recognition Detail screen is available).  This option can only be used for modems and ATA cards.

#### Custom Code

Use this option to select a customized code file that you have written to support a particular card type.  The specified file must be in a certain format, as defined in the Library Specification in Appendix A.

| µ § | Type in a search string OR Select the drive/directory, then select the file name in the File List box and click on <OK>. |
| --- | --- |

### Defining the Card Recognition Configuration

When you select <OK> from the Card Recognition screen, the Card Configuration screen appears.  This screen is where you select the Configuration Algorithm and specify whether or not you want to use Custom Configuration code or Custom Removal code.

| µ § If you selected 'Custom Configuration Code' or 'Custom Removal Code', use these selections to specify the code file you want to use. | Select the Configuration Algorithm by clicking on it, or by highlighting it and pressing the space bar. An 'X' indicates that custom code will be used. If you select 'Specify Configuration Parameters', use these selections to edit/view the detail record. |
| --- | --- |

### Configuration Algorithms

#### Reject This Card

When the card is inserted into a slot, it will always be rejected, and an audible beep will sound.

#### Do Not Configure, or Use Custom Code only

When the card is inserted into a slot, it will not be configured unless 'Custom Configuration Code' is selected.  If 'Custom Configuration Code' is selected, click on <Specify Custom Configuration Code> to select the custom code file you want to load.  The file you specify must contain entries for the configuration parameters you want to load for the card.  The format of the custom code file is defined in the Library Specification in Appendix A.

#### Standard Configuration

The card will use the standard configuration for card recognition.

#### Specify Configuration Parameters

This selection enables you to define customized configuration parameters for card recognition, including default and override CONFIG tuples, as well as default, required, and override CFTABLE_ENTRY tuples.  For more information on this selection, refer to 'Defining Specific Card Recognition Parameters'.

### Using Custom Code

If you want to use custom configuration code to define or override one or more of the settings for the selected configuration algorithm, click between the brackets to the left of Custom Configuration Code to select it (it is selected when an 'X' is displayed between the brackets).  Then click on <Specify Custom Configuration Code> to select the code file that contains the configuration information you want to use (a dialog box is displayed, from which you can select the file).  For example, if you selected the Standard Configuration Algorithm, but the card is configured with an incorrect or invalid base address for Memory Window 1, you would have to create a custom configuration code file that defines the correct Memory Window 1 base address.  The custom configuration code file must follow the format described in the Library Specification in Appendix A.

If a special event must occur when the card is removed, click between the brackets to the left of Custom Removal Code to select it (it is selected when an 'X' is displayed between the brackets).  For example, you may have to clear a BIOS data area that was set up for a modem card when it was inserted.  Then click on <Specify Custom Removal Code> (a dialog box is displayed, from which you can select the file).  The custom removal code file must follow the format described in the Library Specification in Appendix A.

### Defining Specific Card Recognition Parameters

If you want to define customized configuration parameters for card recognition, select 'Specify Configuration Parameters' on the Card Recognition screen.  Then select <Edit Configuration Detail> to display the Specify Configuration Parameters screen.

| µ § |  |
| --- | --- |
| If you want an edited tuple record to be used, click on the appropriate item in this column (or highlight it and press the space bar) to mark it with an 'X'. | To edit a particular tuple, select the appropriate item from this column to display the Edit screen for that item. The edited tuple record does not take effect unless the item is marked with an 'X' in the left column. |

The parameters that can be defined are:

·	CONFIG tuples (Default and Override)

-	Configuration Base Address

-	Configuration Register Mask

·	CFTABLE_ENTRY tuples (Default, Override, and Required)

-	Configuration Index

-	Power Requirements (Vcc and Vpp)

-	Address Range for I/O Windows 0 and 1

-	IRQ Definition

-	Starting Segment, Length, Card Offset, and Memory Window Type for Memory Windows 1 and 2

-	Audio and Power Down support

Note	For more information on CONFIG and CFTABLE_ENTRY tuples, refer to Section 5.2.8 of the PCMCIA PC Card Standard Release 2.10.

The Default settings are those settings that you want to use by default for the card (usually based on PCMCIA standards).  The Override settings override the Default settings.  For example, if a card normally uses a particular Configuration Base Address, you would specify this address on the Default CONFIG tuple screen.  If, however, you want to override the default setting for Configuration Base Address, you would specify the new address to use on the Override CONFIG tuple screen.  In other words, if no Override value is specified for a particular setting, the Default value is used; if an Override value is specified for a particular setting, the Override value is used.

The Required CFTABLE_ENTRY settings are used as a search string to search for matching tuples on the card.  For example, if the card requires a Memory Window with a starting segment at C000:0h and a size of 16K, you would enter the appropriate values for this configuration in the Memory Window 0 section of the Required CFTABLE_ENTRY tuple screen.  When the card is inserted into a slot, the software will search for a tuple record on the card that matches these required values, and will use the matching record to identify the card.

##### CONFIG Tuple

The following screen appears for the <Edit CONFIG> selections.

µ §

This screen is where you define the default (or override) CONFIG tuple for the card.  If an item does not apply to the card you are configuring, select 'Unspec' by clicking on it (or by highlighting it and pressing the space bar).  Otherwise, select 'Specify' and press the TAB key to move the cursor into the field.  Then type in the appropriate value.  When done, click on <OK>.

Configuration Base Address ¾ This field is used to specify the base address of the Configuration Registers.  Up to eight hex digits can be entered in this field.  This information is used in conjunction with the information in the Configuration Register Mask field to determine which registers are present on the card, as well as their location in the card's register space.  The Configuration Base Address can be from one to four bytes, with each two characters in the field representing one byte.  The length of the field depends on the number of bits required to represent the address.

Configuration Register Mask (Presence Mask) ¾ This field is used to specify the presence mask for the Configuration Registers.  This mask is used to determine the presence or absence of the corresponding Configuration Register, with 1 indicating it is present and 0 indicating it is not present.  This information is used in conjunction with the Configuration Base Address field to determine which registers are present on the card, as well as their location in the card's register space.

##### CFTABLE_ENTRY Tuple

The following screen appears for the CFTABLE_ENTRY selections.  CFTABLE_ENTRY tuples enable you to specify all of the possible configurations for a card, and enables the system to distinguish among the possible configurations.

µ §

This screen is where you define the

·	Configuration Index

·	Power Requirements (Vcc and Vpp)

·	Address Range for I/O Windows 0 and 1

·	IRQ Definition

·	Starting Segment, Length, Card Offset, and Memory Window Type for Memory Windows 1 and 2

·	Audio and Power Down support

If an item does not apply to the card you are configuring, click on 'Unspec' (or highlight 'Unspec' and press the space bar).  Otherwise, select the appropriate setting for the card.  If a field is provided for entering a value, use the TAB key to position the cursor in the field, then type the value.  When done, select <OK> to return to the Specify Configuration Parameters screen.

Configuration Index ¾ This byte contains the value that must be written to the Card Configuration Register to enable the configuration described in the tuple record.  It also contains a bit that indicates that the configuration described in the tuple record should be used as the default for the CFTABLE_ENTRY tuples that follow.  Refer to Section 5.2.8.3.1 of the PCMCIA PC Card Standard Release 2.10 for more information on this byte.

Power Requirements ¾ These fields enable you to select the Vcc and Vpp power requirements for the PC card.  The selected value is indicated by a bullet (·).  To select a value, click on the appropriate item, or highlight it and press the space bar, to toggle the bullet on.  Refer to Section 5.2.8.3.5 of the PCMCIA PC Card Standard Release 2.10 for more information on power requirements.

I/O Windows ¾ These settings enable you to specify whether or not I/O windows are used by the card.  To enable an I/O window, select the bus width for the window (8-bit or 16-bit) by clicking on the appropriate value (or highlighting it and pressing the space bar).  Then use the TAB key to move into the "Range:" field, and type in the address range to be reserved for the window.  The values must be in the range of 0h-3FFh.

IRQ Definition ¾ These settings enable you to specify whether or not the card is using an IRQ.  Also, you can select either a specific IRQ or an IRQ mask, and specify the IRQ or IRQ mask in the IRQ Level/Mask field.  To select an IRQ, click on Specific IRQ or IRQ Mask (or highlight it and press the space bar).  Then use the TAB key to move into the IRQ/Level Mask field, and type in the IRQ or IRQ Mask for the card.

Memory Windows ¾ These two sets of fields enable you to specify the Memory Window settings for the card being defined.  If the card uses memory windows, you can specify whether they are Common or Attribute memory windows.  Also, you can specify the starting segment (ranging from C000-EF00, normally in the D000-DFFF range) for each window in the Starting Seg field, the window size (4, 8, ..., 64, in KB) in the Length field, and the offset (normally 0) in the Card Offset field.

Notes	If Start Seg is set to 0 for a network card, the starting segment value is extracted from the CARDID.INI file.

If Length is set to 0 on the Override CFTABLE_ENTRY Tuple screen, it indicates that there is no memory window for the card, even if that window is defined on the Default CFTABLE_ENTRY tuple screen.

Audio Support ¾ If this field is enabled (marked with an 'X'), it indicates that the card allows the BVD2 signal to be used as the audio waveform for the system speaker.  To enable audio support, click on 'Specify' (or highlight it and press the space bar) to toggle the bullet (·) on, then click on 'Audio is supported' (or highlight it and press the space bar) to toggle the 'X' on.

Power Down Support ¾ If this field is enabled, it indicates that the card supports a power-down mode.  To enable power-down support, click on 'Specify' (or highlight it and press the space bar) to toggle the bullet (·) on, then click on 'Power Down is supported' (or highlight it and press the space bar) to toggle the 'X' on.

## Defining Parameters for Executable Files

The Binary Adaptation function enables you to define the parameters for each of the executable components of the software.  This section describes the process for defining the parameters.  For information on each of these parameters, their default values, and the possible values that can be assigned to each, refer to Chapter 3.

To modify the parameters for the PCMCIA software's executable components:

1.	Select Binary Adaptation from the Utility menu, then select Open from the dialog box that is displayed.  A list of all executable files in the CARDSOFT directory is displayed (including Flash File System and CardView files, if these PCMCIA components are installed).

µ §

2.	Select the file whose parameters you want to modify, then select <OK>.  A Parameter List screen similar to the following appears.  The list of parameters differs for each executable file.  The sample screen shown below is the Parameter List for the Card Services executable file (CS.EXE).

µ §

3.	To change a parameter, highlight the parameter and select <Edit>.  If the parameter is a numeric value, an Edit Parameter screen similar to the following is displayed.  If the parameter is a Yes/No value, the screen that is displayed enables you to select the Yes or No value.

Type the new value here. µ § Valid range of values is displayed here.

4.	After editing the parameter, select <OK>.  The new value should now be displayed on the Parameter List screen.

5.	When you are done changing parameters, select <OK>.  A dialog box is displayed.  Select <Save> to save your changes (or select <Done>, then select <Yes>).  If you want to cancel the changes you made, select <Done>, then select <No>.

##### Dumping the Parameter Values Into CONFIG.LST

The Binary Adaptation function allows you to save the executable file parameters into the file CONFIG.LST, which enables you to easily display or print out the list of configuration settings (using a TYPE CONFIG.LST or PRINT CONFIG.LST command).  You can overwrite the existing CONFIG.LST file, or you can append the parameters to the end of the file.

To dump the parameters into CONFIG.LST, select <Dump> from the Parameter List screen.  If CONFIG.LST already exists, a dialog box is displayed, giving you the option to Append to the end of the file (select <Append>) or Replace the existing file (select <Replace>).

Chapter 3

# Configuring Driver Parameters

The following sections describe the parameters for each of the executable components of the PCMCIA software.  The parameters are shown for both the Configuration Utility's Binary Adaptation Program (BAP) and the CONFIG.SYS DEVICEHIGH line.  The Configuration Utility settings are listed on the left, while the DEVICEHIGH line settings are on the right.  For information on how to configure parameters via the BAP, refer to "Defining Parameters for Executable Files" in Chapter 2.

Normally, you should not have to modify these parameters, since CSALLOC can detect the available resources and relay this information to the Card Services software.  However, CSALLOC cannot detect system resources that may be required by system devices if those devices are not activated at the time CSALLOC is run (either from the command line or via the Resource Allocation menu pick in the Configuration Utility).  This could result in a resource conflict when a device becomes active, since resources required for the device may already have been reserved for Card Services.  If this situation occurs, you can use the Configuration Utility or DEVICEHIGH line parameters to ensure that the resources required by these devices are made unavailable to Card Services.  Also, system hardware design and security features may make some available resources undetectable by CSALLOC.  In this situation, you could edit CSALLOC.INI (or create a customized .INI file) to include special keywords to manually include resources that are not being detected (see Chapter 5 of the CardSoft 3.1 Software User's Guide for more information on this capability).

For example, if CSALLOC determines that IRQs 15, 12, 11, 10, 7, and 4 are available for use as the status-change interrupt, but you have an inactive device installed in the system that specifically needs IRQ 7 in order to operate properly, you could change the value for the Card-Status-Change Interrupt IRQ Mask parameter to exclude IRQ 7.  (If however, the device is active at the time CSALLOC is run, CSALLOC would correctly determine that IRQ 7 is not available for Card Services, and would correctly set the Card-Status-Change Interrupt IRQ Mask value to exclude IRQ 7).  As another option, you could edit CSALLOC.INI, and add the line "IRQEXCLUDE=7".  Bo

the methods achiev the same result.

## DEVICEHIGH Line Parameters

Modifications to the executable components can be done by way of adding or modifying DEVICEHIGH line parameters.  Instead of modifying the executable file parameters via the Configuration Utility Binary Adaptation function, parameters are added to the appropriate device driver within CONFIG.SYS.  The following is an example of how the CardSoft device lines might appear in CONFIG.SYS.

devicehigh=c:\cardsoft\ss365sl.exe  /SIRM=FFFF  /SKT=3

devicehigh=c:\cardsoft\cs.exe /BEEPTYPES CF

devicehigh=c:\cardsoft\csalloc.exe c:\cardsoft\csalloc.ini

devicehigh=c:\cardsoft\atadrv.exe

devicehigh=c:\cardsoft\mtddrv.exe /PART=2

devicehigh=c:\cardsoft\ssmsflsh.sys

devicehigh=c:\cardsoft\cardid.exe

install=c:\cardsoft\cs_apm.exe

Note	Parameters consist of an "/idstring" and a "value".  The "/idstring" must always match exactly one of the available strings.  The "value" is entered as ASCII-hexadecimal or ASCII-decimal.  For example, in the example above the /SIRM is the required idstring and FFFF is the value.

## Socket Services

Socket Services provides the standard software interface to the hardware that controls PCMCIA-compatible slots for PC cards.  SystemSoft Socket Services supports the major host adapters that control the slots.  The following section describes all configuration settings available for Socket Services, both from the Configuration Utility and the DEVICEHIGH line.

### Intel 82365SL PCIC (ss365sl.exe or sl365lp.exe)

### IBM ThinkPad 720 (ssibm720.exe)

Card-Status-Change Interrupt IRQ Mask					/SIRM

Range:   0h-FFFFh   							Range:   0h-FFFFh

Default:  DEB8h							Default:  DEB8h

This switch setting depends on the system hardware design and processor chip. It specifies the 16-bit mask of available IRQs that can be assigned as status-change interrupt (event interrupt) IRQs.  The status-change interrupt is generated when the PCMCIA adapter status changes (that is, a card is removed or inserted).  Use this parameter to inform Socket Services of the IRQs that are connected to the Intel PCIC or IBM Thinkpad chip for use as the status-change interrupt signals.  This information should be provided by the manufacturer of the system.

The most significant bit represents IRQ 15 and the least significant bit represents IRQ 0.  For example, a value of DEB8h (1101-1110-1011-1000 binary) indicates that IRQs 15, 14, 12, 11, 10, 9, 7, 5, 4, and 3 are valid for assignment as card-status-change IRQs.

PC Card IREQ Mask							/IIRM

Range:   0h-FFFFh							Range:   0h-FFFFh

Default:  DEB8h							Default:  DEB8h

This switch setting depends on the system hardware design and processor chip. It specifies the 16-bit mask of available IRQs that can be assigned as valid PC card IREQs.  PC card IREQs are interrupts that are required for the operation of the device (modem card, ATA card, etc.) that is inserted into your system.  Use this parameter to inform Socket Services which IRQs are connected to the Intel controller for use as card interrupts.  This information should be provided by the system vendor.

The most significant bit represents IRQ 15 and the least significant bit represents IRQ 0.  For example, a value of DEB8h (1101-1110-1011-1000 binary) indicates that IRQs 15, 14, 12, 11, 10, 9, 7, 5, 4, and 3 are valid for assignment as card-status-change IRQs.

PCMCIA Adapter Base Port Address					/PORT

Range:   0h-3FFh							Range:   0h-3FFh

Default:  3E0h								Default:  3E0h

This switch setting is dependent on the system hardware design.  It specifies the base port address of the first of up to two PCMCIA adapters.  This is the I/O address used by CardSoft to communicate with the PCMCIA Adapter(s).

Voltage Stabilization Delay						/PWRON

Range:   1 through 65535			 			Range:    01h-FFFFh

Default:   6667								Default:   1A0B

Specifies the amount of time, in 15-ms units, that the SS365SL waits for the voltage to stabilize after a voltage change on a slot.  This occurs when a card is inserted into a slot and when a Flash memory card is written to or erased.  The default is 100ms.

Zero Voltage Stabilization Delay	 				/PWROFF

Range:   1 through 65535						Range:  01h-FFFFh

Default:  0								Default:  0

Specifies the amount of time, in 15-ms units, that the SS365SL waits for the voltage to stabilize after voltage has been set to zero for a slot.  This occurs when a card is removed from a slot.

Install As Adapter Zero							/ADA

Range:   Yes or No 							Range: 0 or 1

Default:  No								Default:  1

Forces installation of Socket Services as Adapter Zero.  This feature may be used to override the presence (e.g., in the BIOS) of any previously loaded non-compliant PCMCIA implementations.  The SS365SL installs as the next available Adapter when this parameter is not specified.

Intel 82365SL PCIC ID   						/ID

Range:   0h-FFh							Range: 0h-FFh

Default:   83h (ss365sl)							Default: 83h (ss365sl)

Default:   84h (ss365lp)							Default: 84h (ss365lp)

Default:   88h (ssibm720)						Default: 88h (ssibm720)

Specifies an expected ID byte (Register 0) in addition to the standard values.  The default is no additional IDs.

Number Of Supported Sockets						/SKT

Range:   1-4								Range:   1-4

Default:   0								Default:   0

Specifies the number of slots that the controller supports (1-4).  The default value (0) indicates that Socket Services dynamically determines the number of slots present.  If a number other than zero is specified, it must be set to the exact number of slots present in the system.

Reset On CPU Resume							/RST

Range:   Yes or No							Range:   0 or 1

Default:  No								Default:  1

Controls whether the controller is reset on a CPU Resume event.  A reset returns the controller to its uninitialized power-up state.

Test for Warm Reboot						 	/WRB

Range:   Yes or No						 	Range:   0h or 1h

Default:  No							 	Default:  0

Specifies that Socket Services test for possible system resources depletion due to configured PCMCIA slots during a warm reboot.  If enabled, and configured slots are discovered following a warm reboot, Socket Services clears all slot configurations and reboots the system.

### DataBook TCIC-2/N (ssdbook.exe)

Card-Status-Change Interrupt IRQ Mask					/SIRM

Range:   1h-FFFFh							Range:   1h-FFFFh

Default:  4CF8h							Default:  4CF8h

This switch setting depends on the system hardware design and processor chip. It specifies the 16-bit mask of available IRQs that can be assigned as status-change interrupt (event interrupt) IRQs.  This  interrupt is generated when the adapter status changes (i.e., a card is inserted or removed).  Use this parameter to inform Socket Services of the IRQs that are connected to the DataBook chip for use as the status change interrupt signals.  This information should be provided by the system vendor.

The most significant bit represents IRQ 15 and the least significant bit represents IRQ 0.  For example, a value of 4CF8h (0100-1100-1111-1000 binary) indicates that IRQs 14, 11, 10, 7, 6, 5, 4, and 3 are valid for assignment as card-status-change IRQs.

PC Card IREQ Mask							/IIRM

Range:   1h-FFFFh							Range:   1h-FFFFh

Default:  4CF8h							Default:  4CF8h

This switch setting depends on the system hardware design and processor chip. It specifies the 16-bit mask of available IRQs that can be assigned as valid PC card IREQs.  PC card IREQs are those interrupts that are required for the operation of the device (modem card, ATA card, etc.) that is inserted in the system.  Use this parameter to inform Socket Services which IRQs are connected to the controller for use as card interrupts.  This information should be provided by the system vendor.

The most significant bit represents IRQ 15 and the least significant bit represents IRQ 0.  For example, a value of 4CF8h (0100-1100-1111-1000 binary) indicates that IRQs 14, 11, 10, 7, 6, 5, 4, and 3 are valid for assignment as card-status-change IRQs.

PCMCIA Adapter Base Port Address					/PORT

Range:   0h-3FFh							Range:   0h-3FFh

Default:  240h								Default:  240h

This switch setting is dependent on the system hardware design and processor chip.  It specifies the base port address of the PCMCIA adapters.  This is the I/O address used by CardSoft to communicate with the PCMCIA Adapter(s).

Voltage Stabilization Delay						/PWRON

Range:   1 through 65535						Range:    01h-FFFFh

Default:   6667							Default:   1A0B

Specifies the amount of time, in 15-ms units, to wait for the voltage to stabilize after a voltage change on a slot.  This occurs when a card is inserted into a slot and when a Flash memory card is written to or erased.  The default is 100ms.

Install As Adapter Zero							/ADA

Range:   Yes or No 							Range: 0 or 1

Default:  No								Default:  1

Forces installation of Socket Services as Adapter Zero.  This feature may be used to override the presence (e.g., in the BIOS) of any previously loaded non-compliant PCMCIA implementations.  SSDBOOK installs as the next available Adapter when this parameter is not specified.

Number Of Supported Sockets						/SKT

Range:   1-2								Range:   1-2

Default:   2								Default:   2

Specifies the number of PCMCIA slots that SSDBOOK supports.  On machines that have a PCMCIA adapter that can support more slots than are present in the machine, this value should be set to the exact number of slots present.

HI016 and HMEM16 Control Type					/TOTEM

Range:   0 or 1								Range: 0 or 1

Default:  0								Default:  0

Specifies type of control for HIO16 and HMEM16 ISA bus signals.  0 enables open drain control.  1 enables totem-pole control.

### Cirrus CL-PD67xx (sscirrus.exe)

### Intel AT2092AA PPEC (ssppec.exe)

### Ricoh RF5C266/RF5C366 (ssricoh.exe)

### Vadem VG365/465/468 (ssvadem.exe)

### VLSI VL82C146 (ssvlsi.exe)

Number Of Supported Sockets						/SKT

Range:   1-4								Range:   1-4

Default:   4								Default:   4

Specifies the number of slots that driver supports.  On machines that have a PCMCIA adapter that can support more slots than are present in the machine, this value should be set to the exact number of slots present.

PCMCIA Adapter Base Port Address					/PORT

Range:   0h-3FFh							Range:   0h-3FFh

Default:  3E0h								Default:  3E0h

Specifies the base port address of the PCMCIA adapters.  This is the I/O address used by CardSoft to communicate with the PCMCIA Adapter(s).

PC Card IRQ Mask							/IRM

Range:   1h-FFFFh							Range:   1h-FFFFh

Default:  DEB8h							Default:  DEB8h

This switch setting depends on the system hardware design and processor chip. It specifies the 16-bit mask of available IRQs that can be assigned as valid PC card IRQs.  PC card IRQs are interrupts that are required for the operation of the device (modem card, ATA card, etc.) that is inserted in the system.  Use this parameter to inform Socket Services which IRQs are connected to the controller for use as card interrupts.  This information should be provided by the system manufacturer.

The most significant bit represents IRQ 15 and the least significant bit represents IRQ 0.  For example, a value of DEB8h (1101-1110-1011-1000 binary) indicates that IRQs 15, 14, 12, 11, 10, 9, 7, 5, 4, and 3 are valid for assignment as card-status-change IRQs.

Voltage Stabilization Delay						/PWRON

Range:   1 through 65535						Range:    01h-FFFFh

Default:   6667								Default:   1A0B

Specifies the amount of time, in 15-ms units, to wait for the voltage to stabilize after a voltage change on a slot.  This occurs when a card is inserted into a slot and when a Flash memory card is written to or erased.  The default is 100ms.

Zero Voltage Stabilization Delay					/PWROFF

Range:   1 through 65535						Range:  01h-FFFFh

Default:  0								Default:  0

Specifies the amount of time, in 15-ms units, to wait for the voltage to stabilize after voltage has been set to zero for a slot.  This occurs when a card is removed from a slot.

Install As Adapter Zero							/ADA

Range:   Yes or No 							Range: 0 or 1

Default:  No								Default:  1

Forces installation of Socket Services as Adapter Zero.  This feature may be used to override the presence (e.g., in the BIOS) of any previously loaded non-compliant PCMCIA implementations.  The controller installs as the next available Adapter when this parameter is not specified.

/PULSE 								/PULSE

Range:  0h-1h								Range:  0h-1h

Default:  0h								Default: 0h

Allows interrupts to be generated in pulse mode, rather than level mode.  This parameter applies only to the SSCIRRUS driver.

/5VCORE								/5VCORE

Range:  0h-1h								Range:  0h-1h

Default:  0h								Default: 0h

Used if the chip is being operated with a 5-volt core.  This parameter applies only to the SSCIRRUS driver.

## Card Services (cs.exe)

Card Services software coordinates access to PC cards, PCMCIA slots, and system resources among multiple clients.  These clients may be device drivers, system utilities, or application programs.  Card Services makes access to slot hardware through Socket Services software.

Power-On Delay							/PWR

Range:   1-255								Range:   01h-FFh

Default:  6								Default:  6h

Specifies the power-on delay, in MS-DOS timer ticks, from the application of power during card-insertion processing, until any attempt to access the card.  The default is approximately 660 ms.

Event Processing Delay						/TTD

Range:   1-255								Range:   01h-FFh

Default:  2  								Default:  02h

Specifies a delay, in MS-DOS timer ticks, for processing interrupt-generated events.  An event is only processed if the delay expires without recurrence of the same event.  If the event recurs prior to the timeout, the earlier event is discarded.

Card-Status-Change IRQ						/IRQ

Range:   1-255								Range:   01h-FFh

Default:  10								Default:  0Ah

This switch setting depends on the system hardware design and processor chip.  It specifies the preferred IRQ number to be used for the card-status-change interrupt.  This value is used if Card Services cannot determine a better IRQ number to use for the card-status-change interrupt.  The status-change interrupt is generated when the adapter status changes (i.e., a card is inserted or removed).  The IRQ you should specify with this switch should be provided by the system vendor.

Card-Status-Change Interrupt Mask					/SCIM

Range:   0h-FFh							Range:   0h-FFh

Default:  FFh								Default:  FFh

Specifies the Card-Status-Change interrupts that can be enabled.  This value should be left as FFh unless unusual hardware conditions exist with the Card-Status-Change interrupt.  For such circumstances, a vendor specific function exists to allow all Card-Status-Change interrupts to be enabled on a slot.

Block INT 1A Calls To Socket Services					/BLOCKSS

Range:   Yes or No							Range:   0h or 1h

Default: Yes								Default:  1h

Specifies that direct calls to Socket Services must NOT be allowed via the INT 1A interface. The caller must retrieve an entry point from Card Services to make calls to Socket Services.

/VERSIONTEST							/VERSIONTEST

Range: 0-1								Range: 0-1

Default: 1								Default: 1

This parameter is used for debugging only.  Do not modify the value in this field.

/HEAPSIZE								/HEAPSIZE

Range: 2h-Ah								Range:  2h-Ah

Default: 2h								Default: 2h

Specifies the amount of memory (in KB) that is reserved for Card Services for use as an internal heap, which is used to store information that Card Services needs to operate (for example, the number and type of cards inserted into PCMCIA slots).

/BEEPTYPES								/BEEPTYPES

Range: 0-FFh								Range: 0-FFh

Default: CEh								Default: CEh

This parameter is used to specify the situations in which the system will beep for PC card events.  Card Services can be configured to beep on the selected events in DOS, Windows, or both DOS and Windows.  Card Services can also be configured to beep on card insertion, card removal, configuration errors, and artificial events.  The operating system takes precedence.  For example, if you specify no beeps in both DOS and Windows, and you select beeps for card insertion and removal, no beeps will be emitted when cards are inserted or removed, because no operating system was selected.

The bit values for this parameter determine which operating systems/events are selected:

Bit		Beep Action

7		Beep in DOS

6		Beep in Windows

5		Reserved

4		Reserved

3		Beep on card insertion

2		Beep on card removal

1		Beep on card configuration error

0		Beep on artificial events*

For example, the default value (CEh) sets these bits to 11001110, which causes the system to emit beeps for card insertion, card removal, and card configuration errors in both DOS and Windows.  As another example, the value 8Ch sets these bits to 10001100, which causes the system to emit beeps for card insertion and card removal in DOS only.

*	Artificial events refer to "logical" events (as opposed to physical events, such as physically removing a card).  For example, a card is logically removed when the system suspends, and logically inserted when the system resumes.

/BEEPDELAY								/BEEPDELAY

Range: 0-14h								Range: 0-14h

Default: Fh								Default: Fh

A numeric delay (in .05 second increments) that defines how long Card Services waits for the card client to configure a PC card before attempting to verify that the card was configured.  This setting is intended to account for a latent delay in card clients (such as MTDs) when they are attempting to configure a PC card.  The default value of 15 (Fh) specifies a .75 second delay.  The maximum setting is 20 (14h), which is equivalent to one second.

/TONEFREQ								/TONEFREQ

Range: 2710-FFFFh							Range: 2710-FFFFh

Default: 2E7Ch							Default: 2E7Ch

Defines the frequency of the clock that is used with the system time to generate tones.  The standard value for a U.S. PC is 11900 (2E7Ch).

/POLL									/POLL

Range: 0-1								Range: 0-1

Default: 0								Default: 0

When set to 1, Card Services will not use a card-status-change interrupt to determine status changes on the system.  It will instead poll for status changes (inserted card has been removed, empty slot is now occupied, etc.).  This parameter should be set to 1 if the system does not have an available IRQ to use as a card-status-change interrupt, or if it does not support a card-status-change interrupt.

/EMS									/EMS

Range: 0-1								Range: 0-1

Default: 1								Default: 1

When set to 1, Card Services will attempt to load in EMS (if it is available).  If the system is not configured for EMS, setting this parameter to 1 has no effect.

/MISSTUP								/MISSTUP

Range: 0-FFFFh							Range: 0-FFFFh

Default: 100h								Default: 100h

This parameter determines the number of time GetNextTuple will search before returning NO_MORE_ITEMS when the desired tuple is not found.

## CSALLOC

This is a DOS utility that scans the system for available memory, I/O Port, and IRQ resources.  It then writes this information to the file CSALLOC.INI, which is used by Card Services to determine what system resources are available for PC card configuration

/PPSIG0								/PPSIG0

Range: 0-FFFFh							Range: 0-FFFFh

Default: 5024h								Default: 5024h

Enables OEMs to specify the low-order 16 bits of the Plug-and-Play signature that is expected to be found in the BIOS Plug-and-Play support structure, without making changes to the source code.

/PPSIG1								/PPSIG1

Range: 0-FFFFh							Range: 0-FFFFh

Default: 506Eh								Default: 506Eh

Enables OEMs to specify the high-order 16 bits of the Plug and Play signature that is expected to be found in the BIOS Plug-and-Play support structure, without making changes to the source code.

/IOALIASES								/IOALIASES

Range: 0-3Fh								Range: 0-3Fh

Default: 3h								Default: 3h

Specifies the number of times I/O Port addresses are aliased at 400h and above, and given to Card Services.  For example, if this value is set to 1, and the I/O Port address is 3E8, it is given to Card Services as 3E8 and 7E8.  As another example, the default value (3) with an I/O Port address of 3E8 causes 3E8, 7E8, BE8, and FE8 to be given to Card Services.

/SCANFF								/SON, /SOFF

Range: 0-1h								Range: 0-1h

Default: 1								Default: 1

This parameter is used to alter the memory scan algorithm when searching for memory windows.  A value of 1 (or use of the command line switch /SON) causes the system to scan using the value FFh.  A value of 0 (or use of the command line switch /SOFF) causes the system to scan for any consistent value.

/COM1IRQ								/COM1IRQ

Range:  1-15								Range: 1-15

Default: 4								Default: 4

Specifies the IRQ for COM 1.  This value should be changed only if the system uses an IRQ other than IRQ 4 for COM 1.

/COM2IRQ								/COM2IRQ

Range:  1-15								Range: 1-15

Default: 3								Default: 3

Specifies the IRQ for COM 2.  This value should be changed only if the system uses an IRQ other than IRQ 3 for COM 2.

/COM3IRQ								/COM3IRQ

Range:  1-15								Range: 1-15

Default: 4								Default: 4

Specifies the IRQ for COM 3.  This value should be changed only if the system uses an IRQ other than IRQ 4 for COM 3.

/COM4IRQ								/COM4IRQ

Range:  1-15								Range: 1-15

Default: 3								Default: 3

Specifies the IRQ for COM 4.  This value should be changed only if the system uses an IRQ other than IRQ 3 for COM 4.

/TESTIO								/TESTIO

Range:  0-FFFFh							Range: 0-FFFFh

Default: 80h								Default: 80h

Specifies the I/O address to be used for I/O scanning.  Always specify an I/O address that is safe for performing input or output operations without interference from other components.

## SRAM Driver (MTSRAM.EXE)

MTSRAM is a driver that supports SRAM cards.

/NUMWINS								/NUMWINS

Range:  1-4h								Range: 1-4h

Default: 4h								Default: 4h

Specifies the number of memory windows that will be configured and used for accessing SRAM cards.  The memory windows are set up in upper memory (for exammple, the D000-DFFF range), in any available area that is found.  A value of 2 provides better SRAM card performance than a value of 1.  Optimum performance is achieved with a value of 3.  The total number of memory windows configured is a multiple of this value and the number of SRAM memory cards inserted (and recognized) in the system's PCMCIA slots.

Memory Card Memory Window Size					/SIZ

Values:  4, 8, 16, and 32							Values: 4, 8, 16, or 32

Default:  4								Default: 4

Specifies the size of the system memory window to be used to access SRAM cards.  The value is stated in KB and must be 4, 8, 16, or 32.  The default value of 4 represents a 4KB window.

## Memory Technology Driver (MTDDRV.EXE)

This component must be installed in order to support Flash Memory cards.  It works in conjunction with card-specific MTDs to support a wide variety of current Flash Memory cards.  It also supports SRAM cards (providing MTSRAM.EXE is also installed), and allows sharing of drive letters between the different types of memory cards (Flash, SRAM, and ATA).

Maximum Partitions Per Card							/PART

Range: 1-4									Range: 1-4

Default: 1									Default: 1

Specifies the maximum number of partitions that a Flash Memory or SRAM card can contain (not including any area on the card that may be used for boot code).  In most cases, it is best to use the default value of 1, since there are no significant advantages to having multiple partitions on a memory card (and there are some drawbacks to having multiple partitions).  However, multiple partitions are required for combination Flash/SRAM cards.  For more information on memory card partitions, refer to the Flash File System User's Guide.

## IDE/ATA Device Driver (atadrv.exe)

ATADRV.EXE is a Hard Disk or Flash Disk configuration device driver for removable PCMCIA devices that emulate an IDE type hard disk.  ATADRV.EXE is strictly a Hard Disk or Flash Disk driver that communicates to industry-standard hard disk controllers.  ATADRV.EXE can manage up to eight PCMCIA devices.

ATADRV communicates with the CardID component to obtain information on card insertion and removal.  Once a card is inserted and recognized by CardID, ATADRV communicates with the card as if it were an IDE disk.  ATADRV must be installed before the CARDID.EXE driver.  ATADRV supports the following parameters:

### Configuration Parameters

Slave Mode

Range: No or Yes

Default: No

Allows the driver to be slaved to a master control driver for drive letter selection.  MTD Driver (MTDDRV) is the only master control driver currently available.  When this parameter is used, ATADRV must be installed in CONFIG.SYS before MTDDRV.

Number of Drive Units

Range: 1-8

Default: 2

Specifies the number of drives that the system supports when installed either as a block device driver or as a slave device driver.

### DEVICEHIGH Line Parameters

/S:

Range:  1-8

Default: 2

Example:   devicehigh=c:\cardsoft\atadrv.exe /S:1

Installs the ATADRV device driver as a slave to MTDDRV.  It also specifies the number of drives it can support.  When this switch is used, ATADRV must be installed in CONFIG.SYS before MTDDRV.

## Disk Partitioning Utility (atainit.exe)

ATAINIT.EXE is a disk partitioning utility that must be used to create a partition on any drive supported by ATADRV.  When a new drive is inserted into a socket, it is not recognized since there is no CMOS or other common method to find out its physical parameters.  ATAINIT interrogates the drive to find the physical parameters to use.

Language								/LANG

Range: 0-4								Range:  0-4

Default: 0								Default: 0

The disk utility may be configured to display messages in five different languages:

0 = English, 1 = French, 2 = German, 3 = Italian, 4 = Spanish.

### ATAINIT Return Code

There are five return codes for ATAINIT, which are defined as follows:

| Return Code | Meaning |
| --- | --- |
| 00 | No error |
| 81 | Unable to address card |
| 82 | Bad write |
| 84 | Requested quit via keyboard |
| 88 | Unable to ID the drive |

# Flash File System Executables

This section lists the parameters for each of the Flash File System executables that are configurable.

## Card-Specific Memory Technology Drivers

## MTAA.EXE, MTAB.EXE, MTI1.EXE, MTI2P.EXE

The card-specific memory technology drivers work in conjunction with MTDDRV to support various types of Flash Memory cards.  Card types currently supported are AMD Type A, AMD Type B, Intel Series 1, and Intel Series 2 Plus.

/NUMWINS								/NUMWINS

Range:  1-4h								Range: 1-4h

Default: 4h								Default: 4h

Specifies the number of memory windows that will be configured and used for accessing Flash Memory cards.  The memory windows are set up in upper memory (for exammple, the D000-DFFF range), in any available area that is found.  A value of 2 provides better Flash card performance than a value of 1.  Optimum performance is achieved with a value of 3.  The total number of memory windows configured is a multiple of this value and the number of Flash memory cards inserted (and recognized) in the system's PCMCIA slots.

Memory Card Memory Window Size					/SIZ

Values:  4, 8, 16, and 32							Values: 4, 8, 16, or 32

Default:  4								Default: 4

Specifies the size of the system memory window to be used to access Flash Memory cards.  The value is stated in KB and must be 4, 8, 16, or 32.  The default value of 4 represents a 4KB window.

Appendix A

# CardID

CardID is a Card Services client that provides file-driven detection and configuration services for the different types of PCMCIA cards (fax/modems, ATA cards, network cards, communications, etc.).  CardID utilizes an initialization file, CARDID.INI, that stores configuration information about these cards.

The CardID Configuration file contains two sections.  The "[Common]" section defines the program parameters common to all inserted cards.  The "[Libraries]" section contains references to card libraries of Card Recognition and Configuration records, which drive the process of card recognition and configuration.

## The [Common] Section

The [Common] section consists of lines of the form

parameter=value

where the format of the value depends on the parameter.  Following is table lists all currently-defined parameters, their defaults, and their value types.

Note	The CARDID.INI file you receive may not list all of the parameters shown in this table.  The parameters in your CARDID.INI file depend on your hardware platform and the PCMCIA support you are providing.

CARDID.INI Parameters

| Parameter/Default | Value Type |
| --- | --- |
| Entries for AT-style modem cards |  |
| COMPort1=3F8 | Hexadecimal |
| COMIRQ1=4 | Integer, 1-4 |
| COMPort2=2F8 | Hexadecimal |
| COMIRQ2=3 | Integer, 1-4 |
| COMPort3=3E8 | Hexadecimal |
| COMIRQ3=4 | Integer, 1-4 |
| COMPort4=2E8 | Hexadecimal |
| COMIRQ4=3 | Integer, 1-4 |
| FirstModem=1 | Integer, 1-4 |
| SecondModem=2 | Integer, 1-4 |
| ThirdModem=3 | Integer, 1-4 |
| FourthModem=4 | Integer, 1-4 |
| COMBySocket=No | Yes/No |
| COMBDAAbsolute=Yes | Yes/No |
| Audio=Yes | Yes/No |
| RingIndicate=No | Yes/No |
| Entries for 9800-style COM cards |  |
| 9800COM2IRQ=5 | Integer, 2-15 |
| 9800COM3IRQ=12 | Integer, 2-15 |
| First9800Modem=2 | Integer, 2-3 |
| Second9800Modem=3 | Integer, 2-3 |
| 9800ComBySocket=No | Yes/No |

Continued...

CARDID.INI Parameters   Continued...

| Parameter/Default | Value Type |
| --- | --- |
| Entries for LAN Cards |  |
| LAN1Port=300 | Hexadecimal |
| LAN1IRQ=5 | Integer, 2-15 |
| LAN1MemoryBase1=D8 | Hexadecimal |
| LAN1MemoryBase2=DC | Hexadecimal |
| LAN2Port=310 | Hexadecimal |
| LAN2IRQ=10 | Integer 2-15 |
| LAN2MemoryBase1=0 | Hexadecimal |
| LAN2MemoryBase2=0 | Hexadecimal |
| FirstLAN=1 | Integer, 1-2 |
| SecondLAN=2 | Integer, 1-2 |
| FastTokenRing=Yes | Yes/No |
| Entries for "Other" Cards |  |
| Other1Port=300 | Hexadecimal |
| Other1IRQ=5 | Integer, 2-15 |
| Other1MemoryBase1=D8 | Hexadecimal |
| Other1MemoryBase2=DC | Hexadecimal |
| Other2Port=320 | Hexadecimal |
| Other2IRQ=5 | Integer, 2-15 |
| Other2MemoryBase1=0 | Hexadecimal |
| Other2MemoryBase2=0 | Hexadecimal |
| FirstOther=1 | Integer, 1-2 |
| SecondOther=2 | Integer, 1-2 |
| Entries for ATA Cards |  |
| ATATryLinear=Yes | Yes/No |
| ATATryMemory=No | Yes/No |
| ATATrySecondary=Yes | Yes/No |
| ATATryPrimary=Yes | Yes/No |
| ATAFirstConf=0 | 0=Linear |
| ATASecondConf=1 | 1=Memory |
| ATAThirdConf=2 | 2=Secondary ATA Address |
| ATAFourthConf=3 | 3=Primary ATA Address |
| ATASocketOffset=0 | Integer, 0-63 |
| Miscellaneous Card Configuration Parameters |  |
| DetectNonCompliant=No | Yes/No |
| AllowIOOffsets=No | Yes/No |
| Miscellaneous Entries |  |
| BeepinDOS=Yes | Yes/No |
| BeepinWindows=Yes | Yes/No |
| BeepOnConfigFailure=Yes | Yes/No |
| BeepOnUnrecognizedCard=No | Yes/No |
| ContinueOnConfigFailure=No | Yes/No |
| CardID Parameters |  |
| GrabInitFile=Yes | Yes/No |
| InitBufferSize=512 | Integer |
| UseEMS=Yes | Yes/No |

## The [Libraries] Section

The [Libraries] Section contains lines of the form

CardLib=pathname

where pathname is interpreted relative to the directory containing the initialization file if it is not a full pathname.  The named file (i.e., CARDID.CLB) contains binary data records, each of which is a Comment Record, a Card Services Resource record, or a Card Recognition and Configuration Record.  These record types are defined in the following sections.

### Comment Record

This record allows comments (i.e., any desired text or binary data) to be inserted into a library.

Comment Record

| Offset | Bytes | Description | Value |
| --- | --- | --- | --- |
| 00 | 2 | Record ID # | 00E5h |
| 02 | 2 | Record Length | Bytes in record, including the header |
| 04 | n | Comment | Arbitrary binary data, extending to the end of the record, as defined by the Record Length. |

### Card Services Resource Record

This record describes the resources to be allocated to Card Services.  At most, there should be one Card Services Resource Record.  It is processed when CardID is initialized.  Since resources are supposed to be allocated by CSALLOC, this record should not be necessary.  It has been retained only to provide a workaround if, for some reason, CSALLOC is not functioning properly.  This record consists of a header and a variable number of resource definition fields.

Record Header: Card Services Resource Record

| Offset | Bytes | Description | Value |
| --- | --- | --- | --- |
| 00 | 2 | Record ID # | 01E5h |
| 02 | 2 | Record Length | Bytes in record, including the header |

Field # 00 = Memory Windows Available

| Offset | Bytes | Description | Value |
| --- | --- | --- | --- |
| 00 | 1 | Field ID # | 00h |
| 01 | 2 | Mem Win 1 Size | Size (in paragraphs) of window |
| 03 | 2 | Mem Win 1 Start | First paragraph of window |
| 05 | 1 | Mem Win 1 Flags | Bit 5 = 1 for shared |
| 06 | 2 | Mem Win 2 Size | Size (in paragraphs) of window |
| 08 | 2 | Mem Win 2 Start | First paragraph of window |
| 0A | 1 | Mem Win 2 Flags | Bit 5 = 1 for shared |
| " | " | " | " |
| nn | 2 | End of List flag | Size = 0 marks end of list |

Field # 01 = I/O Windows Available

| Offset | Bytes | Description | Value |
| --- | --- | --- | --- |
| 00 | 1 | Field ID # | 01h |
| 01 | 1 | I/O Win 1 Size | Size in bytes of first I/O window |
| 02 | 2 | I/O Win 1 Start | Start of I/O window 1 |
| 04 | 1 | I/O Win 1 Flags | Bit 0 = 1 for shared |
| 05 | 1 | I/O Win 2 Size | Size in bytes of second I/O window |
| 06 | 2 | I/O Win 2 Start | Start of I/O window 2 |
| 08 | 1 | I/O Win 2 Flags | Bit 0 = 1 for shared |
| " | " | " | " |
| nn | 1 | End of List Flag | Size = 0 marks end of list |

Field # 02 = IRQ Available

| Offset | Bytes | Description | Value |
| --- | --- | --- | --- |
| 00 | 1 | Field ID # | 02h |
| 01 | 2 | IRQ Bit Mask | Bits 15:0 = IRQ 15-0 mask If bit = 0, IRQ is not available If bit = 1, IRQ is available |
| 03 | 2 | Share Bit Mask | Bits 15:0 = IRQ sharing mask If bit = 0, IRQ cannot be shared If bit = 1, IRQ can be shared |
| 05 | 2 | Sharing Type Bit Mask | Bits 15:0 = IRQ sharing type mask If bit = 0, IRQ must be time-multiplexed If bit = 1, IRQ can be dynamically shared Note: This mask is present if some bit in the Share Bit Mask is set. Each bit in this mask is significant only if the corresponding bit in the Share Bit Mask is set. |

### Card Recognition and Configuration Record

A Card Recognition and Configuration record consists of a variable number of recognition fields, followed by a variable number of configuration fields, which should be specified in order of field ID.  If the criteria specified in the recognition fields are satisfied, the card should be configured in accordance with the configuration fields.  There should be one Card Recognition and Configuration record for each type of card to be supported.  The "type" in the previous sentence is a loose term; it can mean "a particular model of a particular card from a particular manufacturer", or something as loose as "any modem".

When a card is inserted, CardID reads Card Recognition and Configuration records in order, and uses the first one whose recognition algorithm succeeds on the card.

The Card Recognition and Configuration record begins with a record header:

Record Header: Card Recognition and Configuration Record

| Offset | Bytes | Description | Value |
| --- | --- | --- | --- |
| 00 | 2 | Record ID # | 03E5h |
| 02 | 2 | Record Length | Bytes in record, including the header |
| 04 | 2 | System Mask | Bitmask of systems to which this record applies. (Bit 0 = DOS, Bit 1 = Windows) |
| 06 | 2 | Socket Mask | Bitmask of logical slots to which this record applies. (Bit 0 = Slot 0, Bit 1 = Slot 1, etc.). |
| 08 | n | Card ID String | ASCIIZ String identifying the card, for use by programs that display and edit this file. |
| nn | n | Card Type String | ASCIIZ String identifying the type of card. If this string is null, the card type string is generated from the card type information specified in a Card Type field, or read from the CIS. |

The Socket Mask field allows different sockets to be treated differently.  For example, a modem in Slot 0 might always be configured as COM2, while a modem in Slot 1 might always be configured as COM3.  This field may be set to FFFF to treat all slots identically.

### Card Recognition Fields

Field # 00 = Recognition Algorithm

| Offset | Bytes | Description | Value |
| --- | --- | --- | --- |
| 00 | 1 | Field ID # | 00h |
| 01 | 1 | Recognition Algorithm | 0 = Use CISTPL_VERS_1 1 = CIS literal comparison 2 = Generic type recognition 3 = Custom code |

If the value is 0 (use CISTPL_VERS_1), then 1 to 4 fields follow, containing values to search for in some subset of the 4 fields of the CISTPL_VERS_1 tuple (manufacturer name, product name, and two "additional product information" fields).  These fields have IDs 1, 2, 3, and 4, respectively.  This algorithm presupposes a card with a CIS that is sufficiently correct that Card Services can return the CISTPL_VERS_1 tuple data to CardID.  All fields must match for the card to be recognized using this algorithm.

If the recognition algorithm is 1 (CIS literal comparison), then a variable number of CIS comparison fields follow, each of which has ID 5.  See the entry for field #05 for the format and use of these fields.  At least one CIS comparison field must be present, and all CIS comparisons must succeed for the card to be recognized using this algorithm.

If the recognition algorithm is 2 (Generic type recognition), then a card type field (ID 06) must follow.  CardID uses its internal recognition algorithm for cards of this type.  If CardID has no internal recognition algorithm for this type, the card is not recognized.  The card types recognized by CardID are modems (type 2) and ATA cards (type 4).

If the recognition algorithm is 3 (Custom code), then a custom recognition code record (ID 07) must follow.

Field # 01 = Manufacturer Name

| Offset | Bytes | Description | Value |
| --- | --- | --- | --- |
| 00 | 1 | Field ID # | 01h |
| 01 | n | Manufacturer's Name | ASCIIZ string (null terminated) |

Field # 02 = Product Name

| Offset | Bytes | Description | Value |
| --- | --- | --- | --- |
| 00 | 1 | Field ID # | 02h |
| 01 | n | Product Name | ASCIIZ string (null terminated) |

Field # 03 = Additional Product Information (1)

| Offset | Bytes | Description | Value |
| --- | --- | --- | --- |
| 00 | 1 | Field ID # | 03h |
| 01 | n | Additional Product Information String | ASCIIZ string (null terminated) |

This field describes the contents of the third string in the CISTPL_VERS_1 tuple (if one is present).

Field # 04 = Additional Product Information (2)

| Offset | Bytes | Description | Value |
| --- | --- | --- | --- |
| 00 | 1 | Field ID # | 04h |
| 01 | n | Additional Product Information String | ASCIIZ string (null terminated) |

This field describes the contents of the fourth string in the CISTPL_VERS_1 tuple (if one is present).

Field # 05 = CIS Comparison

| Offset | Bytes | Description | Value |
| --- | --- | --- | --- |
| 00 | 1 | Field ID # | 05h |
| 01 | 1 | Memory Type | 0 = Common memory 1 = Attribute memory |
| 02 | 4 | Offset | Offset from start of memory (includes even and odd bytes, in both attribute and common memory) |
| 06 | 1 | Length | Number of bytes of comparison data |
| 07 | n | Comparison Data | Bytes to compare |

This field specifies an area of the card data to compare against the specified data.  If attribute memory is being compared, the Offset should be even, and odd bytes in attribute memory space are skipped.  All bytes of the comparison data are used, in both common and attribute memory.

Field # 06 = Card Type

| Offset | Bytes | Description | Value |
| --- | --- | --- | --- |
| 00 | 1 | Field ID # | 06h |
| 01 | 1 | Card Type | Card Type |

Field # 07 = Custom Recognition Code

| Offset | Bytes | Description | Value |
| --- | --- | --- | --- |
| 00 | 1 | Field ID # | 07h |
| 01 | 2 | Custom Code Length | # bytes in custom code |
| 03 | n | Custom Recognition Code | Variable length custom recognition code |

The Custom Recognition code is position-independent code called by CardID when this record is read during card insertion processing.  The interface is as follows:

Entry point:

At the start of the record data (like a COM file).  At entry, DS = CS = SS (tiny model).

Inputs:

AX	Pointer to beginning of custom code (call is via CALL AX)

BX	Pointer to a common_info structure

CX	System type (0 = DOS, 1 = Windows)

DX	Client Handle for CardID

SI	Pointer to a SOCKET_DATA structure for this slot

Outputs:

AX	Return value (0 = success, 2 = failure)

Registers that must be preserved:

BP, DI, DS

The custom recognition code may make Card Services calls using the Client Handle passed in DX, but must free any Card Services resources it allocates before returning.

### Card Configuration Fields

These fields describe the configuration to be performed when a card is recognized by the Card Recognition algorithm of this Card Recognition and Configuration Record.

Field # 10 = Configuration Algorithm

| Offset | Bytes | Description | Value |
| --- | --- | --- | --- |
| 00 | 1 | Field ID # | 10h |
| 01 | 1 | Configuration Algorithm | 0 = Do not configure 1 = Use internal configuration algorithm 2 = Configure as specified 3 = Reserved 4 = Custom configuration 5 = Internal + Custom 6 = Specific + Custom |
| 02 | 1 | Status Interrupt Mask | Mask to pass to Card Services for enabling status interrupt sources. Should be FF for cards that drive all status lines properly. The bits are as described in the Socket Services specification for SetSocket. |

If the configuration algorithm is 0 (do not configure), the card should be rejected and no configuration should be attempted.  This is like the "ignore" function of the current CardID.

If the configuration algorithm is 1 (use internal configuration algorithm), a Card Type field (ID 6) follows, to define what sort of card it is.  This field may be omitted if a Card Type field was specified in the recognition section, or if a CISTPL_FUNCID tuple (possibly modified by one or more CISTPL_FUNCE tuples) is present in the CIS.  CardID chooses I/O windows and IRQ assignments from its internal configuration code for this card type.  If it has no internal configuration code for this card type, the card is rejected.

If the configuration algorithm is 2 (configure as specified), an optional CIS requirements field (ID 11), an optional CIS default field (ID 12), and an optional CIS override field (ID 13) follow.  These fields, combined with the actual CIS, if present, specify the card configuration, as follows:

1.	The CIS default field, if present, is read into a default CIS information structure.

2.	Unspecified fields are set to their backup values, as defined by the PCMCIA PC Card Specification.

3.	The CIS is scanned, with initial default values for all fields set as if the values in the default CIS information structure had been specified in a CISTPL_CFTABLE_ENTRY tuple with the "default" bit set.

4.	If the CIS requirements field is present, the first CISTPL_CFTABLE_ENTRY tuple matching the requirements is selected, and read into a working CIS information structure.  Otherwise, the first CISTPL_CFTABLE_ENTRY tuple found is read.  If no tuple is read, the working CIS information structure is copied from the default CIS information structure.

5.	If a CIS override field is present, the override fields are copied into the working CIS information structure.

6.	Finally, the working CIS information structure is analyzed to determine if it contains enough information to perform a configuration.  If so, the card is configured.  If not, the card is not configured.

If the configuration algorithm is 4 (custom config), a custom configuration field (ID 14) follows.  The code in that field is used to configure the card.

If the configuration algorithm is 5 (internal + custom), the internal configuration algorithm is run.  If it succeeds, the custom configuration code is then called.

If the configuration algorithm is 6 (specific + custom) the specific configuration algorithm is run.  If it succeeds, the custom configuration code is then called.

Field # 11 = CIS requirements

| Offset | Bytes | Description | Value |
| --- | --- | --- | --- |
| 00 | 1 | Field ID # | 11h |
| 01 | 2 | Field Mask | Bitmask defining which CIS fields are significant. |
| 03 | n | CE | CISTPL_CFTABLE_ENTRY image, with required values in the fields marked as significant. |

The Field Mask determines which fields in CISTPL_CONF and CISTPL_CFTABLE_ENTRY images are valid.  The format of a Field Mask is identical for Fields 11, 12, and 13, and is as follows:

Bit 0:	Configuration registers base address valid (CONF)

Bit 1:	Configuration register presence mask valid (CONF)

Bit 2:	Configuration index valid (CE)

Bit 3:	Vcc power description valid (CE)

Bit 4:	Vcc and Vpp power descriptions valid (CE)

Bit 5:	I/O space description valid (CE)

Bit 6:	IRQ description valid (CE)

Bit 7:	Memory space description valid (CE)

Bit 8:	Miscellaneous-fields structure valid (CE)

In this field, and in the other fields that use tuple images, the image is stored complete, with tuple code and tuple length fields, to allow the same code in CardID that processes CIS tuples to process tuple images.  The tuple images conform to the PCMCIA PC Card Standard, Release 2.10, with the following extra semantics:

·	If the I/O base of an I/O range has Bit 31 set, that range is considered to require a 16-bit data path, regardless of the setting of the Bus8 and Bus16 bits of TPCE_IO.

·	If the length of a memory range has Bit 31 set, that range is to be mapped to attribute memory space, rather than common memory space.  It is presumed that attribute memory space requires an 8-bit data path, and common memory space requires a 16-bit data path.

Field # 12 = CIS defaults

| Offset | Bytes | Description | Value |
| --- | --- | --- | --- |
| 00 | 1 | Field ID # | 12h |
| 01 | 2 | Field Mask | Bitmask defining which CIS fields are significant. |
| 03 | n | CONF | CISTPL_CONFIG image. Present only if at least one CONF bit is set in the bitmask. |
| mm | n | CE | CISTPL_CFTABLE_ENTRY image, with special default values in the fields marked as significant. Present only if at least one CE bit is set in the bitmask. |

Field # 13 = CIS override

| Offset | Bytes | Description | Value |
| --- | --- | --- | --- |
| 00 | 1 | Field ID # | 12h |
| 01 | 2 | Field Mask | Bitmask defining which CIS fields are significant. |
| 03 | n | CONF | CISTPL_CONFIG image. Present only if the configuration base bit is set in the bitmask. |
| mm | n | CE | CISTPL_CFTABLE_ENTRY image, with override values in the fields marked as significant. Present only if at least one CE bit is set in the bitmask. |

Field # 14 = Custom Configuration Code

| Offset | Bytes | Description | Value |
| --- | --- | --- | --- |
| 00 | 1 | Field ID # | 14h |
| 01 | 2 | Custom Code Length | # bytes in custom code |
| 03 | n | Custom Code | Variable length custom configuration code |

The Custom Configuration code is position-independent code called by CardID to configure a card.  The interface is as follows:

Entry point:

At the start of the record data (like a COM file).  At entry, CS = DS = SS (tiny model).  The call is a near call.

Inputs:

AX	Pointer to beginning of custom code

BX	Pointer to a common_info structure

CX	System type (0 = DOS, 1 = Windows)

DX	Client Handle for CardID

SI	Pointer to a SOCKET_DATA structure for this slot

Outputs:

AX	Return value (0 = success, 4 = failure)

Registers that must be preserved:

BP, DI, DS

The custom configuration code may make Card Services calls using the Client Handle passed in DX.  If it allocates Card Services resources, it should store the corresponding request structures in the appropriate fields of the SOCKET_DATA structure, so they can be freed when the card is removed.

Field # 15 = Custom Removal Code

| Offset | Bytes | Description | Value |
| --- | --- | --- | --- |
| 00 | 1 | Field ID # | 15h |
| 01 | 2 | Custom Code Length | # bytes in custom code |
| 03 | n | Custom Removal Code | Variable length custom removal code |

Normally, on card removal, CardID releases the configuration.  In addition, if any special code was performed on insertion (e.g., setting up the BIOS Data Area for a modem), it must be undone on removal.  This field defines custom removal code for cards that need it.  It may be present even if the card was not configured with custom code.  The Custom Removal code is position-independent code called by CardID when a card is removed that was recognized and configured according to this record.  The interface is as follows:

Entry point:

At the start of the record data (like a COM file).  At entry, DS = CS = SS (tiny model).

Inputs:

AX	Pointer to beginning of custom code (call is via CALL AX)

BX	Pointer to a common_info structure

CX	System type (0 = DOS, 1 = Windows)

DX	Client Handle for CardID

SI	Pointer to a SOCKET_DATA structure for this slot

Outputs:

None.

Registers that must be preserved:

BP, DI, DS

The custom removal code may make Card Services calls using the Client Handle passed in DX, but must free any Card Services resources it allocates before returning.  Custom removal code need not free Card Services resources described in the SOCKET_DATA structure; those will be freed by CardID after the custom removal code returns.

Field # 16 = I/O Redirection

| Offset | Bytes | Description | Value |
| --- | --- | --- | --- |
| 00 | 1 | Field ID # | 16h |
| 01 | 2 | Field Mask | 0020h |
| 03 | n | CE | CISTPL_CFTABLE_ENTRY image, with card base values in the I/O base fields. No other fields are significant. |

Appendix B

Card Services Vendor-Specific API Definitions

This Appendix contains the Card Services Application Programming Interface (API) definitions for SystemSoft vendor-specific functions.

# Subcomponent IDs

The following table lists the Card Services API subcomponent IDs.

| Subcomponent ID | Subcomponent |
| --- | --- |
| 0 | Card Services Special API |
| 1 | Power Management |
| 2 | Ring Resume |
| 3 | Reserved |
| 4 | Reserved |
| 5 | CardBoot |
| 6 | BeepControl |
| 7 | GetWindowInfo |

# RegisterSpecialClient

CardServices (VendorSpecific, null/ClientHandle, ClientEntry,  
    ArgLength, ArgPointer)

Card Services argument binding:

Inputs

[AL]	= Function (VendorSpecific - 34h)

[DX]	= Handle (Null)

[DI]:[SI]	= ClientEntry, [DI] is 16-bit segment, [SI] is 16-bit offset

[CX]	= ArgLength

[ES]:[BX]	= ArgPointer, [ES] is 16-bit segment, [BX] is 16-bit offset

Outputs

[AX]	= Status

[DX}	= ClientHandle

[CF]	= Reset to zero when request was successful, set to one when request failed

All entry registers are preserved unless the same register is used for a return value.

All calls and returns are FAR.

All pointers are SEGMENT16:OFFSET16.

Card Services is invoked by INT 1Ah with [AH] = AFh.

This function is used to make vendor-specific Card Services requests for registering with Card Services clients that have special requirements.

Argument Packet Definition

| Offset | Field | Size | Type | Value | Description |
| --- | --- | --- | --- | --- | --- |
| 0 | InfoLen | 2 | I/O | N | Length of returned info in packet |
| 2 | SubComponentID | 1 | I | 0 | Special Card Services API Subcomponent ID |
| 3 | FunctionID | 1 | I | 10h | RegisterClient Function ID |
| 4 | Argument Length | 2 | I | N | Length of RegisterClient Argument Packet |
| 6 | Argument Packet | *N | I | N | RegisterClient Argument Packet |
| *N+6 | Special Attributes | 4 | I | N | Bit-mapped (defined in the following table) |
| *N+10 | Callback Offset | 2 | I | N | Vendor-specific Client Callback Entry |
| *N+12 | Callback Segment | 2 | I | N |  |

*N = Size of RegisterClient Argument Packet

The Special Attributes field is bit-mapped.  It identifies the special handling the client requires:

| Bit | Function |
| --- | --- |
| 31:2 | Reserved (reset to zero) |
| 1 | Issue vendor-specific callback for card configurations |
| 0 | Issue card removal events for powered-off sockets |

Function ID is used to indicate Ring Resume Disable (1) or Enable (2).

# SpecialGetStatus

CardServices (VendorSpecific, null, null, ArgLength, ArgPointer)

Card Services argument binding:

Inputs

[AL]	= Function (VendorSpecific - 34h)

[DX]	= Handle (Null)

[DI]:[SI]	= ClientEntry, [DI] is 16-bit segment, [SI] is 16-bit offset

[CX]	= ArgLength

[ES]:[BX]	= ArgPointer, [ES] is 16-bit segment, [BX] is 16-bit offset

Outputs

[AX]	= Status

[CF]	= Reset to zero when request was successful, set to one when request failed

All entry registers are preserved unless the same register is used for a return value.

All calls and returns are FAR.

All pointers are SEGMENT16:OFFSET16.

Card Services is invoked by INT 1Ah with [AH] = AFh.

This function is used to make vendor-specific Card Services requests for retrieving socket status information from Card Services that contains special information.

Argument Packet Definition

| Offset | Field | Size | Type | Value | Description |
| --- | --- | --- | --- | --- | --- |
| 0 | InfoLen | 2 | I | N | Length of returned info in packet |
| 2 | SubComponentID | 1 | I | 0 | Special Card Services API Subcomponent ID |
| 3 | FunctionID | 1 | I | 0Ch | GetStatus Function ID |
| 4 | Argument Length | 2 | I | N | Length of GetStatus Argument Packet |
| 6 | Argument Packet | *N | I/O | N | GetStatus Argument Packet |
| *N+6 | Configuration State | 2 | O | N | Bit-mapped (defined in the following table) |

*N = Size of GetStatus Argument Packet

The Configuration State field is bit-mapped.  It identifies the socket configuration state:

| Bit | Function |
| --- | --- |
| 15:5 | Reserved (reset to zero) |
| 4 | Memory Handle assigned to Socket (via OpenMemory) |
| 3 | MTD is registered on Socket |
| 2 | Socket is in I/O mode |
| 1 | Socket is configured |
| 0 | Socket has memory window(s) |

# Suspend/Resume

CardServices (VendorSpecific, null, null, ArgLength, ArgPointer)

Card Services argument binding:

Inputs

[AL]	= Function (VendorSpecific - 34h)

[DX]	= Handle (Null)

[DI]:[SI]	= Pointer, [DI] is 16-bit segment, [SI] is 16-bit offset (Null)

[CX]	= ArgLength (4)

[ES]:[BX]	= ArgPointer, [ES] is 16-bit segment, [BX] is 16-bit offset

Outputs

[AX]	= Status

[CF]	= Reset to zero when request was successful, set to one when request failed

All entry registers are preserved unless the same register is used for a return value.

All calls and returns are FAR.

All pointers are SEGMENT16:OFFSET16.

Card Services is invoked by INT 1Ah with [AH] = AFh.

This function is used to make vendor specific Card Services requests for Suspend/Resume (if FunctionID is 1 or 2) or Socket Suspend/Resume (if FunctionID is 3 or 4).

Argument Packet Definition

| Offset | Field | Size | Type | Value | Description |
| --- | --- | --- | --- | --- | --- |
| 0 | InfoLen | 2 | I/O | 4 | Length of returned info in packet |
| 2 | SubComponentID | 1 | I | 1 | Power Management SubComponent ID |
| 3 | FunctionID | 1 | I | N | Suspend/Resume Function ID |

Function ID is used to indicate Controller Suspend (1), Controller Resume (2), Socket Suspend (3), or Socket Resume (4).  The Socket field is required only when FunctionID is set to 3 or 4.

# Socket Suspend/Resume

CardServices (VendorSpecific, null, null, ArgLength, ArgPointer)

Card Services argument binding:

Inputs

[AL]	= Function (VendorSpecific - 34h)

[DX]	= Handle (Null)

[DI]:[SI]	= Pointer, [DI] is 16-bit segment, [SI] is 16-bit offset (Null)

[CX]	= ArgLength (6)

[ES]:[BX]	= ArgPointer, [ES] is 16-bit segment, [BX] is 16-bit offset

Outputs

[AX]	= Status

[CF]	= Reset to zero when request was successful, set to one when request failed

All entry registers are preserved unless the same register is used for a return value.

All calls and returns are FAR.

All pointers are SEGMENT16:OFFSET16.

Card Services is invoked by INT 1Ah with [AH] = AFh.

This function is used to make vendor specific Card Services requests for Suspend/Resume (if FunctionID is 1 or 2) or Socket Suspend/Resume (if FunctionID is 3 or 4).

Argument Packet Definition

| Offset | Field | Size | Type | Value | Description |
| --- | --- | --- | --- | --- | --- |
| 0 | InfoLen | 2 | I/O | 4 | Length of returned info in packet |
| 2 | SubComponentID | 1 | I | 1 | Power Management SubComponent ID |
| 3 | FunctionID | 1 | I | N | Suspend/Resume Function ID |
| 4 | Socket | 2 | I | N | Logical Socket Number |

Function ID is used to indicate Controller Suspend (1), Controller Resume (2), Socket Suspend (3), or Socket Resume (4).  The Socket field is required only when FunctionID is set to 3 or 4.

# Ring Resume

CardServices (VendorSpecific, null, null, ArgLength, ArgPointer)

Card Services argument binding:

Inputs

[AL]	= Function (VendorSpecific - 34h)

[DX]	= Handle (Null)

[DI]:[SI]	= Pointer, [DI] is 16-bit segment, [SI] is 16-bit offset (Null)

[CX]	= ArgLength (6)

[ES]:[BX]	= ArgPointer, [ES] is 16-bit segment, [BX] is 16-bit offset

Outputs

[AX]	= Status

[CF]	= Reset to zero when request was successful, set to one when request failed

All entry registers are preserved unless the same register is used for a return value.

All calls and returns are FAR.

All pointers are SEGMENT16:OFFSET16.

Card Services is invoked by INT 1Ah with [AH] = AFh.

This function is used to make vendor-specific Card Services requests for Ring Resume.

Argument Packet Definition

| Offset | Field | Size | Type | Value | Description |
| --- | --- | --- | --- | --- | --- |
| 0 | InfoLen | 2 | I/O | 6 | Length of returned info in packet |
| 2 | SubComponentID | 1 | I | 2 | Ring Resume SubComponent ID |
| 3 | FunctionID | 1 | I | N | Ring Resume Function ID |
| 4 | Socket | 2 | I | N | Logical Socket |

Function ID is used to indicate Ring Resume Disable (1) or Enable (2).

# CardBoot

CardServices (VendorSpecific, null, null, ArgLength, ArgPointer)

Card Services argument binding:

Inputs

[AL]	= Function (VendorSpecific - 34h)

[DX]	= Handle (Null)

[DI]:[SI]	= Pointer, [DI] is 16-bit segment, [SI] is 16-bit offset (Null)

[CX]	= ArgLength (6)

[ES]:[BX]	= ArgPointer, [ES] is 16-bit segment, [BX] is 16-bit offset

Outputs

[AX]	= Status

[CF]	= Reset to zero when request was successful, set to one when request failed

All entry registers are preserved unless the same register is used for a return value.

All calls and returns are FAR.

All pointers are SEGMENT16:OFFSET16.

Card Services is invoked by INT 1Ah with [AH] = AFh.

This function is used to make vendor specific Card Services requests for getting information regarding the existence and status of the CardBoot product.

Argument Packet Definition

| Offset | Field | Size | Type | Value | Description |
| --- | --- | --- | --- | --- | --- |
| 0 | InfoLen | 2 | I/O | N | Length of returned info in packet |
| 2 | SubComponentID | 1 | I | 5 | Resource Management SubComponent ID |
| 3 | FunctionID | 1 | I | 0 | N/A |
| 4 | Resource Type | 1 | O | N | CardBoot Resource Type |
| 5 | Boot Status | 1 | O | N | Bits 2-0: 000 = SRAM/Series II Flash 001 = Floppy Disk Card 010 = ATA Card Bits 6-3 are unused Bit 7 (Boot Status Bit) 0 = System not booting from PCMCIA card 1 = System booting from PCMCIA card |
| 6 | ID String Segment | 2 | O | N | Segment of address of CardBoot ID string |
| 8 | ID String Offset | 2 | O | N | Offset of address of CardBoot ID string |
| 10 | Adapter | 1 | O | N | Physical adapter number being used by CardBoot |
| 11 | Socket | 1 | O | N | Physical socket being used by CardBoot |
| 12 | Window | 1 | O | N | Physical window being used by CardBoot |
| 13 | Base | 2 | O | N | Base address of window being used by CardBoot |
| 15 | Size | 2 | O | N | Size of window being used by CardBoot |
| 17 | IREQ Information | 2 | O | N | Bimap (see below) |

The Resource Type field identifies the CardBoot resource type:

0 = Memory Resource

1 = I/O Resource

2 = IRQ Resource

The IREQ Information field is a bitmap.  If Resource Type=2, it is defined as follows:

| Bit | Description |
| --- | --- |
| 15:11 | Unused |
| 10 | DMA Usage: 0=DMA not used 1=DMA used |
| 9 | I/O Interface Type: 1=I/O Interface |
| 8 | Memory Interface Type: 1=Memory Interface |
| 7:5 | Unused |
| 4:0 | IREQ being used by CardBoot |

# BeepControl

CardServices (VendorSpecific, null, null, ArgLength, ArgPointer)

Card Services argument binding:

Inputs

[AL]	= Function (VendorSpecific - 34h)

[DX]	= Handle (Null)

[DI]:[SI]	= Pointer, [DI] is 16-bit segment, [SI] is 16-bit offset (Null)

[CX]	= ArgLength (8)

[ES]:[BX]	= ArgPointer, [ES] is 16-bit segment, [BX] is 16-bit offset

Outputs

[AX]	= Status

[CF]	= Reset to zero when request was successful, set to one when request failed

All entry registers are preserved unless the same register is used for a return value.

All calls and returns are FAR.

All pointers are SEGMENT16:OFFSET16.

Card Services is invoked by INT 1Ah with [AH] = AFh.

This function is used to make vendor specific Card Services requests for BeepControl version 2.10.

Argument Packet Definition

| Offset | Field | Size | Type | Value | Description |
| --- | --- | --- | --- | --- | --- |
| 0 | InfoLen | 2 | I/O | 8 | Length of returned info in packet |
| 2 | SubComponentID | 1 | I | 6 | Beep Control Subcomponent ID |
| 3 | FunctionID | 1 | I | 0 | N/A |
| 4 | Request | 2 | I/O | N | Set or retrieve beep settings |
| 6 | Setting | 2 | I/O | N | Beep settings |

The Request field is defined as follows:

0=Request to retrieve current beep settings

1=Reqest to set beep settings

The Setting field is a bitmap, defined as follows:

| Bit | Function |
| --- | --- |
| 15:8 | Reserved for future use |
| 7 | Beep in DOS (overrides bits 3,2,1) |
| 6 | Beep in Windows (overrides bits 3,2,1) |
| 5:4 | Reserved for future use |
| 3 | Beep on card insertion |
| 2 | Beep on card removal |
| 1 | Beep on configuration conflict |
| 0 | Beep on artificial events |

# GetWindowInfo

CardServices (VendorSpecific, null, null, ArgLength, ArgPointer)

Card Services argument binding:

Inputs

[AL]	= Function (VendorSpecific - 34h)

[DX]	= Handle (Null)

[DI]:[SI]	= Pointer, [DI] is 16-bit segment, [SI] is 16-bit offset (Null)

[CX]	= ArgLength

[ES]:[BX]	= ArgPointer, [ES] is 16-bit segment, [BX] is 16-bit offset

Outputs

[AX]	= Status

[CF]	= Reset to zero when request was successful, set to one when request failed

All entry registers are preserved unless the same register is used for a return value.

All calls and returns are FAR.

All pointers are SEGMENT16:OFFSET16.

Card Services is invoked by INT 1Ah with [AH] = AFh.

This function is used to retrieve information about memory windows in use on a specific slot.

Argument Packet Definition

| Offset | Field | Size | Type | Value | Description |
| --- | --- | --- | --- | --- | --- |
| 0 | InfoLen | 2 | I/O | N | Length of returned info in packet |
| 2 | SubComponentID | 1 | I | 7 | Get Information Subcomponent ID |
| 3 | FunctionID | 1 | I | 0 | Get Memory Window Info Function ID |
| 4 | Socket | 2 | I | N | Logical Socket |
| 6 | WindowEntryCnt | 2 | I/O | N | Number of entries in the window array |
| 8 | WindowEntryArray | N | O | N | Array of window entries |

The WindowEntryCnt field specifies the number of entries in the window array as input, and specifies the number of possible entries returned as output (which may exceed the input).

The WindowEntry has the following structure:

| Offset | Field | Size | Type | Value | Description |
| --- | --- | --- | --- | --- | --- |
| 0 | Base | 4 | O | N | System Base Address |
| 4 | Size | 4 | O | N | Memory Window Size |
| 8 | Card Offset | 4 | O | N | Card Offset Address |
| 10 | Attributes | 2 | O | N | (See RequestWindow Attributes) |
| 11 | AccessSpeed | 1 | O | N | Window Speed Field |
| 12 | Page | 1 | O | N | Page Number |

Appendix C

Socket Services Vendor-Specific API Definitions

This Appendix contains the Socket Services Application Programming Interface (API) definitions for Vendor-specific functions.

# Subcomponent IDs

The following table lists the supported subcomponent IDs and subfunctions.

| Subcomponent ID | Subfunction |
| --- | --- |
| 2 | Set/reset DMA mode |
| 100 | Get CardBoot information |
| 8000* | Reinitialize adapter (after resume) |
| 8001* | Set/reset Ring Indicate resume mode |

* SystemSoft standard subfunctions.

# Vendor Specific Functions

Inputs:

[AL]       	Adapter Number

All other registers are vendor-specific

[SI]        	Subfunction ID

Outputs:

[CF]       	Error

[AL]   	Adapter Number

[AH] 	Error code

01 = BAD_ADAPTER

# Set/Reset DMA Mode

Inputs:

[BL]      	Slot Number

[BH]     	DMA Control Bits

[7:6]	DMA Mode

00	Non-DMA mode

01	DMA (DREQ = -INPACK)

10	DMA (DREQ = WP/-IOIS16)

11	DMA (DREQ = SPKR/LED)

[5:3]	Reserved (set to zero)

[2:0]	000	Use DRQ0

001	Use DRQ1

010	Use DRQ2

011	Use DRQ3

100	No DMA request steering

[SI]	2h

# Get CardBoot Info

Inputs:

[AH]	AEh (vendor specific code)

[AL]	Adapter Number

[SI]	100h

This function serves as a pass-through to CardBoot for applications requesting the Get CardBoot Info function.

Outputs:

If [CY] = 1		CardBoot ROM not present

[AH] 	RETCODE

15 = BAD_FUNCTION

Else [CY] = 0	Successful

[AH] 	Physical adapter number used by CardBoot

[AL] 	Boot Status Flag

[7]	Boot Status

0	System is not booting from PCMCIA card

1	System is booting from PCMCIA card

[6:3]	Unused

[2:0]	Card Type of booting card

000	SRAM/Series II Flash Card

001	Floppy Disk Card

[ES:DI]	Pointer to CardBoot ID string

[DH]	Boot window

[DL]	Boot slot

[BX]	Base address of used resource

(4KB units for memory, absolute address for I/O)

[CX]	Size of used resource

(4KB units for memory, absolute address for I/O)

[SI]	IREQ information

[15:11]	Unused

[10]	DMA Usage

0	DMA not used

1	DMA used

[9]	Interface type = I/O

1	I/O interface

[8]	Interface type = memory

1	Memory

[7:4]	Unused

[3:0]	IRQ used

# Re-Initialize Adapter After Resume

Inputs:

[AH]	AEh (Vendor specific code)

[AL]	Adapter number

[SI]	8000h

# Set/Reset Ring Indicate Mode

Inputs:

[AH]	AEh (Vendor specific code)

[AL]	Adapter number

[SI]	8001h

[BL]	Slot number

[DL]	Control flags

[7:1]	Reserved

[0]	Set/reset Ring Indicate mode

0 = Reset

1 = Set

Outputs:

[CF]	Status

0 = Success

1 = Error

[AH]	RETCODE

0B = BAD_SOCKET

15 = BAD_FUNCTION

Appendix D

# SystemSoft CardSoft API for Windows

This appendix describes the interface to Card Services that is provided for Windows 3.1 applications via the SystemSoft API for Windows.  These libraries and drivers enable Windows 3.1 applications to register as Card Services clients, thereby allowing these applications to receive callbacks when PC cards are inserted into or removed from a slot.

The following illustrates the hierarchy of the libraries and drivers of the SystemSoft API for Windows.

µ §

## Component Descriptions

Socket Services provides the interface to the socket adapter hardware.  Card Services provides the software interface that follows the PCMCIA Card Services Specification (Version 2.10).  CARDID.EXE is the Card Installation Client driver that provides detection and configuration services for certain PCMCIA cards.  CardID provides much of the information that is displayed by the Windows application.  All of these components comprise the SystemSoft CardSoft PCMCIA software suite.  These drivers are loaded at boot-up time via lines in CONFIG.SYS.  As a result, they run in Virtual 86 mode under Windows.

The most difficult problem posed by having a Windows application communicate with Card Services and Socket Services is the crossing back and forth over the line between protected mode and Virtual 86 mode.  The virtual driver SSVRDD.386 (which stands for SystemSoft Virtual Removable Device Driver) provides the mapping of parameters and calls across this line.  The work accomplished by SSVRDD is transparent to the Windows application.  The application never makes direct calls to this virtual driver.

The file SSVCD.386 is a modified version of the Virtual Comm Driver VCD.386.  When the SSCOMM driver makes a request for a COM port, SSVCD.386 returns "OK" if and when the request is for a "removable" COM port.

In the 16-bit protected mode layer, SSCOMM.DRV is a PCMCIA-aware version of the Windows COMM.DRV.  It handles all of the regular duties of the COMM driver.  In addition, it must have knowledge about which devices are removable (through SYSTEM.INI) and which devices are present (through calls to Card Services).  When this driver loads, it registers as a Card Services client, and is therefore informed of subsequent card insertions and removals.  If an application (such as Terminal) accesses a COM port, this driver must handle the possibility that the card is no longer present.  It must recover without allowing the system to hang on I/O access, and it must inform the user that the card is not present.

The dynamic link library SSWINCS.DLL simply provides the mapping of a "C" call to invoke Card Services to the Pascal format.

## Card Services API Description

The Card Services API that is contained in the file SSWINCS.DLL consists of one function with the following prototype:

DWORD FAR InvokeCardServices( BYTE Function, WORD Handle,

WORD far *Pointer, WORD ArgLength, WORD far *ArgPointer );

This function is really the entire API to Card Services.  All calls that adhere to the Card Services specification are called through this function.

The following type definitions are contained in a header file:

typedef WORD (far *CallbackPtr)();

typedef DWORD (far *PCMCIA_API)(BYTE Function, WORD Handle, \

WORD far *Pointer, WORD ArgLength, WORD far *ArgPointer);

The pointer to the "InvocCardServices" function is declared as a global:

PCMCIA_API InvocPtr;              // the Card Services invocation call

In the initialization function, where the windows class is defined and the main window is created, we load the SSWINCS.DLL library and get the proc instance to the "InvokeCardServices" function.  Part of our initialization routine looks like this:

BOOL InitApplication( HINSTANCE hInstance )

{

WNDCLASS       wndclass;

CSIBufPtr      CSIPtr;

HWND	         hwnd;

char szFormat[70];

ourInstance = hInstance;

// get a Card Services invocation pointer and an instance

hinstLibrary = LoadLibrary( "SSWINCS.DLL" );

if ( hinstLibrary < HINSTANCE_ERROR ) {

LoadString( hInstance, IDS_ERR_SSWINCS, szFormat, 70 );

wsprintf( output_buffer, szFormat, (UINT)hinstLibrary);

bad_error:

MessageBox(HWND_DESKTOP, output_buffer, szAppName,

MB_OK | MB_ICONINFORMATION);

return FALSE;

}

if ((InvocPtr = (PCMCIA_API) GetProcAddress(

hinstLibrary,"InvokeCardServices")) == 0L) {

LoadString( hInstance, IDS_ERR_CS_ENTRYPOINT,

output_buffer, OUTB_SIZE );

goto bad_error;

}

// Get Card Services information, including the number of sockets

if ( GetCardServicesInfo( (CSIBufPtr)ArgPtrbuf )) {

LoadString( hInstance, IDS_ERR_NO_CARDSOFT,

output_buffer, OUTB_SIZE );

goto bad_error;

}

CSIPtr = (CSIBufPtr) ArgPtrbuf;

numSockets = CSIPtr->Count;

// Other initialization code would follow.....

return TRUE;

} // InitApplication

In another module, we have the function that CardView uses whenever it needs to call Card Services:

DWORD InvocCardServices(BYTE Function, WORD Handle,

WORD far *Pointer,

WORD ArgLength, WORD far *ArgPointer)

{

return ((*InvocPtr)(Function, Handle, Pointer, ArgLength,

ArgPointer));

} // InvocCardServices

The double word returned by this function contains the return status in AX and the return handle in DX.

In the MainWindowProc that handles the Window messages, we register as a Card Services client during our handling of the WM_CREATE message.  CS_Callback is the name of the callback function provided for Card Services.

case WM_CREATE:

// Create any child windows and/or brushes you may need...

// Make an instance of our callback routine and

// register as CS client

CBPtr = (CallbackPtr) CS_Callback;

CBPtr = (CallbackPtr) MakeProcInstance((FARPROC) CBPtr,

ourInstance);

if ((status = RegisterAsClient( MEM_CLIENT, CBPtr )) != 0) {

CS_Error( "registering as a CS client", status );

return 0;

}

// Continue with any more WM_CREATE chores...

The RegisterAsClient() function fills an argument buffer and then makes a call to Card Services, using the InvocCardServices() function described above.

## Socket Services API Description

The Socket Services API that is contained in the file SSWINCS.DLL consists of one function with the following prototype:

WORD FAR InvokeSocketServices( union REGS far *RegPointer,

struct SREGS far *SRegPointer );

The following type definitions are contained in a header file:

typedef WORD (far *PCMCIA_SSAPI)(union REGS far *RegPointer, \

struct SREGS far *SRegPointer);

The pointer to the "InvocSocketServices" function is declared as a global:

PCMCIA_SSAPI InvocSSPtr;     // the Socket Services invocation call

In the initialization function, where the windows class is defined and the main window is created, we load the SSWINCS.DLL library and get the proc instance to the "InvokeSocketServices" function.

Part of our initialization routine looks like this:

BOOL InitApplication( HINSTANCE hInstance )

{

WNDCLASS       wndclass;

CSIBufPtr      CSIPtr;

HWND	         hwnd;

char szFormat[70];

ourInstance = hInstance;

// get a Socket Services invocation pointer and an instance

hinstLibrary = LoadLibrary( "SSWINCS.DLL" );

if ( hinstLibrary < HINSTANCE_ERROR ) {

LoadString( hInstance, IDS_ERR_SSWINCS, szFormat, 70 );

wsprintf( output_buffer, szFormat, (UINT)hinstLibrary);

bad_error:

MessageBox(HWND_DESKTOP, output_buffer, szAppName,

MB_OK | MB_ICONINFORMATION);

return FALSE;

}

if ((InvocSSPtr = (PCMCIA_SSAPI) GetProcAddress(

hinstLibrary,"InvokeSocketServices")) == 0L) {

LoadString( hInstance, IDS_ERR_SS_ENTRYPOINT,

output_buffer, OUTB_SIZE );

goto bad_error;

}

// Other initialization code would follow.....

return TRUE;

} // InitApplication

In another module, we have the function that CardView uses whenever it needs to call Socket Services:

WORD InvocSocketServices( union REGS far *RegPointer,

struct SREGS far *SRegPointer )

{

return ((*InvocSSPtr)(RegPointer, SRegPointer));

} // InvocSocketServices

Note	The following table lists the Socket Services for Windows functions that are supported by SystemSoft.  For more information on these functions, refer to Appendix B of the PCMCIA Socket Services Specification, Release 2.1.  For information on the semantics for these functions, refer to Chapter 5 of the PCMCIA Socket Services Specification, Release 2.1.

| Subfunction ID | Function |
| --- | --- |
| 83h | Get_SSInfo |
| 85h | Get_Adapter |
| 88h | Get_Window |
| 8Ah | Get_Page |
| 8Dh | Get_Socket |
| 8Fh | Get_Status |

Appendix E

# Extensions to the Card Services Specification Provided by CardID

The CardID driver provides extensions to the Card Services specification.  These extensions are accessed through the GetClientInfo subfunction (03h) by specifying an Info Subfunction in the range 80h-FFh.  The extensions are listed here by their Info Subfunction value.

## GetClientInfo Subfunctions

The following table lists the Info subfunctions for GetClientInfo.

| Info Subfunction | Function |
| --- | --- |
| 80h | Returns a bit-mapped field showing which GetClientInfo extensions are supported by the specified client. |
| 81h | Returns strings identifying which card resides in the specified logical slot. |
| 82h | Reserved by SystemSoft |
| 83h | Reserved by SystemSoft |
| 84h | Reserved by SystemSoft |
| 85h | Reserved by SystemSoft |
| 86h | Fills a buffer with values that describe the current CardID parameters. |
| 87h | Sets the current CardID parameters with values contained in a buffer. |
| 88h | Reserved |
| 89h | Reserved |
| 8Ah | Reserved |
| 8Bh | Reserved |
| 8Ch | Reserved |
| 8Dh | Reserved |
| 8Eh | Reserved |
| 8Fh | Returns error information for a specific slot. |

## GetGCIExtensionsInfo (Info Subfunction 80h)

This function returns a bit-mapped field showing which GetClientInfo extensions the specified client supports.

On Entry:

Attributes: 80XXh

Client Info Packet:

| Offset | Size | Contents |
| --- | --- | --- |
| 6 | 2 | Reset to zero |

On Exit:

Client Info Packet:

| Offset | Size | Contents |
| --- | --- | --- |
| 6 | 2 | Supported functions |

The Supported Functions field is bit-mapped and is defined as follows:

| Bit | Description if set |
| --- | --- |
| 15:6 | Reserved (reset to zero) |
| 5 | Has system diagram bitmap |
| 4 | Supports configured range for memory cards |
| 3 | Supports size for memory cards |
| 2 | Has Device Independent Bitmap |
| 1 | Supports Device Identification |
| 0 | Supports Card Identification |

## GetCardID (Info Subfunction 81h)

This function returns strings identifying which card resides in the specified logical socket.  Zero length strings are returned when the information is not available.

On Entry:

Attributes: 81XXh

Client Info Packet:

| Offset | Size | Contents |
| --- | --- | --- |
| 6 | 2 | Logical socket number |

On Exit:

Client Info Packet:

| Offset | Size | Description |
| --- | --- | --- |
| 6 | 2 | Logical socket number |
| 8 | 2 | Offset in packet to Card Type string buffer |
| 10 | 2 | Length of Card Type string |
| 12 | 2 | Offset in packet to Manufacturer string buffer |
| 14 | 2 | Length of Manufacturer string |
| 16 | 2 | Offset in packet Product string buffer |
| 18 | 2 | Length of Product string |
| N | N | Card Type ASCIIZ string |
| N | N | Manufacturer ASCIIZ string |
| N | N | Product ASCIIZ string |

Legal card type strings are defined as follows (case sensitive):

| Card Type String | Description |
| --- | --- |
| "Modem" | Modems, FAX/Modems |
| "Ethernet" | Ethernet Adapters |
| "ATA Disk" | ATA Disks |
| "Flash Card" | Flash Memory Cards |
| "SRAM Card" | SRAM Memory Cards |

## Reserved Functions

Subfunctions 82h, 84h, and 85h are reserved by SystemSoft.

## GetCardIDArgList (Subfunction 86h)

This function fills a buffer with values that describe the current CardID parameters.  These parameters match those that are contained in the CARDID.INI file.  The buffer filled has the following "C" structures:

#define MAX_COM_PORTS	   4

#define MAX_LAN_PORTS	   2

typedef struct {

WORD  addr;

BYTE  irq;

}  PORT_INFO;

typedef struct {

BYTE  flags;

PORT_INFO   port[MAX_COM_PORTS];

BYTE        com_order[MAX_COM_PORTS];

} COM_DEF;

// Flags for the COM_DEF.flags byte

#define SEL_SOCKET    0x01   // 0 = Insertion order, 1 = Socket number

#define BDA_ABSOLUTE  0x02   // BIOS data area address algorithm

// unused             0x04

#define AUDIO_ON      0x08   // 1 = Audio on in modem card

#define USE_RI        0x10   // 1 = Ring Indicate on in modem card

// The "mem_base" values in here are in 4K units.

typedef struct {

BYTE  flags;            // None currently defined

PORT_INFO  port[MAX_LAN_PORTS];

WORD  mem_base[MAX_LAN_PORTS][2];

} LAN_DEF;

typedef struct {

WORD  split_port_1;     // Currently not used

WORD  split_port_2;     // Currently not used

WORD  linear_port;      // Currently not used

} ATA_DEF;

/* Here is where we keep any other CardID arguments that have no

* home in the COM, LAN, or ATA definitions.

*/

typedef struct {

WORD  flags;            // Miscellaneous flags as described below

} MISC_DEF;

// Definitions for the MISC_DEF.flags field:

#define F_MISC_TRSPD      0x02    // Token ring speed: 0 = 4; 1 = 16

typedef struct {

COM_DEF  com_info;           // COM port information

LAN_DEF  lan_info;           // LAN information

ATA_DEF  ata_info;           // ATA drive information

MISC_DEF misc_info;          // Miscellaneous information

} COMMON_INFO;

typedef struct {

WORD  max_length;

WORD  info_length;

WORD  attr;                  // Subfunction number in high byte

char  cmd_path[80];          // Full file spec for CARDID.INI

COMMON_INFO  dos_info;       // Common info for the DOS world

COMMON_INFO  win_info;       // Common info for Windows world

} GETSETARGS;

On Entry:

Attributes: 86XXh

Client Info Packet: Empty buffer as described above.

On exit:

Client Info Packet: Filled with current CardID parameters.

## SetCardIDArgList (Subfunction 87h)

This function sets the current CardID parameters with values contained in a buffer.  These parameters match those that are contained in the CARDID.INI file.  The buffer is identical to that described in Subfunction 86h.

On Entry:

Attributes: 87XXh

Client Info Packet: Buffer filled with CardID parameters.

On exit:

Nothing.

## GetLastError (Subfunction 8Fh)

This function returns error information for a specific socket.  Specifically, it returns the function number of the last function to get a non-zero return code, and the error code that it received.

On Entry:

Attributes: 8FXXh

Client Info Packet:

| Offset | Size | Contents |
| --- | --- | --- |
| 6 | 2 | Logical socket number |

On exit:

| Offset | Size | Description |
| --- | --- | --- |
| 6 | 2 | Logical socket number |
| 8 | 2 | Last error code received |
| 10 | 2 | Function number of the call that returned the error |

Appendix F

# Card Services Vendor-Specific Callbacks

Currently, Card Services implements only one vendor-specific callback, Socket Configuration (83h).

## Socket Configuration (83h)

Callback (Socket Configuration, Socket, ConfigState, null, null, 0, ClientData)

The Socket Configuration event indicates that Card Insertion events for a PCMCIA slot have been completed, and notifies the client as to how the slot has been configured.

Cause	Card Insertion.

Pre-Client	Card Services issues a Card Insertion event to all clients that have used the RegisterClient function to indicate their interest in insertion events (see the description of Card Insertion in the PCMCIA Card Services Specification, Release 2.10).

Client	All clients that have used the RegisterSpecialClient vendor-specific function to indicate their interest in socket configuration events are notified by Card Services via the vendor-specific client callback entry.  If further information about the configuration is needed, the client should call the GetConfigurationInfo function if the slot is configured, and should call the GetWindowInfo vendor-specific function if the slot has memory windows.

Post_Client	Card Services does not perform any post-client processing.

The ConfigState is bit-mapped.  It identifies the socket configuration state:

| Bit | Function |
| --- | --- |
| 15:5 | Reserved |
| 4 | Memory Handle assigned to slot (via Open Memory) |
| 3 | MTD is registered on slot |
| 2 | Slot is in I/O mode |
| 1 | Slot is configured |
| 0 | Slot has memory window(s) |