README.DOC for Crystal Semiconductor Corp. 1.51 Windows NT Drivers

This Document contains the following:

1. Version Numbers

2. List of files

3. File Summary

4. Feature Summary

5. Installing Crystal Audio Drivers

6. Installing Compatible MPU-401 Driver

7. Installing Compatible Joystick Drivers

8. Un-installing Crystal Audio Drivers

1. Version Numbers

1.51	CS32BA11.SYS

1.51	CS32BA11.DLL

1.51	OEMSETUP.INF

1.51	CWAUDIO.BIN

79	CS4232	microcode

5     CS4232A	microcode

54	CS4236	microcode

1.51	CS.INI

1.51	MIDIMAP.CFG

2. List of Files:

10/30/96  03:15p               137,264 CS32BA11.SYS

10/30/96  03:15p               154,112 CS32BA11.DLL

10/30/96  03:15p                   312 OEMSETUP.INF

09/16/96  03:15p                 6,973 CWAUDIO.BIN

09/26/96  03:15p                 8,086 CS.INI

10/30/96  03:15p                40,962 MIDIMAP.CFG

3. File Summary:

CS32BA11.SYS   	kernel mode driver support for CS423x family

CWAUDIO.BIN  	Data file containing microcode for the CS4232, CS4232A, and CS4236.

CS.INI   	Default initialization parameters for REGINI.EXE.

It should be modified by OEM’s to meet their specific needs.

CS32BA11.DLL	User mode level driver support.

OEMSETUP.INF	Installation file.

MIDIMAP.CFG   	File utilized to allow the crystal midi support to be added to MIDI Mapper list.

4. Feature Summary:

1.	WSS Support for Microsoft Mixer Panels (NT 3.51/NT 4.0)

2.	Support for Microsoft Joystick driver (NT 4.0)

3.	Generic WINNT 4.0 PnP Support

4.	MIDI Internal FM Support (NT 3.51/NT 4.0)

5.	Support for Generic MPU-401 Microsoft driver (NT 4.0)

5. Installing Crystal Audio Drivers

1.	Turn PC off and insert audio card into an ISA slot unless audio

device is present in your motherboard.

2.	Turn PC on and start up WindowsNT.

3.	NT 3.51:

Double click the DRIVERS icon in the control panel.

NT 4.0:

Double click the Multimedia icon in the control panel and click on 			the Devices tab.

4.	Press the ADD button. Select Unlisted or Updated Drivers and press 	OK.

5. A dialog box will appear and request the path of the location of the

driver files. You may search this path by clicking the BROWSE 	button.

6.	Another dialog box will appear with the CrystalWare Audio Driver

label. Press OK to continue with the installation.

7.	If your system BIOS does not support Plug-and-Play, the Business

Audio Setup dialog box will appear and request you to select the

configuration resources.

8. WindowsNT may ask you to restart your system.  Even though it is not

necessary to do so, we recommend that you click on the Restart Now

button.

6. Installing Compatible MPU-401 Driver - WindowsNT 4.0 ONLY

NOTE: You will need your WindowsNT 4.0 installation disc(s).

1.	Double click the Multimedia icon in the control panel and click on

the Devices tab.

2.	Upon clicking the ADD button, the ADD dialog box will appear.

3.	Scroll down the list of drivers until you find the one labeled

MPU-401 Compatible Driver.  Selected and press OK.

4.	A dialog box will appear and request the path of the location of the 	drivers to be installed. Place your installation CD or Setup Disk #2

on the corresponding drive and press OK (the default path is the

i386 directory).

5.	The Generic MPU-401 Setup dialog box will pop up. To select the

default configuration resources, press OK.

6.	WindowsNT will ask you to restart your system. Press the Restart Now

button.

7. Installing Compatible Joystick Drivers - WindowsNT 4.0 ONLY

NOTE: You will need your WindowsNT 4.0 installation disc(s).

1.	Double click the Multimedia icon in the control panel and click on

2.	Upon clicking the ADD button, the ADD dialog box will appear.

3.	Scroll down the list of drivers until you find the one labeled

Microsoft Sidewinder 3D Pro Joystick. 	Selected and press OK.

4.	The Install Driver dialog box will appear and request the location of

the driver files. Place your installation CD or Setup Disk #2

on the corresponding drive. The default path to the Microsoft

Sidewinder Joystick drivers is the i386 directory.

5.	Once you have found the driver, press OK to complete the 	installation.

6.	WindowsNT will ask you to restart your system. Press the Restart Now

button.

8. Un-installing Crystal Audio Drivers

1.	NT 3.51:

Double click the DRIVERS icon in the control panel.  Once the

DRIVERS dialog box appears, scroll down the list of drivers and

select the entry labeled Audio for CrystalWare Audio Driver.

NT 4.0:

Double click the Multimedia icon in the control panel and click on 			the Devices tab.  Double click on the Audio Devices entry and then

select Audio for CrystalWare Audio Driver.

2.	Press the REMOVE button.  Verify your decision by pressing the YES

button of the REMOVE dialog box.

3.	The System Setting Change dialog box will appear and ask you to

restart WindowsNT.  Press the Restart Now button.