|~~\\\\\\\\\\\\\\\\\\\\\\\\

|___|//////////////////////

|      __  __ ___ _       |

|      |  /\ /\  / |      |

|      |_/__v__\/__|      |

|                         |

| MCI MIXER DOCUMENTATION |

|                         |

|_________________________|

|                         |

|  (c) 1991 Media Vision  |

|         v.001a          |

|_________________________|

INTRODUCTION

For some time we have been promising (threatening?) the development

of an mixer driver for MCI.  Accompanying this document is the

initial release of mcimixer.drv, the first MCI mixer driver for the

MPC level 1 platform.

With this MCI driver, you can begin adding mixer control

functionality to your applications without having to writing C code

that calls our MMMIXER.DLL.

If you are a presentation designer using a tool that allows you to

send MCI strings, you now have new volume control commands at your

disposal.

We encourage you to begin using this new capability right away.  If

you have any feature requests or questions on how to use

mcimixer.drv, don't hesitate to call Media Vision's Ken Nicholson at

(415) 770-8600.

----------------------------------------------------------------------

DISTRIBUTION

Media Vision is offering the virtual mixer interface to MPC

developers license free in the hopes of a standard mixer interface

being developed.  We encourage developers to ship the following files

with their products license free:

MCIMIXER.DRV

MMMIXER.DLL

MVMIXER.DRV

If you are shipping these files, please let us know.  We may wish to

assist you in promoting your product!

----------------------------------------------------------------------

INSTALLATION

The mcimixer.drv file should be in your \windows\system directory.

The driver entry in your system.ini file should be:

mixer=mvmixer.drv

----------------------------------------------------------------------

STATUS

All of the commands have been through initial testing and appear to

be robust.  However, some permutations of command options may not

work properly.  If you have any such difficulties, please contract

Ken Nicholson at the above number or our Technical Support number.

We will be adding commands to the MCI layer in the near future.

----------------------------------------------------------------------

reset      <device>

set        <device> <linetype> <linespec> <control> [channel] [to] <value>

get        <device> <linetype> <linespec> <control> [channel]

adjust     <device> <linetype> <linespec> <control> [channel] [to] <value>

connect    <device> <linetype> <linespec> [to] <linetype> <linespec>

disconnect <device> <linetype> <linespec> [from] <linetype> <linespec>

fade       <device> < [state <filename>] |

[<linetype> <linespec> <control> [channel]

[to] <value>]

> [delay] [duration]

KEYWORD DESCRIPTIONS:

=====================

the <device>  parameter must be either a valid mci device name or a

valid alias

the <linetype> parameter must be one of the following key words:

LINE_IN

LINE_OUT

DEVICE_IN

DEVICE_OUT

the <linespec> parameter is dependent upon the <linetype parameter>.

For "line_in" and "line_out", <linespec> must be a number

between 1 and the number of lines (input or ouput) supported

by the mixer.

For "device_in" and "device_out", <linespec> must be a one

of the standard 3-letter names for device types:

name:        Description:

--------------------------------------------------

N/C        No Connection

AAT        Analog Audio Tape

AMP        Amplifier

AUX        Auxillary

CDA        Compact Disk Player

DAT        Digital Audio Tape

EQU        Equalizer

LPA        Phonograph

MIC        Microphone

MIX        Mixer

MUS        Musical Instrument

RAD        Radio

SPK        PC speaker

TEL        Telephone

TVA        Television Audio

VTR        Video Tape Recorder

VDP        Video Disc Player

WAV        Digital Waveform Player

UNK        Unknown Device Type

the <control> parameter is an optional parameter with the default

being volume.  If used, this parameter must be one of

the following key words:

VOLUME          volume control

ALC             auto level control

BASS            Bass

MIDRANGE        Midrange

TREBLE          Treble

CROSSOVER       Stereo to left, right or exchange channels

LOUDNESS        Bass boost for low volumes

MUTE            disables audio, maintains current volume

REVERB          reverb control

STEREOENHANCE   increase stereo separation

CUSTOM1

CUSTOM2

CUSTOM3

the <channel> parameter is an optional parameter with the default

being "BOTH".  If used, this parameter must be one of the

following key words:

LEFT

RIGHT

BOTH

the <value> parameter must be an integer between 0 and 100

(inclusive).  This value represents the percentage

of the maximum allowable value.

the <delay> parameter must be a legal integer.  It represents the

number of tenths of seconds that are to elapse before

the specified fade operation is to begin.

the <duration> parameter must be a legal integer.  It represents the

time, in tenths of seconds, for the specified fade operation

to occur.

the <filename> parameter must be a fully qualified pathname to a

mixer state file (.MIX extension).  Promix.exe is used to

create these files.

!@!@!@!@!@!@!@!@!@!@!@!@!@!@!@!@!@!@!@!@!@!@!@!@!@!@!@!@!@!@!@!@!@!@!@

The following are some sample MCI commands that can be used to

control the mixer hardware.  The program MCITEST.EXE, which is

included as a sample application with the Microsoft Multimedia

Development Kit (MDK), provides an easy way to try out some of

these commands.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

You should precede each of the commands by the following line:

open mixer alias mix

When you are finished sending commands, send  this line:

close mix

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

To reset the mixer to the startup settings as specified in WIN.INI:

reset mix

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

To instantly change the volume of a specific mixer input line to 75%:

set mix line_in 3 control volume to 75

which is the same as:

set mix line_in 3 control volume 75

set mix line_in 3 to 75

set mix line_in 3 75

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

To instantly change the volume of a CD-ROM device on an unknown mixer input

line to 75%:

set mix device_in CDA control volume to 75

which is the same as:

set mix device_in CDA control volume 75

set mix device_in CDA to 75

set mix device_in CDA 75

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

To find out the current volume of a CD-ROM device on an unknown mixer input

line to 75%:

get mix device_in CDA control volume

which is the same as:

set mix device_in CDA

set mix device_in CDA 75

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

To find out the current bass EQ setting of the AMPLIFIER output line:

get mix device_out AMP control bass

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

To instantly reduce the setting of the AUX input volume to 10% less

than the current setting:

adjust mix device_in AUX control volume -10

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

To instantly connect both the microphone and the CD audio inputs to

the output which goes to the record circuitry:

connect mix device_in MIC to device_out WAV

connect mix device_in CDA to device_out WAV

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

To instantly disconnect the microphone input from the record circuitry:

disconnect mix device_in MIC from device_out WAV

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

To instantly change the state of the mixer to a state that was

previously saved to disk:

fade mix state c:\windows\saved.mix

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

To gradually fade out the volume of the Adlib-compatible FM

synthesizer input over 30 seconds:

fade mix device_in MUS to 0 duration 300

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

To wait 10 seconds then gradually fade the master volume to 50% over

the course of 5 seconds:

fade mix device_out AMP to 50 delay 100 duration 50

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Note: It is not necessary to fade a volume gradualy over .1 seconds

to avoid noise.  All mixer volume changes are implemented by

changing the volumes a step at a time to reach the desired

setting.