-----------------------------------------------------------------------

cFos Modem Command Set

-----------------------------------------------------------------------

?       Show a small help screen of AT Commands and important

S-registers.

A       Force Answer

Bn      Select protocol ONLY for outgoing calls

0       X75 without B3 protocoll, 64000bps      ("ISDNC")

1       V110 async, 38400bps, 8n1               ("ISDNB")

2       V110 async, 19200bps, 8n1               ("ISDNA")

3       128kbps Bundle Protokoll

4       X75, Framelen=256, Win.Size=7 (ELINK mode)

5       Datex-J (BTX)

D..     Dial

L       dial last number

S       dial stored number (hardcoded, 0=Zaphods BBS)

V       flag connection as SPV

0..9    digits

A..D#*  special "digits"

En      Command Echo

0       echo off

1       echo on

Hn      On/Off Hook control

0       on hook, terminate an active connection

1       off hook, reject an incoming call

In      Inquiry

0       product identification and CAPI version and

manufacturer

1       status line output

2       last call statistics

4       last 10 CAPI info dumps

O       Return On-line

Qn      Quiet mode, result codes display

0       display result codes

1       supress result code; be Quiet

Sr=n    Set register r to value n (registers are unsigned 16bit ints)

Sr.b=n  Set bit b to n

Sr,b=n  Set bit b to n

Sr?     Query register r

SrH=n   Set high byte of register r to value n

SrL=n   Set low byte of register r to value n

Vn      Verbal mode

0       numeric result codes

1       verbal result codes

Xn      Result code options

0       modem looks like a standard 9600 bps modem

1       enables CONNECT <rate>

2       enables RINGING

3       enables RING <incoming caller's id>

4       enables CONNECT[...]/<protocols..>

5       enables ?

6       enables /<cause> on ERROR, NO CARRIER, NO ANSWER

and NO DIAL TONE

Zn      Restore profile from number n (out of the file PROFILE).

This will further reregister the application at the CAPI.

If no number is given, 0 will be default.

Fn      \

Kn       |

Ln       |

Mn       |  ignored for compatibility

Nn       |

P        |

T       /

-----------------------------------------------------------------------

cFos extended Modem Command Set

-----------------------------------------------------------------------

{+}     &Bn     Number of Channels to use for 'cFos' Channel Bundling, for

next dial command

&Dn     Debug mode

0       disable status bar (default)

1       enable status bar (only if port is initialized)

2       enable status bar (only if carrier is high)

&En     EAZ: n = '0' ... '9': changes register 18. Default is

EAZ 0.

&F      Reset to Factory defaults and reset (like ATZ)

&Lnnn   Listen to what EAZ's; changes register 13; takes multiple

digits or '*' for all. Default is EAZ 1.

&Onnn   Outdial Number. This number will be used as prefix for

all phone numbers dialed with the ATD command. Default is

none.

&V      View profile

&Wn     Write profile to number n (into a file named PROFILE). If

no number is given, it defaults to 0.

-----------------------------------------------------------------------

cFos extended debug Modem Command Set

-----------------------------------------------------------------------

[ #C      Clear the alternate screen ]

#Rn     Reregister the application if all ports are idle; use

#R1 to force reregister (and lockout of all active

connections!)

#T      TRANX: set the clock to the actual time/date (gotten from

the last connection attempt).

Commands in brackets '[' and ']' are only available in debug

versions;  in the release version these commands are invalid.

-----------------------------------------------------------------------

cFos Modem Register Chart

-----------------------------------------------------------------------

Reg#      Description                                     Default

S0      Auto Answer on (>=1) / off (==0)                0

S1      Ring count                                      0

S2      Escape code                                     '+'

S3

S4

S5

S6

S7      Number of seconds modem waits for a carrier     60

S8

S9      Bit-mapped register:                            32737

.0      enable any further result code

.1      enable CONNECT <rate>, otherwise

CONNECT 9600 is always returned

.2      enable RINGING

.3      enable RING <number>

.4      enable /X75.. after CONNECT

.5      enable ERROR/<cause>

.6      enable NO CARRIER/<cause>

S10     Bit-mapped register:                            Default: 0

.0      enable CONNECT.../ID=<callerid>

.1      disable full status line, just display the callerid

on the left side

.2      display numerical values fuer service indicator /

add. si instead of names

.3      display the callerid, requested eaz, si/add si on

a RING like RING 1234/EAZ1/07/00, in spite of the

default method: RING<cr>ID=1234/EAZ1/07/00

S11     Status line line number (0 - screen length-1)   0

Note: only displayed if at&d1

S12

S13     Serviced EAZ Mask                               2

Every bit from bit0.. bit9 enables RING on

this EAZ ('0' - '9')

ex:     0000000010b     answer on '1'

0100000000b     answer on '8'

0001111111b     answer on '0' - '6'

1111111111b     answer all calls

This register is also modified by the AT&L... command.

S14     Serviced SI Mask                                0x80

S15     B-Channel Identification (ffs. 1.TR.6,          0x83

3.2.3.4.4.7)

S16     Source Service Indicator                        7

S17     Source Additional Service Indicator             0

S18     Source EAZ (character: '0'=48.. '9'=57)         '0'

Sent with connect_req message; information only

This register is also modified by an AT&En

command.

S19     Inactivity timer: modem disconnects if          0

no traffic in N minutes; 0==disabled

S20     B-Channel, level 2 protocol                     1

1       X.75 SLP Basis Operation Mode,

mit Implementation Rules nach T.90

2       Transparent-HDLC mit Bit-Stuffing,

Frame-Erkennung und CRC-Check

3       Bittransparent

4       SNA-SDLC

5       X.75 Btx

8       V110 mit transparentem B2 protokoll

9       V110 SNA-SDLC

10      V110 X75 SLP nach T.90

S21     B-Channel, level 3 protocol                     4

1       T70 NL fr Leitungsvermittlung (CSPDN)

2       ISO 8208 (DTE/DTE)

3       Level 3 according to T.90, Appendix II

4       transparent

S22     SELECT_B2_PROTOCOL_REQ: Data-length             2048

S23     SELECT_B2_PROTOCOL_REQ: Link-Address-A          3

S24     SELECT_B2_PROTOCOL_REQ: Link-Address-B          1

S25     SELECT_B2_PROTOCOL_REQ: Modulo-Mode             8

S26     SELECT_B2_PROTOCOL_REQ: Window-Size             1

S27     SELECT_B2_PROTOCOL_REQ: V110 user rate          64

S28     Maximum SEND B2-Framesize                       2048

S29     B3-Protocol-Overhead in Bytes                   0

b3 == 1 (t70nl): 2

b3 == 2 (iso8208): 3

b3 == 4 (transparent): 0

S30     SELECT_B3_PROTOCOL_REQ: LIC                     0

S31     SELECT_B3_PROTOCOL_REQ: HIC                     0

S32     SELECT_B3_PROTOCOL_REQ: LTC                     0

S33     SELECT_B3_PROTOCOL_REQ: HTC                     0

S34     SELECT_B3_PROTOCOL_REQ: LOC                     0

S35     SELECT_B3_PROTOCOL_REQ: HOC                     0

S36     SELECT_B3_PROTOCOL_REQ: Modulo-Mode             8

S37

S38

S39     Numbering Type/Plan                             0x81

S40     Controller                                      0

S41     Info-Mask-low                                   63

S42     Info-Mask-high                                  0

S43     Default V110 user rate & Protocol               64

S44

S45

S46

S47

S48

{+}     S49   number of digits of caller id to ignore           0

(from right to left) when called with CCB

S50    \

S51     |

S52     |   Default Additional Service Indicator and B2-Protocol

S53     |   for every EAZ:

S54     |   If the incoming AddSI is 0 then we get the AddSI

S55     |   from the low byte of register 50+EAZ; the B2-Protocol

S56     |   is got from the high byte.

S57     |

S58     |

S59    /

-----------------------------------------------------------------------