STPCB Version 3.05B, 7/10/95

This document is a Beta draft!

---[ Introduction ]--------------------------------------------------

STPCB is a PCBoard PPE program which acts as a "front-end" or "shell"

for USRSTAT2.EXE.   This allows your callers to use USRSTATS and view

the BBS modem's diagnostic reports while they are online, no matter

what type of modem *they* are using.  It allows capturing the reports

into the message base, and allows the caller to initiate a modem

retrain from the BBS end of the connection.

* Again, the _caller_ can be using ANY brand or model of modem. *

That is one of the most important aspects of STPCB.   Many modems have

little or no online diagnostics, leaving the owner completely in the

dark when trying to diagnose problems or test performance.   STPCB

provides a way for _callers_ to take advantage of the tremendous

diagnostic abilities of _your_ USR modem.

To use this PPE program, you must be running PCBoard version 15.1 or

later, and USRobotics Courier or Sportster modems.   STPCB provides the

most detailed information when used with V.34 and VFC models, but it

will also work with many of the earlier 14400, 16800, and 21600 models.

If your BBS uses a mixture of USR and other brand modems, STPCB can be

configured to run only on the USR-equipped nodes.

If for some reason you cannot use STPCB on your system, there is an

alternative called STGEN.  (ST Generic.)   This is a more traditional

"Door" type of program which works with many different BBS packages.

it is similar to STPCB in operation, but does not have the

message-capture ability built into it, and it is installed through the

old "shell to a batch file" system interface.

STPCB is compatible with single and multi-node PCBoard setups.  It has

been extensively tested on a single-node system running under DesqView

and Lantastic, and on a large multi-node system running Netware.

STPCB is a featured attraction on the USRobotics Customer Support BBS,

and has been running successfully there for over a year without a

single trouble report.   STPCB is currently in use on hundreds of

PCBoard BBS systems world-wide.

---[ Important Information ]------------------------------------------

Before installing STPCB, make sure you understand what "Online Command

Mode" is and how it works.   (See Appendix A for detailed info.) STPCB

can be configured to use either Escape Character or DTR switching.

You must select the method that best suits your system and your

preferences.   If you are in doubt, use Escape Character switching.

---[ STPCB File Locations ]------------------------------------------

Create a directory or subdirectory to hold STPCB and its related files.

You may select any name or location for this directory, provided that

STPCB will have read and write access there.   STPCB creates temporary

files during execution.

Example: If you have PCBoard installed to C:\PCB, you

might want to use a subdirectory called STPCB, like this:

MKDIR C:\PCB\STPCB

Copy all of the STPCB files into the new directory.

---[ Modifying CMD.LST ]----------------------------------------------

Using PCBSETUP, modify the CMD.LST files for the Main Board and/or any

conferences that you wish to be able to call the program from.  Edit

the CMD.LST file, entering the command name you wish to use to call the

program, and full path and filename of USRSTATS.PPE.

Example:

���������������������������������������������������������������������ͻ

�                      CMD.LST Editor                                 �

�                                                                     �

�                  Charges Per     PPE/MNU File Specification -or-    �

�   Command  Sec  Minute    Use    Keystroke Substitution             �

� ���������� ��� ����������������� ���������������������������������  �

� 1) USRSTATS    0      0        0 C:\PCB\STPCB\STPCB.PPE             �

�                                                                     �

�                                                                     �

�                                                                     �

�   ESC=Exit AltI=Insert AltD=Delete AltR=Repeat Alt5=25/50 F2=Edit   �

�� 20:31:55 � 07-02-95 ��� F1 � help �� caps: OFF  num: ON  ins: OFF ͼ

---[ Modify modem init string ]---------------------------------------

In most cases, installation of STPCB does not require re-programming

your modem's stored settings.   The required changes in modem setup can

be done entirely in the modem init strings found in PCBSETUP.

BE SURE YOU UNDERSTAND THE ADVANTAGES AND DISADVANTAGES OF EACH COMMAND

MODE SWITCHING METHOD BEFORE YOU GO ANY FARTHER.  THIS INFORMATION CAN

BE FOUND IN APPENDIX A.

IT IS YOUR RESPONSIBILITY TO VERIFY THAT THE SELECTED METHOD WILL NOT

HAVE ANY ADVERSE REACTION WITH OTHER SOFTWARE OR HARDWARE IN YOUR

SYSTEM.

The normal init string for PCBoard is "ATZ" which simply resets the

modem.  The standard modem setting of &D2 allows PCBoard to hang up by

simply dropping the DTR lead on the serial port.  The standard modem

setting of S2=255 disables the Escape Sequence detection.

STPCB requires that these settings be modified to match the Online

Command Mode switching method defined in STPCB.CFG.  For DTR switching,

the modem must be running with the &D1 setting.  For Escape Character

switching, the S2 register must be set to the ASCII value of the

desired Escape Character.

---[ Modem init string for Escape Character switching ]---------------

Using PCBSETUP, modify the modem initialization string as shown below.

Modem initialization string: ATHZ^M~~~~ATS2=1

The S2 value of 1 is only an example.  This number must match the

number defined in the STPCB.CFG file.  The factory default S2=43 can be

used, but really isn't a good choice.

PCboard v15.2 has two entries for the modem init string, so the

commands may be split up, like this:

Modem initialization string 1: ATHZ

Modem initialization string 2: ATS2=1

Again, S2=1 is only an example.  Make sure this number matches the one

defined in STPCB.CFG.

Helpful Hint:  When using Escape Character switching, Courier modems

must have DIP switch #9 in the ???  or DOWN position.   Sportster

modems do not have an equivalent DIP switch, they require S14=1 to

imitate this function.  If you forget to do this, the modem will drop

the connection every time you run STPCB.

---[ Modem init string for DTR switching ]----------------------------

Using PCBSETUP, modify the modem initialization string as shown below.

Modem initialization string: ATHZ^M~~~~AT&D1

Be sure that STPCB.CFG is correctly set for DTR switching mode.

PCboard v15.2 has two entries for the modem init string, so the

commands may be split up, like this:

Modem initialization string 1: ATHZ

Modem initialization string 2: AT&D1

Be sure that STPCB.CFG is correctly set for DTR switching mode.

Because PCBoard expects to be able to drop a connection by toggling

DTR, there are several operational problems that can result from using

DTR switching.  When the modem is running with &D1, it can't be forced

to drop the connection simply by dropping DTR, it must also be sent an

ATH or ATZ command.

We can work around the minor issues involved with PCBoard, however,

you may have a front-end mailer or a door installed that also expects

to be able to drop connections in a manner that conflicts with the

STPCB modem setup.  THOROUGHLY TEST YOUR SYSTEM FOR ANY PROBLEMS THAT

MIGHT RESULT FROM USING DTR SWITCHING.

---[ Optional add-on for DTR switching ]------------------------------

THE FOLLOWING SECTION IS AN OPTIONAL PART OF THE DTR SWITCHING METHOD.

To improve the way PCBoard hangs up when using DTR switching, we need

to replace 3 entries in the PCBTEXT file so that instead of just

displaying the "goodbye" message to the caller, it also hangs the modem

up properly.  This is done with a tiny PPE program to replace the

default PCBoard text strings.

Use the MKPCBTXT utility to modify the following records in PCBTEXT.

���������������������������������������������������������������͸

� Record No.:   55   Record Length:  56   Justification:  Left  �

���������������������������������������������������������������Ĵ

�Keyboard Time Expired!                                         �

�!c:\pcb\stpcb\rec55.ppe                                        �

���������������������������������������������������������������;

���������������������������������������������������������������͸

� Record No.:  166   Record Length:  75   Justification:  Left  �

���������������������������������������������������������������Ĵ

�Thanks for calling, @FIRST@!                                   �

�!c:\pcb\stpcb\rec166.ppe                                       �

���������������������������������������������������������������;

���������������������������������������������������������������͸

� Record No.:  435   Record Length:  70   Justification:  Left  �

���������������������������������������������������������������Ĵ

�Hanging up now.  Thank you for the upload(s).                  �

�!c:\pcb\stpcb\rec435.ppe                                       �

���������������������������������������������������������������;

The REC55.PPE, REC166.PPE, and REC435.PPE files are included in the

STPCB package.  You may modify them as you wish, and of course the file

and path names in the example above should reflect whatever your system

requirements are.

There may be other entries in PCBTEXT that would benefit from this

sort of modification.  PPL source code is included to allow patching of

any additional fields that you need.

---[ STPCB configuration file ]-------------------------------------

STPCB uses a configuration file to control its optional features.

This file MUST be located in the same directory as the PPE file, and it

MUST be named STPCB.CFG.  A *sample* file named STPCB.DEF is supplied

in the distribution archive, or the program will create another one for

you if no config file is found.

*********************************************************************

* All documentation for the fields in the config file, and the      *

* STPCB options, are contained as comments within the sample config *

* file itself.                                                      *

*********************************************************************

** Copy the STPCB.DEF file to STPCB.CFG. **

Then edit STPCB.CFG, setting the options as desired for your system.

Be sure to read all the comments in the file, they form a large portion

of the STPCB documentation!  Save the STPCB.DEF file for future

reference.

---[ User interface ]-----------------------------------------------

The last installation step is to edit your system's menu files so

that your users will be able to find the new feature.

You might also consider installing REPORT.DOC as a bulletin for users

to read or download.   When callers have questions about the data in

the report, you can refer them to this bulletin for the technical info.

That's it, installation is complete.   Now, when your callers enter the

new command you have defined, the BBS will chug and puff for a few

seconds and then display a nicely formatted screen of modem status

info.

Please note that it doesn't work in local mode, there has to be a real

live modem connection.

---[ STPCB/USRSTATS Support ]-----------------------------------------

The home of STPCB and USRSTATS is:

The Majestik Moose BBS,  708-843-2871

Only calls at 9600 BPS and faster are accepted.  MNP/V42 is required.

The latest version of this program is always available as a free

download in the Main Board area.  First time callers can get immediate

free access to these files by bypassing the registration prompt.

You will be allowed 30 minutes per call and complete access to the

"Free Downloads" area of the BBS.

If you need help with STPCB or USRSTATS, you may leave a message for

the SYSOP.  I'll help you if I can, but I make no promises.  After all,

the software is free.  Do NOT call and ask questions that are already

answered in the documentation.  You will receive only a very brief

answer under those circumstances.

STPCB and USRSTATS are available through ftp.usr.com on the Internet.

Use anonymous login, and your Internet ID as the password.

STPCB and USRSTATS are available (and running) on the USRobotics

Customer Support BBS at 708-982-5092.

At the present time, there is no registration required for this

software.  If you really want to pay me a few bucks for the many hours

of effort that went into this, I'll be glad to take it from you.  Call

The Majestik Moose and leave a (C)omment.

---[ Disclaimer ]----------------------------------------------------

Use this software at your own risk.  It works fine for me.  All product

and feature names are trademarks of their respective owners.  This

software is NOT a product of USRobotics, Inc.

Joseph C. Frankiewicz

The Majestik Moose BBS

708-843-2871

---[ Appendix A - Modem Escape Sequences ]---------------------------

An "Escape Sequence" is a mechanism or method for forcing the modem

into "Command Mode" while it is already connected and on-line.

"Command Mode" is normally associated with the OFF-LINE condition,

where the modem will accept and act upon "AT" commands entered from the

terminal.  However, it is also possible to put the modem into this mode

DURING a call, WITHOUT disconnecting the call first.  This ability is

required for making use of certain diagnostic commands which only

return valid data when used in this on-line command mode.

USRobotics modems offer 2 methods of getting into on-line command mode.

Each has its own advantages and disadvantages, depending on the point

of view.

The first method is the &D1 setting.   When the modem is set to &D1, a

momentary "toggling" of the DTR signal (pin 20), from ON to OFF and

back ON again, will suspend the data connection and place the modem

into command mode.  USRSTATS documents refer to this as "DTR

switching".

The second method is the infamous "Escape Character with Guard Time."

A designated ASCII character is defined as the "Escape Character", and

when the modem receives three of these characters in a row, with a

defined period of inactivity both before and after, it suspends the

data connection and places the modem in command mode.  The character

used for the escape is defined in the S2 register, and the default

value is 43, the "+" character.  (This is where the often-seen "+++"

comes from.)  The inactivity or "Guard" time is defined in the S12

register, with a default of 1 second.  USRSTATS documents refer to this

as "Escape Character switching".

When using DTR switching, the remote user does not see the "+++"

characters on the screen when the BBS modem is placed into command

mode.  This provides a professional seamless interface.  Also, the BBS

modem is not susceptible to entering command mode if the _caller_ is

also using Escape Character switching with an identical Escape

Character definition.

The disadvantage of DTR switching is that software cannot drop

connections by simply toggling the DTR pin, it must _also_ send an ATH

or ATZ command.  It is possible to work around this in PCBoard, however

you may be using other software in your system that cannot be altered

this way.

Using Escape Character switching, your software retains the ability to

drop connections by toggling the DTR pin, as normal.  The disadvantages

are that the remote user will see the "+++" sequence on screen when

STPCB runs, and there is a possibility that the remote user may ALSO

be trying to use Escape Character switching with the same Escape

Character that the BBS is using.   This results in the BBS modem going

into command mode when the intended target was the remote modem.  Since

the BBS modem can no longer pass any data to the user, and the BBS

software is unaware of what happened, the user has no alternative but

to drop the connection and call back.  For this reason, avoid using the

standard S2 register value of 43.  (The "+" character.)   A value of 1

(S2=1) usually works well.

[End of File]