FIX16550 Device Driver v01.06.01  25 FEB 1994 (94056) 21:46:58

Copyright (c) 1993-94 Professional Software Engineering, Ltd.

All rights reserved.

This is the documentation file for the FIX16550 program.

This program has a pretty narrow function -- to fix problems with AMI BIOSes

and 16550 ("hi-speed/buffered") UARTS.  As an added benefit, the program also

happens to permit (usually much older) computers which only recognize COM1: and

COM2: to automatically recognize COM3: and COM4: if they are present.

If you can power-off your computer and power it back on and all of your serial

ports get recognized, but they don't if you simply reboot the computer using

the keyboard, then this program is probably what you're looking for.

While this document makes a number of references to the AMI BIOS, this program

is not dependant IN ANY WAY on the presence of an AMI BIOS -- there may very

well be other BIOSes out there that exibit this or a similar problem.  When

using this program to extend older BIOSes to COM3: and COM4: capability, the

BIOSes will certainly vary in manufacturer.

Operation is simple:  Simply add the line:

DEVICE=C:\PATH\FIX16550.SYS [8514] [NOBEEP] [BRIEF]

(where "C:\PATH\" is replaced by where you really put the file.) into your

CONFIG.SYS file.  Near the top is best (before any other programs initialize,

so they too will know how many ports your machine has, if they care).

Normally, there is no need for any parameters at all.  The exceptions being:

*	You have an 8514/A video controller (see v01.06.00 release notes

below), in which case you'd specify "8514" on the commandline.

*	You don't like the warning tones the program generates to inform you

that it is doing it's job (when it detects a port the BIOS didn't, or

detects a conflict with COM4: and the 8514/A), in which case you would

want to add "NOBEEP" to disable the beep tones from being emitted when

these error/warning conditions occur.

*	You don't really want to see all the port information each time your

computer boots, in which case you would specify "BRIEF" to disable all

the port status messages from being displayed, keeping your boot screen

a bit less cluttered.  Since BRIEF also inhibits status messages

indicating automatic port additions and removal (such as a COM4: with

the 8514/A), it also happens to inhibit the beep tones which are part

of the messages being displayed under those circumstances.

Commandline options are not case-sensitive, but also DO NOT REQUIRE dashes or

slashes (in other words, don't use slashes or dashes in front of the arguments).

Since this driver is an initialization-only driver (it does it's job and quits,

much like a word processor would -- it doesn't remain in memory to do

anything), there is no purpose to loading it high (although the program will

operate fine if loaded high to run), and it will not affect memory use in a

computer.  If you loaded it at the top of your CONFIG.SYS, loading high

wouldn't be an option anyway -- your upper memory driver (QEMM, EMM386,

NetRoom, 386MAX, etc.) wouldn't be loaded yet.

When the program is executed, it displays a version/copyright banner, and

(unless BRIEF is specified on the commandline), four lines in the format:

COM[n]:  BIOS [Y/N]  SCAN:  [type of UART found / conflict message]

Where 'n' is the port number (1-4), "BIOS Y" indicates that the BIOS table

showed the presence of this port, and "BIOS N" indicates that the BIOS table

did not have the port listed.  The text following "SCAN:" indicates the type of

chip found, or lack thereof.  When BIOS shows 'N', and SCAN shows a port was

found (or when BIOS shows 'Y', and SCAN reports an 8514/A conflict) is when

FIX16550 has done something for you.

*-------------*

I don't know how widespread the AMI BIOS problem with 16550 FIFOs is, but it

appears to exist in the (as of this original writing in late 1993) "current"

version of the AMI BIOS dated DEC 1991.  The problem manifests itself when you

have a high-speed UART installed (something which is advised if you use a high

speed modem -- most internal high speed modems are equipped with a 16550

equivalent), and use a program which enables the FIFO (the special feature of

the 16550 UART chip), but which either does not disable it on exit, or is

rebooted while running (which is a valid thing to do, say if the computer is

running a BBS, and a user shells to DOS and drops carrier).  When the AMI BIOS

goes looking for serial ports on startup, it fails to see the port (because it

doesn't appear normal, since the FIFO is enabled), and the computer boots up

without that port in the list of available ports.  Then other software cannot

"see" the port as being present, and won't work (such as attempting to run your

comm program or the BBS after the reboot).  A hard reboot (power cycle, or that

red button on the front of the case) will reset the UART at the hardware level,

and thus the AMI BIOS will recognize it.  This program simply looks for the

hardware in a better way than the AMI BIOS does, and adds the ports

automatically for you.

I've heard that some people have thought that this problem was something wrong

with their system board (since it doesn't show up if you hard reboot the

machine).  Take another look -- you may have an AMI BIOS.

Don't get me wrong -- I like AMI BIOSes very much (I could be using a different

one if I wanted).  They have a lot of great features, and aside from this one

problem, appear quite bug free (unlike the old DTK/Erso).

As for systems that don't recognize COM3: and COM4:, they are fairly

widespread, and include PC, 286, and 386 machines, though I don't know about

486 boxes (most companies should have it straight by now).  Name brand

manufacturers known to have this problem, at least in some models, include

Compaq and AST.

After a lengthly discussion with a user, it was determined that QMODEM (at

least certain versions of it, and I suspect other comm programs as well) may

require a base port address in order to operate (i.e. "COM4" might be a

configuration option, but you need to tell it that "COM4" is 0x02E8 elsewhere

in the configuration).  Telix is similar (v3.10 - v3.22 I know), but has all

the base addresses pre-initialized, although it allows you to change them.

For reference, the standard assignments of ports and their base addresses and

IRQs are:

COM1:	03F8	IRQ4

COM2:	02F8	IRQ3

COM3:	03E8	IRQ4

COM4:	02E8	IRQ3

This program will not enable support for a serial port that a specific software

program cannot otherwise recognize (some games, for instance allow modem or

direct serial connection, but usually only through COM1: or COM2:).  Microsoft

Windows, for instance (v3.0-v3.11, though it might be fixed in v4.0) will only

recognize a Microsoft mouse on COM1: or COM2:, even though you can use the DOS

Microsoft mouse driver on COM1-COM4 (other mouse drivers may differ).

I originally wrote this utility strictly to deal with my experiences with the

AMI BIOS problem.  When I called AMI, they told me I needed to get in touch

with the system board manufacturer -- which in all likelyhood is some offshore

company, and they wouldn't even give me a company name!  The AMI BIOS is

mentioned in the Microsoft Windows Resource Kit as having serial problems --

and my BIOS is for a 486DX2 machine -- not exactly old technology.

*-------------*

v01.00.00

v01.01.00

v01.02.00 unreleased versions, used internally before finding that AMI had no

interest in being given code to correct a problem in their BIOSes, since they

wouldn't admit they even had a problem.

v01.03.00 incorporates a detection for the 8514/A video adapter, which

conflicts with the use of a COM4: port.  With all the port addresses they use

for the 8514/A, you'd think they could have left 2E8h alone!  First public

release.

v01.04.00

v01.04.01 debugging versions, not released.

v01.05.00 incorporates a fix for an odd bug which caused some ports not to be

properly added back to the port table (although they were detected and reset).

8514/A support has been disabled until better documentation is available.

v01.06.00 adds a commandline switch (which I'd like to have avoided the need

for, but still I cannot locate good docs on the 8514/A) for dealing with 8514/A

based video systems.  The switch is simply "8514" (no slash, dash, or anything,

example:  "DEVICE=FIX16550.SYS 8514").  This will flag the program to not

initialize the COM4: device (which would usually cause an 8514/A system to

blank it's screen), and to REMOVE any occurrence of COM4: (02E8h) from the BIOS

data table if it is found, and alert you in the process.  If you have a COM4:

and specify this switch, FIX16550 will beep at you each time it is run (you

have a hardware conflict that should be resolved).  A specific example of a

board with 8514/A capability is the ATI Ultra (a VLB Windows accelerator using

ATIs MACH-32 chipset).  The ATI Ultra does not support having the 8514/A mode

disabled -- which means (as is stated in it's documentation), you cannot have a

COM4: in a machine with an ATI Ultra.  I had one of these, and discarded it in

favour of the Diamond Viper VLB (no 8514/A crud, MUCH faster, and less buggy

Windows drivers).  If that sounds like a plug for the Viper, it is.

Also with this release, the official formatting of the distribution archive

name changes to something a bit more version-identifiable:

FX550106 ( F(i)X (16)550 (v)1(.)06 )

Not released.

v01.06.01 added two other options: NOBEEP and BRIEF, described above in the

usage section.  Added comments to the documentation about some comm programs

requiring additional configuration to see a port properly.

*-------------*

The archive this program was distributed in was originally a PKZIP v2.x

authenticated archive with only 4 files in it: this document, the program, and

two types of BBS description files (FILE_ID.DIZ and DESC.SDI).  Nothing else

should have been in the archive, and at completion of extraction of the

archive, you should have seen the following message, with an "-AV" after each

and every file in the archive:

Authentic files Verified!   # DQP160

Professional Software Engineering

If not, then you did not recieve an original of the archive as produced by the

author.

This program is ShareWare.  A fee is requested for it's use.  If you use it

(and the only reason you should be using it is if it addresses a problem you do

have), a registration of US$10.00 PER COPY USED is required.  The grief and

annoyance this program can save you over time will be well worth the small

registration, especially if you are a BBS operator.  Please make the cheque or

money order (drafted ib a U.S. bank) payable to: "Sean B. Straw", and send it

to the address below.

You will recieve notifications of major updates to the program for a period of

one year from the date of registration (although you will not get a disk each

time), and you are entitled to use any subsequent versions of FIX16550 without

re-registering.

Comments, suggestions, and corrections are invited.

About the author:

=================

Professional Software Engineering is a developer of system enhancement

utilities and a variety of anti-virus tools.  We also provide custom systems

programming services in assembler and C, and perform password recovery for

files in PKZIP files which have been encrypted.

Professional Software Engineering

Post Box 2395

San Rafael, CA  94912-2395

(415) 459-7401

If calling, and you get an answering device, please be sure to start your

message with your complete phone number (including area code), and the name and

version of the program you are calling about ("Your program" is not descriptive

enough).  Then describe the problem you are having, and end the message with

your phone number repeated a second time, just for good measure -- if I can't

make out your number, I cannot very well get back to you to help you with your

problem.