L A P T A L K

A

Tiny

Telecommunications

Program

for

Laptop Computers

Release: 1.1

Revised: 09-Jan-95

Copyright 1991-1995 Dave Dunfield

All rights reserved.

LAPTALK Users Guide                                              Page: 1

1. INTRODUCTION

I first conceived of LAPTALK when I did a great deal of  traveling

with a small 8088 dual-floppy (no hard disk) laptop computer. Being a

shareware author, I try to log into local BBS's wherever I travel, to

upload the latest versions of my software.

Some of my archives are quite large, requiring up to 1/2  hour  to

transfer (at 2400 bps). Using the floppies  was  unacceptable,  since

the drives just sat  there  starting  and  stopping....  The  obvious

solution  was  to  define  a  RAMdisk,  allowing  smooth,  quiet  and

non-destructive operation.

Then I ran into a small problem.... The  telecomm  packages  which

contained the features I used were all very large, typically  200-300

Kbytes in size. This not only used up a good portion of the 640K  ram

on my laptop (thereby reducing the size of my RAMdisk), but  it  took

lot of space on the 720K floppy drive,  and  took  FOREVER  to  load.

There just had to be a better solution...

I decided to write my own very small "no frills" terminal program,

which would still have the features I needed, which were:

1) Reasonable subset of ANSI (VT100) terminal emulation

2) Built in capture and upload of ASCII files.

3) Ability to link to external transfer protocols

4) Powerful "command" language to automate uploads etc.

5) A simple "host" capability for remote access.

The results  of  my  effort  was  LAPTALK,  a  ".COM"  file  which

addresses the above requirements, and weighs in at just over 20K.

The complete source code to LAPTALK is distributed with my MICRO-C

compiler as an example program. See the enclosed CATALOG file.

LAPTALK Users Guide                                              Page: 2

2. USING LAPTALK

When you first run LAPTALK, it  will  attempt  to  open  the  file

LAPTALK.CFG in your current directory. If found,  it  will  read  its

configuration from it.  If  the  configuration  file  is  not  found,

LAPTALK will display a warning message, and assume various defaults.

Once started, LAPTALK will present you with a  terminal  emulation

screen. The top 24 lines are the actual data  window,  and  the  25th

line is a status line. You may now type on the keyboard to send  data

to the serial port, and see any incoming data in the terminal window.

2.1 Command Line Options

The format of the LAPTALK command line is:

LAPTALK [-i c=file] [script command]

The '-i' switch tells  LAPTALK  to  NOT  enter  an  interactive

session. This is most useful when used with a command line  script

file, so that control will  return  to  DOS  when  the  script  is

finished.

The 'c=<file>' parameter allows  you  to  specify  a  different

configuration file which is to be  used  instead  of  the  default

"LAPTALK.CFG".

If [script command] is given, LAPTALK will  execute  the  named

script and pass  it  any  additional  arguments  BEFORE  the  main

interactive mode is entered.

2.2 LAPTALK Functions

To access LAPTALK's functions, press CTRL-HOME or CTRL-END. You

will be presented with this menu:

+--------------+

|Function menu |

|Download file |

|Upload file   |

|Kill capture  |

|Perform script|

|Hangup modem  |

|Clear screen  |

|Configuration |

|Shell to DOS  |

|Exit to DOS   |

+--------------+

You may activate a function by moving  the  highlighted  cursor

bar to it (with the UP and DOWN arrow keys) and pressing ENTER. To

quickly move the cursor bar to a particular entry, type the  first

letter of the entry.

LAPTALK Users Guide                                              Page: 3

2.3 Function menu

This function presents you with a list of "functions" that have

been configured. Each function  represents  a  "script"  (command)

file, complete with arguments, that will run when that function is

selected.

This is a convienent way to set up automatic dialing lists etc.

2.4 Download file

This function will prompt with a menu of  available  protocols,

and then for the name of  the  file  to  be  transferred.  If  you

specify a filename which contains wildcard character ('*' or '?'),

LAPTALK will show you a list of matching files, and allow  you  to

pick one.

Once the filename is selected, LAPTALK will invoke the selected

protocol, and attempt to download the file from the remote  system

to your PC.

2.5 Upload file

This function is similar to "Download file",  except  that  the

file is transferred from your PC to the remote system.

2.6 Kill capture

This function  is  used  following  a  download  using  "ASCII"

protocol, to stop LAPTALK from capturing the terminal  session  to

the file. When an ASCII download is in effect, you  will  see  the

string "cap" in the status line  at  the  bottom  of  the  screen.

Selecting this function will stop the  data  capture,  and  remove

that indication from the status line.

2.7 Perform script

This function prompts for  a  script  filename  and  arguments.

Unless you specify otherwise, the file is assumed to be located in

the LAPTALK  home  directory,  and  it  is  assumed  to  have  the

extension ".SCR". LAPTALK will process  the  file  as  a  "script"

program.

This method of running a script program  is  useful  for  those

files which you do not use  frequently  enough  to  have  in  your

"function  menu".  It  is  also  useful  in  that  you  may  enter

parameters to the program when you specify its name.

If the filename you supply contains wildcard character ('*'  or

'?'), LAPTALK will show you a list of the matching  script  files,

and allow you to select one.

LAPTALK Users Guide                                              Page: 4

2.8 Hangup modem

This command drops the DTR and RTS signals to the modem  for  a

brief period of time, causing most modems to go offline  and  hang

up any calls which are in  progress.  See  you  modem  manual  for

information on how to set up your modem to do this.

2.9 Clear screen

This functions clears the TTY screen.

2.10 Configuration

This function invokes the LAPTALK configuration menu,  allowing

you to alter the various parameters  controlling  all  aspects  of

LAPTALK's operation.

See the "configuration" section for more details.

2.11 Shell to DOS

This function invokes the DOS command processor (as defined  in

the environment variable COMSPEC),  to  allow  you  to  enter  and

execute DOS commands. LAPTALK  remains  active.  Use  the  command

"EXIT" to return to LAPTALK.

2.12 Exit to DOS

This function terminates LAPTALK, returning to the DOS  command

processor.

LAPTALK Users Guide                                              Page: 5

3. CONFIGURATION

When you select 'Configuration' from LAPTALK's main function menu,

you will be presented with this sub-menu:

+---------------------+

|General switches     |

|General parameters   |

|Serial port settings |

|Transfer protocols   |

|Function menu scripts|

|Video attributes     |

|Load configuration   |

|Save configuration   |

+---------------------+

3.1 General switches

This configuration sub-menu presents you with  a  selection  of

ON/OFF switches, which control the  way  LAPTALK  handles  certain

events. To toggle a particular switch between 'Y'es (ON) and  'N'o

(OFF), simply press the indicated number:

1: Sound ALARM in messages

When set to 'Y'es (ON), this switch causes LAPTALK to  sound

a BEEP whenever an error  (or  informational)  message  box  is

presented.

2: Echo data send in tty

When set to  'Y'es  (ON),  this  switch  causes  LAPTALK  to

display on the terminal  screen,  any  data  entered  from  the

keyboard, as well as sending it  to  the  remove  device.  This

option should be  set  ON  whenever  you  are  connected  to  a

computer which DOES NOT echo input data, and OFF  whenever  you

are using a system that DOES echo its input.

Incorrect setting of this option will cause the effect  that

either you cannot see what you are typing, OR you will get  two

characters displayed for every one that you type.

3: Echo data sent in scripts

When set to  'Y'es  (ON),  this  switch  causes  LAPTALK  to

display on the terminal screen, any data that is transmitted by

scripts files (as well as send it to the remote device).

The correct setting of this switch will depend on the nature

of the script file, and the remote system.

LAPTALK Users Guide                                              Page: 6

4: Send LF on ASCII uploads

When set to 'Y'es (ON), this switch causes LAPTALK to output

both a CARRIAGE RETURN and a LINE FEED at the end of each  line

of text which is transmitted during an ASCII upload.  When  set

to 'N'o (OFF), only a CARRIAGE RETURN.

When uploading to a computer expecting keyboard input,  this

switch is usually set OFF, because you do not normally  include

LINE FEED on keyboard input. When uploading  to  a  printer  or

terminal, setting this switch ON will prevent the  output  from

continuously overwriting itself one line.

5: Send space in NULL lines

When set to 'Y'es (ON), this switch causes LAPTALK to expand

any lines of ZERO LENGTH which  are  being  transmitted  during

ASCII uploads to contain a single space.

The text entry interface of many  online  systems  interpret

NULL lines as "end of entry". A null line in an  uploaded  file

will cause these system to stop accepting text (and usually get

very confused by the remainder of the uploaded  file).  Setting

this switch ON will prevent this from occuring.

6: Drop DTR+RTS on exit

When set to  'Y'es  (ON),  this  switch  causes  LAPTALK  to

de-assert the DTR and RTS modem control signals of the PC  COMM

port whenever you select "Exit to DOS" from the main menu. This

will usually hangup a modem, or terminate you login session  on

a multitasking system. The exact  effect  will  depend  on  the

device to which you are connected.

When this  switch  set  OFF,  LAPTALK  leaves  DTR  and  RTS

asserted on the PC COMM port, insuring that device to which you

are connected is unaware that you have terminated the  program.

You may then re-enter LAPTALK and resume your communication  at

some later time.

3.2 General parameters

This configuration sub-menu presents you with  a  selection  of

parameters which control various aspects of LAPTALK's operation:

Home directory

The directory name entered at this prompt determines the DOS

directory  where  LAPTALK  will  search   for   SCRIPT   FILES,

CONFIGURATION FILES, and FILE  TRANSFER  PROTOCOL  MODULES.  If

left blank, LAPTALK will search the current directory.

LAPTALK Users Guide                                              Page: 7

Tab spacing

This parameter defines the spacing of TAB STOPS in the  main

terminal screen. When a  TAB  character  is  received,  LAPTALK

advances te cursor to the next tab stop.

Modem hangup delay

This parameter defines  the  number  of  seconds  for  which

LAPTALK will drop the DTR and  RTS  modem  control  signals  in

order to hang up the modem. LAPTALK will hang up the  modem  in

rsponse to your selection of the "Hnagup modem" option  of  the

main function menu, or the occurance of a "hangup" command in a

script file.

String TX character delay

This parameter defines the length of time which LAPTALK will

wait between characters in a string being  transmitted.  String

transmissions occur as a result of several SCRIPT commands,  as

well as the ANSI function keys in tty mode.

The length if time  indicated  by  each  unit  of  value  is

dependant on the CPU  speed  of  your  PC.  You  will  have  to

experiment to obtain appropriate values for your application.

ASCII Upload char delay

This parameter defines the length of time which LAPTALK will

wait between characters transmitted during ASCII uploads.

The length if time  indicated  by  each  unit  of  value  is

dependant on the CPU  speed  of  your  PC.  You  will  have  to

experiment to obtain appropriate values for your application.

ASCII Upload line delay

This parameter defines the length of time which LAPTALK will

wait between lines transmitted during ASCII uploads.

The length if time  indicated  by  each  unit  of  value  is

dependant on the CPU  speed  of  your  PC.  You  will  have  to

experiment to obtain appropriate values for your application.

LAPTALK Users Guide                                              Page: 8

ASCII Upload sync. char

This parameter defines an ASCII character  value  for  which

LAPTALK will wait, after sending a line during ASCII uploads. A

value of zero (0) is a special  case,  and  causes  LAPTALK  to

ignore the sync character, proceeding immediately (after  ASCII

Upload line delay) with tramsission of the next line.

LAPTALK will only match characters which are received  AFTER

the line has been transmitted. This prevents any occurances  of

the defined character which  are  imbedded  in  the  line  from

triggering the next transmission.

LAPTALK will always wait for the  minimum  time  defined  by

"ASCII  Upload  line  delay"   before   proceeding   with   the

transmission,  regardless  of  when  the  sync.  character   is

received. It will also wait for one interval of time defined by

"ASCII Upload char delay" before proceeding  with  transmission

if the sync. character is received after the "line  delay"  has

expired.

In the event that a communication error or some other  event

prevents LAPTALK from  seeing  the  sync.  character,  you  can

manually proceed to the next line by pressing the SPACE bar.

3.3 Serial port settings

This configuration sub-menu allows you to select the COMM port,

Baudrate, Parity, Number of Data bits, Number of  Stop  bits,  and

Enable/Disable XON/XOFF flow control.

3.4 Transfer protocols

This configuration sub-menu permits you to define up  to  eight

file transfer protocols,  which  can  be  invoked  from  the  main

function menu,

For details on configuring the protocols, refer  to  the  "FILE

TRANSFER PROTOCOLS" section of this manual.

LAPTALK Users Guide                                              Page: 9

3.5 Function menu scripts

This configuration sub-menu permits you to define up  to  eight

script files (complete with arguments) which may be  invoked  from

the "Function menu" selection  of  the  main  LAPTALK  menu.  This

provides "fast access" to your favorite scripts.

Each entry has the following format:

[name;][script file name] [arguments]

[name;] is the  name  that  will  appear  in  the  script  file

selection window. It is delimited from the filename by a semicolon

(';'). If no name is given,  LAPTALK  displays  the  name  of  the

script file.

[script file name] is the full name of the script file which is

to run when this entry is  selected.  If  not  specified,  LAPTALK

assumes the extension ".SCR". Unless the name includes a directory

path, LAPTALK assumes that the file will be found in  the  LAPTALK

home directory.

[arguments] are any command  line  arguments  which  should  be

passed to the script file.

Here are a few example entries which implement a simple  "phone

list" using the "dial.scr" script supplied with LAPTALK:

BBS;dial 555-1234

WORK;dial 555-5678

JOHN;dial 555-9012

NOTE: See 'PHONE.SCR' for another way to set up a phone list.

LAPTALK Users Guide                                              Page: 10

3.6 Video attributes

This  configuration  option  allows  you  to  set   the   video

attributes (colors)  of  the  various  windows  and  screens  that

LAPTALK presents.

The following entries may be selected:

Main TTY screen

Video attribute (1-7)   *

Status line

Error message box's

String input fields     **

Main menu's

Sub-menu's

* - Video attributes 1-7 are selected with the ANSI escape sequence

"ESC[#n", where '#' is the number from 1-7. If '#' is specified

as '0', the "Main TTY screen" attribute is restored.

**- Also includes FILE SELECTION WINDOWS.

When you select this option, you will  be  presented  with  the

"Main TTY screen" attributes. You may use the PgDn and  PgUp  keys

to cycle between the selections listed above.

The Right and Left arrow keys may be used to cycle between  the

available FOREGROUND video options.

The Up and Down arrow keys may be used  to  cycle  between  the

available BACKGROUND video options.

Press ESCAPE when you have finished. NOTE: If you have  altered

the "Main TTY screen" or the "Status line" entries, the TTY screen

will be cleared to new attributes.

3.7 Load configuration

This option prompts you for a filename,  and  then  loads  that

file as a new LAPTALK configuration. The  LAPTALK  home  directory

and the extension ".CFG" are  assumed  if  not  specified  in  the

filename. Wildcard characters ('*' and '?') may be used, in  which

case you will be presented with a menu of the  matching  files  to

pick from.

3.8 Save configuration

This options prompts you for a filename,  and  then  saves  the

current LAPTALK configuration into that  file.  The  LAPTALK  home

directory and the extension ".CFG" are assumed if not specified in

the filename. Wildcard characters ('*' and '?') may  be  used,  in

which case you will be presented with a menu of matching files  to

pick from.

LAPTALK Users Guide                                              Page: 11

4. FILE TRANSFER PROTOCOLS

LAPTALK allows up to 8 file  transfer  protocols  to  be  defined,

which are implemented as external programs. Only the  ASCII  transfer

protocol is implemented internally.

4.1 Configuration

You set up  the  file  transfer  protocols  via  the  "Transfer

Protocols" option under the configuration menu.  You  may  make  8

entries, one for each protocol.

The format of the protocol definition entry is as follows:

[name;][executable file name] [arguments]

[name;] is the name that  appears  in  the  protocol  selection

window, of the file upload/download  functions.  It  is  delimited

from the filename by a semicolon  (';').  If  no  name  is  given,

LAPTALK displays the name of the executable file.

[executable file name] is the full name of the program which is

to run when the protocol is selected.  It  must  be  a  ".COM"  or

".EXE" file (LAPTALK assumes ".COM" is no type is  given).  Unless

the name includes a directory path, LAPTALK assumes that the  file

will be found in the LAPTALK home directory.

[arguments] are any command  line  arguments  which  should  be

passed to the program. The following "special  sequences"  may  be

used in the arguments field:

\A          - Use ASCII transfer instead of COM/EXE file.

\B          - Insert BAUD rate ('110'-'38400').

\C          - Insert COMM port ('1'-'4').

\D<string>\ - Insert <string> for DOWNLOADS only.

\F          - Obtain and insert a filename.

\G          - Like '\F' but ignored for downloads.

\H          - Insert LAPTALK home directory.

\N          - Insert number of DATA BITS (5-8).

\P          - Insert PARITY (Odd, Even, Mark, Space or None).

\S          - Insert number of STOP BITS (1-2).

\U<string>\ - Insert <string> for UPLOADS only.

\W          - Wait for ENTER after transfer.

\X          - Like '\F' but passes wildcards through.

\Y          - Like '\X' but ignored for downloads.

\Z          - ZAP (save, clear and restore) screen.

\\          - Insert a single '\'.

LAPTALK Users Guide                                              Page: 12

4.2 The XMODEM module

The  LAPTALK  package  includes  XMODEM.COM,  a  program  which

implements basic XMODEM  file  transfers.  It  has  the  following

command line syntax:

XMODEM <filename> [-C -D -P R=size P=port F=value S=speed]

The meanings of the command line options are:

-C      - Skip CRC negotiation & proceed with CHECKSUM

-D      - Perform DOWNLOAD instead of UPLOAD

-P      - Force pause ("Press any key") when finished

R=size  - Specify record size

P=port  - Specify COMM port (1, 2, 3 or 4)

F=value - Fill unused bytes of last upload record

with this value.

S=speed - Communicate at this baud rate

Here are some LAPTALK protocol entries for use with XMODEM.COM:

Xmodem \F s=\B\D -d\

Xmodem/1K;xmodem \F s=\B r=1024\D -d\

4.3 Examples

The internal ASCII transfer can be activiated with  a  transfer

protocol entry such as:

ASCII \F\A

Here are XMODEM, YMODEM and ZMODEM  transfer  protocol  entries

which make use of a popular shareware communications module called

DSZ:

XMODEM;DSZ port \C \Drx\\Usx\ \F\Z

YMODEM;DSZ port \C \Drb\\Usb\ \F\Z

ZMODEM;DSZ port \C \Drz\\Usz\ \G\Z

DSZ is available from:

Omen Technology INC

P.O. Box 4681

Portland, OR 97208-4681

FAX: 503-621-3735

LAPTALK Users Guide                                              Page: 13

5. SCRIPT INTERPRETER

LAPTALK includes a powerful "script" interpreter, which allows you

to set up automatic sequences of events to be performed as  a  single

operation.

The script interpreter can be considered as a simple  "programming

language", complete with commands, expressions and variables.

5.1 Insert Codes

Any any point in a script  file,  you  can  use  any  of  these

"Insert Codes", to cause the script interpreter to insert a  ASCII

string representing some value  into  the  command  line  at  that

point:

^c      - Insert control code of 'c'. (Ie: '^C' = 0x03)

\b      - Insert a BACKSPACE character. (0x08)

\d      - Insert a DELETE character.    (0x7F)

\e      - Insert a ESCAPE character.    (0x1B)

\h      - Insert LAPTALK home directory.

\r      - Insert a RETURN character.    (0x0D)

\n      - Insert a NEWLINE sequence.    (0X0D, 0x0A)

\s      - Insert the MODEM SIGNALS as a decimal value,

made up of these components (OR'd together):

1   - CTS has changed state.

2   - DSR has changed state.

4   - Ringing has just ended.

8   - CD has changed state.

16  - CTS state (1 = asserted)

32  - DSR state (1 = asserted)

64  - RI  state (1 = asserted)

128 - CD  state (1 = asserted)

* You can test the individual signals by ANDing ('&')

this value with a mask.

\#n     - Insert character with decimal value n.

\0-9    - Insert the corresponding argument (0-9) from the script

command line. Argument 0 is the name of the script.

\A-Z    - Insert the contents of the corresponding variable.

\\      - Insert a single '\'.

\^      - Insert a single '^'.

LAPTALK Users Guide                                              Page: 14

5.2 Script File Commands

Some of commands  described  in  this  section  accept  various

arguments which they use to perform their actions. Arguments shown

in angle braces ('<>') are REQUIRED.  Arguments  shown  in  square

braces ('[]') are OPTIONAL.

The various arguments accepted by the script commands are:

text        - The rest of the command line, which is treated as a

text string.

variable    - A script variable name ('A'-'Z'). Note that for

commands which specifically require a variable name,

you DO NOT include a preceeding '\' (otherwise the

CONTENTS of the variable would be passed).

script      - The name of a script file.

arguments   - Any arguments to a script file.

protocol    - A file transfer protocol name. This is the name

defined under the "Transfer Protocols" option of

the configuration menu, which appears in the

protocol selection list for uploads and downloads.

filename    - The name of a file to upload or download.

value       - An expression resulting in a numeric value. This

may consist of numbers (-32768 to 32767) and any

of the following operators:

+   - Addition          -   - Subtraction

*   - Multiplication    /   - Division

%   - Modulus           &   - Bitwise AND

|   - Bitwise OR        ^   - EXCLUSIVE OR

<<  - Shift Left        >>  - Shift Right

==  - Test for EQUALITY (0=False, 1=True)

!=  - Test for INEQUALITY

>   - Test for GREATER THAN

<   - Test for LESS THAN

>=  - Test for GREATER THAN or EQUAL

<=  - Test for LESS THAN or EQUAL

When multiple operators are present, they are

processed from LEFT to RIGHT, without precedence.

Round brackets '()' may be used for force the

grouping of operators into sub-expressions.

string      - A text string enclosed within delimiters.

It does not matter what characters are used for the

delimiters, as long as they are identical, and do not

occur within the string.

command     - Any script file command

label       - A line label. Any line can be labeled by placing a

colon (':') at the beginning of the line, followed

immediately by a label name.

LAPTALK Users Guide                                              Page: 15

5.2.1 abort <text>

Displays <text> as an error message, and then terminates all

scripts (including any which  called  the  one  containing  the

"abort" command).

5.2.2 assign <variable> <text>

Places  <text>  into  the  indicated  variable  (A-Z).  Each

variable can hold up to 50 characters.

5.2.3 call <script> [arguments]

Suspends processing of the current script file,  and  begins

executing the named one. The new script file will have its  own

environment (arguments, variables etc.) and will not affect the

environment of the  calling  script.  When  the  called  script

terminates, the original script will resume execution.

5.2.4 chain <script> [arguments]

Terminates execution  of  the  current  script,  and  begins

executing the named one. This function is  similar  to  "call",

except that the original script will not resume  when  the  new

one terminates.

5.2.5 clear

This command clears the main terminal screen.

5.2.6 close

This command is used to  stop  capturing  data  to  a  file,

following an ASCII download.

5.2.7 config [config filename]

Loads all LAPTALK options  and  parameters  from  the  named

configuration file.

5.2.8 dos [text]

The remainer of the  command  line  is  passed  to  the  DOS

command processor to be executed as a system command.

5.2.9 download <protocol> [filename]

This  command  will  download  the  named  file  using   the

specified protocol (From your protocol setup menu).

If <filename> is not supplied, you will be prompted for it.

LAPTALK Users Guide                                              Page: 16

5.2.10 echo <text>

The remainder of  the  command  line  is  displayed  on  the

console terminal screen,  without  a  following  line-feed  and

carriage-return. See also 'print'.

5.2.11 error <text>

Similar to "abort", except that the message is displayed  in

a window, and you must press ENTER to remove it.

5.2.12 equate <variable> <value>

Similar to  "assign",  except  that  the  remainder  of  the

command line is evaluated as  a  numeric  expression,  and  the

result is assigned to the variable as an ASCII number.

5.2.13 exittodos

Causes the entire LAPTALK program to terminate.  Control  is

returned to the DOS prompt.

5.2.14 flush [text]

Removes  all  previously  received  data  from  the  capture

buffer. If [text] is supplied, that text is then  written  into

the capture buffer.

5.2.15 goto [+]<label>

Searches the script file  for  the  location  of  the  named

label, and proceeds to execute commands from there.

If '+' is specified, the search will begin  at  the  current

line, instead of the beginning of the file. In  large  scripts,

this will speed up forward references to labels.

5.2.16 hangup

This command drops the DTR and RTS signals to the modem  for

a period of time in order to force it offline.

5.2.17 if <string> : <command>

Executes the named command only if <string> can  be  located

in the capture buffer.

5.2.18 ifnot <string> : <command>

Executes the named command  only  if  <string>  can  NOT  be

located in the capture buffer.

LAPTALK Users Guide                                              Page: 17

5.2.19 ifeq <string> <string> : command

Executes the named command only of the  two  <string>'s  are

identical.

5.2.20 ifne <string> <string> : command

Executes the named command only if the  two  <string>'s  are

NOT identical.

5.2.21 if# <value> : command

Executes the named command only if  the  numeric  result  of

evaluating <value> is not zero.

5.2.22 input <variable> [text]

Issues a prompt containing [text], and then reads  a  string

from the console which is assigned to the named <variable>.

5.2.23 invisible

Causes all data subsequently received from the  serial  port

to NOT be displayed on the terminal screen. See also "visible".

5.2.24 menu <variable> <text>

Presents a menu, with each word from <text> as a  selection.

The <variable> is set to 0 if the first entry  is  selected,  1

for the second, 2 for the third, etc. If the menu is aborted by

pressing ESCAPE, the variable is set to -1.

5.2.25 print [text]

The remainder of  the  command  line  is  displayed  on  the

console  terminal  screen,  followed   by   a   line-feed   and

carriage-return. See also 'echo'.

5.2.26 read <variable> <text>

Similar to "input", except that the prompt  is  issued,  and

the string is read from the serial port.

5.2.27 rem [text]

This command is used to insert remarks into the script file.

The remainder of the command line is simply ignored.

5.2.28 scan <string> <time> [: command]

Waits for either the specified <string> to be received  from

the serial port, or until <time> seconds  have  elapsed.  If  a

command is given, it will be executed if  the  string  was  NOT

seen before the timer expired.

LAPTALK Users Guide                                              Page: 18

5.2.29 send <text>

The remainder of the command  line  is  transmitted  to  the

serial port.

5.2.30 skip <value>

Skips ahead by the specified number of lines,  and  proceeds

to process the script file from  there.  In  other  words,  the

specified number of lines  following  the  one  containing  the

"skip" command will not be processed.

This is a convienent way to perform  multi-line  conditional

statements.

5.2.31 stop

Terminates processing of the current script  file.  If  this

file was called by another script file, that file will resume.

5.2.32 terminal

This command invokes the ANSI  terminal,  which  will  allow

interactive communication with the remote port  until  CTRL-END

if entered, following which the script file will resume.

5.2.33 upload <protocol> [filename]

This command will upload the named file using the  specified

protocol (From your protocol setup menu).

If <filename> is not supplied, you will be prompted for it.

5.2.34 visible

Causes all data subsequently received from the  serial  port

to be displayed on the terminal screen. See also "invisible".

5.2.35 wait <time>

Waits until no data has been received from the  serial  port

for the indicated number of seconds.

LAPTALK Users Guide                                              Page: 19

5.3 Fast Access Scripts

You may set up a "Function menu", with up to  8  "fast  access"

scripts (complete with arguments) that can be invoked from  a  pop

up menu. This is done with  the  "Function  Menu  Scripts"  option

under the "Configuration" menu.

The format of each function menu entry is:

[name;][script file name] [arguments]

[name;] is the name that appears in the script  file  selection

window of the "Function menu" function. It is delimited  from  the

script file name by a semicolon (';').

[script file name] is the full name of the script file which is

to be processed.  Unless  the  name  includes  a  directory  path,

LAPTALK assumes that the file will be found in  the  LAPTALK  home

directory. If not explicitly given, LAPTALK assumes the  extension

".SCR".

[arguments] are any command  line  arguments  which  should  be

passed to the script.

LAPTALK Users Guide                                              Page: 20

6. TERMINAL CONTROL CODES

LAPTALK recognizes the  following  subset  of  the  ANSI  terminal

control codes:

^G          - Sound ALARM

^H          - Backspace

^I          - Horizontal TAB

^J          - Line feed

^M          - Carriage return

<ESC>[H     - Home cursor

<ESC>[f     - Alternate home cursor

<ESC>[#;#H  - Position cursor to Y,X

<ESC>[#;#f  - Alternate position cursor

<ESC>[J     - Clear to end of screen

<ESC>[0J    - Alternate clear to end of screen

<ESC>[2J    - Clear entire screen

<ESC>[K     - Clear to end of line

<ESC>[0K    - Alternate clear to end of line

<ESC>[2K    - Clear entire line

<ESC>[#m    - Select video attributes (1-7 or 0=Normal)

The following special codes are transmitted for keys:

<ESC>[A     - Up arrow

<ESC>[B     - Down arrow

<ESC>[C     - Right arrow

<ESC>[D     - Left arrow

<ESC>OP     - PgUp

<ESC>OS     - PgDn

<ESC>OP     - Home

<ESC>OQ     - End

<ESC>OM     - Keypad '+'

<ESC>Om     - Keypad '-'

<ESC>Op     - Insert

<ESC>Oq     - F1

<ESC>Or     - F2

<ESC>Os     - F3

<ESC>Ot     - F4

<ESC>Ou     - F5

<ESC>Ov     - F6

<ESC>Ow     - F7

<ESC>Ox     - F8

<ESC>Oy     - F9

<ESC>Oz     - F10

^H          - Backspace

^I          - TAB

^M          - ENTER

Hex: 7F     - Delete

LAPTALK Users Guide

TABLE OF CONTENTS

Page

1. INTRODUCTION                                                        1

2. USING LAPTALK                                                       2

2.1 Command Line Options                                            2

2.2 LAPTALK Functions                                               2

2.3 Function menu                                                   3

2.4 Download file                                                   3

2.5 Upload file                                                     3

2.6 Kill capture                                                    3

2.7 Perform script                                                  3

2.8 Hangup modem                                                    4

2.9 Clear screen                                                    4

2.10 Configuration                                                  4

2.11 Shell to DOS                                                   4

2.12 Exit to DOS                                                    4

3. CONFIGURATION                                                       5

3.1 General switches                                                5

3.2 General parameters                                              6

3.3 Serial port settings                                            8

3.4 Transfer protocols                                              8

3.5 Function menu scripts                                           9

3.6 Video attributes                                               10

3.7 Load configuration                                             10

3.8 Save configuration                                             10

4. FILE TRANSFER PROTOCOLS                                            11

4.1 Configuration                                                  11

4.2 The XMODEM module                                              12

4.3 Examples                                                       12

5. SCRIPT INTERPRETER                                                 13

5.1 Insert Codes                                                   13

5.2 Script File Commands                                           14

5.3 Fast Access Scripts                                            19

6. TERMINAL CONTROL CODES                                             20