#

File: sony.info,  Node: Top,  Prev: (sharp.info),  Next: (teac.info),  Up: (remotes.info)

******************************************************************************

SONY

******************************************************************************

* Menu:

* Technical info::

* Tape::

* DAT::

* Video::

* CD::

* Amplifier::

* Phono::

* Tuner::

* TV::

* Laser disc player::

* Video camera::

#

File: sony.info,  Node: Technical info,  Prev: Top,  Next: Tape,  Up: Top

******************************************************************************

TECHNICAL INFO

******************************************************************************

Code length:	12 bits (15 bits for some videocamera functions)

Carrier:		40kHz

+--------+

Header:      |        |

+        +--+..

4T    T

+----+

1 is coded:  |    |

..+    +--+..

2T  T

+--+

0 is coded:  |  |

..+  +--+..

T  T

T = 550us approx

Space between data: 25ms

data: hhhhxxxxxxyyyyyy

^          ^

MSB        LSB

xxxxxx command

yyyyyy address ( the number of the apparatus )

tape deck a    # 20

tape deck b    # 1C

dat       # 38

cd        # 22

#

File: sony.info,  Node: Tape,  Prev: Technical info,  Next: DAT,  Up: Top

******************************************************************************

TAPE

******************************************************************************

deck a	deck b

Code	Code	Function

----	----	--------

#70E		Memory

#70F		Counter reset

#716		Open

#717		Monitor

#74B		Display change

#830		auto play

#832	#71A 	play

#833	#71B    rew

#834	#71C	ff

#835	#71D	rec

#836		rec pause

#837	#720	reverse play

#838	#718	stop

#839	#719	pause

#83A	#712	reverse rec

#

File: sony.info,  Node: DAT,  Prev: Tape,  Next: Video,  Up: Top

******************************************************************************

DAT

******************************************************************************

Code	Function

----	--------

#E3A	play

#E3B	rew

#E3C	ff

#E3E	rec

#E38	stop

#E39	pause

#E00	1

#E01	2

#E02	3

#E03	4

#E04	5

#E05	6

#E06	7

#E07	8

#E08	9

#E14	0

#E20	open/close

#E21	counter mode

#E22	counter reset

#E25	display PGM time

#E26	display absolut time

#E27	display custom counter

#E28	display remaining time

#E29	date, present

#E2A	date, recorded

#E36	repeat one/all

#E4B	display mode (all/time/none) or E77

#E4E	margin reset

#E57	rewind and play

#E5B	skip play mode

#E5F	fader

#E6C	music scan

#E2D	record pause

#E3E	record at once

#E4F	record mute

#E69	title +

#E68	title -

#E50	start id auto

#E53	start id renumber

#E51	start id write

#E52	start id erase

#E54	skip id write

#E55	skip id erase

#E58	end id write

#E59	end id erase

#E71	display mode (level/sampling)

#E71    dimmer low

#E70	dimmer mid

#E6F	dimmer high

#E1B	RMS enter

#E1D	RMS check

#E1E	clear

#E56	?? <- I don't know what this exactly

#E73	?? <- is; maybe any service-features

#

File: sony.info,  Node: Video,  Prev: DAT,  Next: CD,  Up: Top

******************************************************************************

VIDEO

******************************************************************************

vtr 1	vtr 2	vtr 3

Code	Code	Code	Function

----	----	----	--------

#100	#380	#580	1

#101	#381	#581	2

#102	#382	#582	3

#103	#383	#583	4

#104	#384	#584	5

#105	#385	#585	6

#106	#386	#586	7

#107	#387	#587	8

#108	#388	#588	9

#109	#389	#589	0

#10B	#38B	#58B	enter

#110	#390	#590	ch+

#111	#391	#591	ch-

#114	#394	#594	x2

#115	#395	#595	power

#116	#396	#596	eject

#117	#397	#597	audio select

#118	#398	#598	stop

#119	#399	#599	pause

#11A	#39A	#59A	play

#11B	#39B	#59B	rew

#11C	#39C	#59C	ff

#11D	#39D	#59D	rec

#128	#3A8	#5A8	search <<

#129	#3A9	#5A9	search >>

#12A	#3AA	#5AA	ant

#12E	#3AE	#5AE	power on

#12F	#3AF	#5AF	power off

#130	#3B0	#5B0	frame <-

#131	#3B1	#5B1	frame ->

#13A	#3BA	#5BA	rewind/play

#13D	#3BD	#5BD	slow motion +

#13E	#3BE	#5BE	slow motion -

#142	#3C2	#5C2	menu up

#143	#3C3	#5C3	menu down

#146	#3C6	#5C6	counter reset

#148	#3C8	#5C8	mark index

#149	#3C9	#5C9	erase index

#14A	#3CA	#5CA	shuttle >>

#14B	#3CB	#5CB	shuttle

#14D	#3CD	#5CD	menu

#14F	#3CF	#5CF	input sel.

#151	#3D1	#5D1	execute

#152	#3D2	#5D2	quick timer

#153	#3D3	#5D3	index select

#158	#3D8	#5D8	rec. mode

#159	#3D9	#5D9	go to 0

#15A	#3DA	#5DA	data screen

#161	#3E1	#5E1	menu right

#162	#3E2	#5E2	fcn memory

#162	#3E2	#5E2	menu left

#163	#3E3	#5E3	timer clear

#174	#3F4	#5F4	channel scan

Note: Only the "VTR3" position codes have benn tested;

the others I calculated by simple arithmetic subtraction, as this seems

constant for all of the VCR codes in the first codes.

#From: geh@mda.ca (Graeme Hiebert)

File: sony.info,  Node: CD,  Prev: Video,  Next: Amplifier,  Up: Top

******************************************************************************

CD

******************************************************************************

Code	Function

----	--------

#880	1

#881	2

#882	3

#883	4

#884	5

#885	6

#886	7

#888	8

#889	9

#88A

#88B

#88C

#88D	Check   file text

#88E

#88F	Clear

#890	17

#891	18

#892	Line Out Volume Up

#893	Line Out Volume Down

#894

#895

#896	Open/Close

#897	19

#898	Custom Index

#899	File

#89A	File recall

#89B	Erase

#89C

#89D	Continue

#89E

#89F	Program

#8A0	10

#8A1	11

#8A2	12

#8A3	13

#8A4	14

#8A5	15

#8A6	16

#8A7	+10

#8A8	Time

#8A9	20

#8AA	Repeat A<->B

#8AB

#8AC	Repeat 1/All

#8AD

#8AE

#8AF

#8B0	AMS Backward

#8B1	AMS Forward

#8B2	Play

#8B3	Fast Audible Search Backward

#8B4	Fast Audible Search Forward

#8B5	Shuffle

#8B6	Index Backward

#8B7	Index Forward

#8B8	Stop

#8B9	Pause

#8BA	Slow Audible Search Backward

#8BB	Slow Audible Search Forward

#8BC	Music scan

#8BD

#8BE

#8BF

#8C0	Select Disc #1

#8C1	Select Disc #2

#8C2	Select Disc #3

#8C3	Select Disc #4

#8C4	Select Disc #5

#8CE	Auto space

#8CF	Peak search

#8D0

#8D1

#8D2

#8D3

#8D4

#8D5

#8D6	Display mode

#8D7

#8D8

#8D9

#8DA

#8DB

#8DC

#8DD	DSP Select

#8DE	DSP Flat

#8DF	Fade

#8E8	Level file

#8E9	Memo input

#

File: sony.info,  Node: Amplifier,  Prev: CD,  Next: Phono,  Up: Top

******************************************************************************

AMPLIFIER

******************************************************************************

Code	Function

----	--------

#631	dbfb

#642	rear lvl+

#638	off

#639	dolby

#63A	hall

#63B	simulated

#643	rear lvl-

#644	delay time

#812	vol+

#813	vol-

#814	muting

#815	power

#81E	video 2

#820	phono

#821	tuner

#822	video 1

#823	tape

#825	cd

#842	video 3

#843	video 4

#846	dat

#63F	DSP

#

File: sony.info,  Node: Phono,  Prev: Amplifier,  Next: Tuner,  Up: Top

******************************************************************************

PHONO

******************************************************************************

Code	Function

----	--------

#828	start

#829	stop

#

File: sony.info,  Node: Tuner,  Prev: Phono,  Next: TV,  Up: Top

******************************************************************************

TUNER

******************************************************************************

Code	Function

----	--------

#63B	shift

#690	scan+

#691	scan-

#680	1

#681	2

#682	3

#683	4

#684	5

#685	6

#686	7

#687	8

#688	9

#689	10

#68A	11

#68B	12

#6A6	>12

#6B0	A

#6B1	B

#6B2	C

#6B3	D

#6B4	Tuning/Preset

#6CB	Display

#68E	Memory

#68F	Band

#692	Tune +

#693	Tune -

#6A1	Stereo/Mono

#84B	Clock disp

#

File: sony.info,  Node: TV,  Prev: Tuner,  Next: Laser disc player,  Up: Top

******************************************************************************

TV

******************************************************************************

Note:	Many of the codes were taken by Frank Hepmann

from a Sony monitor. But since the monitor has

the same adress as a normal TV these codes were

included here.

Code	Function

----	--------

#080	1

#081	2

#082	3

#083	4

#084	5

#085	6

#086	7

#087	8

#088	9

#089	0

#08B	enter

#090	ch+

#091	ch-

#092	vol+

#093	vol-

#095	power (toggle)

#0AE    power on

#0AF	power off

#0A5	tv/video

#098	picture +

#099	picture -

#09A	colour +

#09B	colour -

#09E	brightness +

#09F	brightness -

#0A0	hue + (purpur)

#0A1	hue - (green)

#0A2	sharpness +

#0A3	sharpness -

#0A6	balance L

#0A7	balance R

#096	normal values

#0C0	input line A

#0C1	input line B

#0C3	input AV

#0C5	input digital

#0C7	input vtr

#094	mute

From: Howard Chu <howard@harry.lloyd.com>

These codes are for my Sony XBR50 TV. It's about 3 years old now, so I bet

there are still new codes out there...

#097	MTS Stereo mode (Stereo/Mono/SAP)

#0A4	TV (tuner input)

#0A8	SuperWoofer level

#0A9	SRS Sound Retrieval System mode (Stereo/Mono/Off)

#0AA	Antenna/Aux RF input toggle

#0B0	Time display

#0B6	Sleep

#0BA	Channel display

#0BB	Jump (previous channel)

#0C2	Video 3 input

#0CE	Cable toggle

#0E0	Guide menu

#0E1	Video menu

#0E2	Audio menu

#0E3	Exit menu

#0E4	ASC (Automatic Signal Correction)

#0E8	Auto program channels

#0F4	AV+ menu selector

#0F5	AV- menu selector

#0F8	Add channel

#0F9	Erase channel

#

File: sony.info,  Node: Laser disc player,  Prev: TV,  Next: Videocamera,  Up: Top

******************************************************************************

LASER DISC PLAYER

******************************************************************************

Code	Function

----	--------

#315	power

#318	stop

#31A	play

#31B	rew

#31C	ff

#335	skip rew

#334	skip ff

#

File: sony.info,  Node: Videocamera,  Prev: Laser disc player,  Up: Top

******************************************************************************

VIDEOCAMERA

******************************************************************************

Code    Function

----    --------

#398	Stop

#399	Pause

#39A	Play

#39B	Rew

#39C	FF

#3DA	Data screen

#5C99	Start/Stop

#6C9A	Tele

#6C9B	Wide

Tele, Wide and Start/Stop uses 15 bits and requires

SONYCAM as infostring in order to work.

The other codes the normal SONY infostring.