-----BEGIN PGP SIGNED MESSAGE-----

- IR / RS-232 Transceiver -

96/04/23 15:05

FILES:     ir_box3.txt      - this file

ir_box3.ps       - schematic diagram

UNIX sum of ir_box3.ps: 13158 192

UNIX sum -r ir_box3.ps: 20767 192

The IR / RS-232 Transceiver module will convert the signal from a

UART port into infrared (IR) format, and from IR back to UART

format.  It will handle baud rates from 300 to 38400 baud.  The

IR format supports half-duplex communications only.  The IR

signal is compatible with the HP48 series calculators.  Although

I have not tried, the IR signal closely resembles the IrDA

format, and I suspect this module should be able to talk to

laptop computers and other devices with an IrDA port.  The RS-232

port is a DTE port, which means it can be plugged directly into a

modem or serial printer.  For connection into a PC computer, a

null modem cable is required (the wiring diagram is included).

The IR format dictates half-duplex communications.  The module

will enforce this by disabling the IR receiver while the IR

transmitter is transmitting data.

The module can use either a 6V battery or a 9V battery.  A 6V

battery (like 4-AA cells) will last longer, but the regulator is

bypassed.  I find a 9V battery is more portable.  The current

drain is 60mA idle, 70mA during file transfer.  Don't plug the

9V battery into the 6V connector.  That would be bad, very bad!

The basic design is based on the document at

"http://www.ee.ualberta.ca/~charro/cookbook/optic/HP48SXIR.asc"

by Tony Duell (ARD@PVA.BRIS.AC.UK).  The IR transmitter and IR

receiver runs on a 16x clock source for the desired baud rate.

U1A, U1B, U2A form the cristal oscillator.  C2 is critical.  The

2.4576MHz crystal do not like to oscillate at its fundamental

frequency (at least the ones I tried don't).  C2 will suppress

the overtones.  U3 and U6A form the 16X baud rate generator.  The

baud rate is selectable using S1.  The IR transmitter is quite

simple.  It will send a 1/2-bit wide pulse for every SPACE on the

RS-232 port.  The IR receiver uses U1, C4 to shape the IR pulse

going to U6B pin 4, and to create a reset pulse to synchronize

U5A with the UART on the IR side.  When the IR transmitter is

active (D1 is on), Q1 is disabled by Q2.  For a short duration

after the D1 switches off, Q1 will still be saturated and require

a recovery period.  This is provided by C3, R5.

Four LED's show the status of the RS-232 DTE interface.  The Tx,

Rx, CTS, and DSR lines are monitored.  DTR and RTS lines can be

pulled high, or left unpowered.  This is selectable using S1.  I

chose a DTE pin-out for the RS-232 port because this allows me to

plug the module directly into a modem or serial printer.  A null

modem cable is required to connect to a PC.  The wiring diagram

for this is included.

My next project will be a DAT cassette size serial/parallel

converter so I can use my EPSON printer from the HP48.

Enjoy,

Tom Wong (tomwong@bnr.ca / tomwong@nortel.ca)

- - -----------------------------------------------------------------

<Standard disclaimers here; freeware; no warranties; not

responsible for anything; email your comments; etc. etc.>

Changes:

- --------

ir_box3: added a missing wire to the schematic

ir_box2: added a warning about 6V/9V connector

ir_box:  original copy

-----BEGIN PGP SIGNATURE-----

Version: 2.6

iQCVAgUBMX0r6N3Bn6FMP/1JAQHMmQQAnriDuvfyy4P6JBsvq6Y3DobIkAflttqX

T61aSglLzsU/qHTVmwjtSk8zzwz7R3Pb54W/VTJRFLVkTgx57OvvehEJgoeJY2nK

pYx80b51JKYTVG6cirN6yuLK2TNfOjV7stcxbYsIjDRIco0pa8+Qy8BgA5ZP4g8C

5hH+2evv2c4=

=Py93

-----END PGP SIGNATURE-----