BODE-ROUTH

v5.1

Plus!

(c)APO

for the HP48-G,GX

FIRST OF ALL

============

English is not my native language so I'd like to apologize for the

errors in this doc.

THE FILES

=========

BodeR5x1.lib	The spanish library v5.1

BodeReng.lib	English version 5.1

BodeR5x1.esp	Spanish manual.

BodeR5x1.eng	English manual.

INSTALATION

==========

WARNING !!!

This library needs the polynomial lib NEOPOLYS (version 3.5 or better

with Laplace transforms ) to work properly.

Just download the .lib you want with the kermit server, press the key

with the name of the file to push the library in level 1 and also push on

the stack the number of the port where you want to store Bode-Routh and

press STO. Now reboot your HP by pressing ON-C or turning it off and on.

THINGS YOU HAVE TO KNOW

=======================

Polynomials are represented as in the NEOPOLYS lib either by their

coefficients or by their roots. For instance:

X^3 - 2X^2 - 5X + 6

can be represented as

{ 1 -2 -5 6 }

or

{ { 1 3 -2 } }

All commands (except "Routh") work only with numerical coefficients.

COMMAND REFERENCE

=================

I think it's very obvious what each command does. Moreover they are

very easy to understand because almost all of them were made with input

forms with fields that have by default the correct values for a nice

drawing.

Bode

----

With this command you can draw both the magnitude and the angle Bode

plots. In the magnitude plot the vertical axis is in decibels and in the

angle one in degrees. In both the horizontal axis is Log(w).

* The fields:

NUM,DEN : Numerator and denominator of the transfer function.

D	: Delay of the transfer function if any.

T	: Choose the type of plot to draw.

RES	: Graphic resolution. A number between 0 and 1 representing the

distance between two consecutive points of the plot.

WARNING !!!

When drawing the angle, a small value in this field can lead

to draw an incorrect plot because this command uses a correction

in the phase to surpass the 180 limit. If you have doubts

about changing or not this value let it as is to ensure that the

plot will be correct.

AUTO: Check this field to automatic scale vertical axis.

HR

VR	: Horizontal and vertical ranges of the plot.

* The menu:

EDIT,CHK

CHOOS: The normal input form menu for editing fields.

PICT : Press here to go to PICTURE.

ERASE: Erase PICT.

DRAW : Press here to start drawing.

After pressing NXT...

LABL : To label the plot before drawing it with the minimum and

maximum values for the axes.

AXES : Check it to draw the axes.

GFM	 : Gain and phase margins and associated frequencies.

NOTES:

Before entering Bode you can push on levels 1 and 2 the transfer

function and the fields NUM and DEN will take those values.

Likewise when exiting Bode you'll get the transfer function on the stack.

At any moment you can press the DEL key to reset values of one field

or the whole fields.

When drawing the angle diagram you'll see a dotted line at -180 and

180.

Nyquist

-------

* The fields:

All fields have the same meanning that in the previous command. There

is just one different which is "w". This value is the range of values that

the variable w takes when drawing the plot. Thus a value of 6.5 means that

the range is from -6.5 yo 6.5. You can increase this value to obtain more

resolution near the (0,0).

* The menu:

Is exactly the same that in the previous command.

NOTES:

If you see that drawing takes a lot of time it must be due to a great

value of the "w" field. Just press the ON key to abort at any moment.

The autoscale function is not very efficient for great values of

"w".

Nichols

-------

With this command you can plot ABS(G(jw)) vs ARG(G(jw)) also known

as Nichols' diagram for the transfer function G(s).

* The fields:

They are exactly the same that those for the Nyquist.

* The menu:

The same thing of the same one.

NOTES:

The correction we talked about in the Bode plot to exceed the 180

limit could have been applied here. But I prefered to let it as is because

thus you can easily calculate the gain and phase margin. For instance:

Push on the stack the next transfer function:

2:               { 4 }

1: 		   { 1 2 4 0 }

and now press "Nichols". Choose a resolution of .1 and press "DRAW". After

a while you'll see how the plot jumps from left to right of the LCD. Well,

press the ON key and place the cursor where the plot cross the horizontal

axis and if you press the (X,Y) menu key you'll see aproximately the next

coords:

X: -1.30E2          Y: -3.17E-1

Now add 180 degrees to the x value and you'll obtain the phase margin (or

substract -180): -130-(-180)=50.

To obtain the gain margin just place the pointer to the right, where the

plot arrived after jumpping one side to another and you'll see the new

coords:

X: 180          Y: -6.03E0

and the gain margin is finally (-1)(-6.03)=6.03.

You can obtain the exact values of these gain and phase margins with the

GFM option in the Bode command menu saw previously:

gain margin  =  6.020599... dB

phase margin = 50.290385...

Easy!.

See also the Nyquist note about the field "w".

RLoc

----

This command calculates and plots the locus of the roots of

NUM

1 + K ------- = 0

DEN

for a set of gains K.

* The fields:

RES : Resolution. This is the increment of the gain K at each step.

AU	: Check it to autoscale horizontal and vertical axes automaticly.

LIN : By default the root locus is drawing with points. Check this

field to connect the points.

* The menu:

There is just one different option:

DATA : By pressing this option you'll see another window where you

can choose between calculate the closed loop poles, open loop

zeros or stability. The first two options are trivial and the

last one let you see the coords where the root locus cross the

vertical axis and values of gain K for those points.

NOTES:

When drawing the root locus you'll see some "x" at the location of the

closed loop poles and some "o" at the open loop zeros.

When drawing, press any key to stop or the ON key to abort.

Resp

----

With this command you can obtain and plot the response of a system to

different inputs such as pulse, step, ramp and sine.

* The fields:

T	: Type of the input signal.

A	: Its meanning is different for each input:

* step: in this case this value is the magnitude of the

step;

* ramp: inclination;

* pulse: in this case its value is not considered;

* sine: wave amplitude.

w	: This field only has meanning with the sine input and it's the

wave frequency.

AUTO: Autoscale vertical axis.

* The menu:

INPU: Activate this option to draw the input with points.

It has no meanning for a "Pulse" input.

CNCT: Activate it to connect the points of the input graph when INPU is

checked.

NOTES:

In the EQ variable you'll obtain the response function which is the

reverse Laplace transform of the transfer function along with the input

signal.

Routh

-----

Applies the Routh-Hurwitz stability test to the polynomial at level 1.

The result is the first column of the Routh matrix at level 1 and the

whole matrix at level 2.

Special cases such as null rows or a null first coef of the previous

row are also considered. In the last case this coef will be replaced with

1.E-50.

You can also use constants and symbolic expressions.

Example 1:

2:                        Routh     2:  { { ......... } }

1:      { 1 2 3 4 5 } ->->->->->->  1:     { 1 2 1 -6 5 }

meanning:

+-----------------------------------+

s^4 + 2s^3 + 3s^2 + 4s + 5 = 0

+-----------------------------------+

+---+

s^4 :  1  3   5

s^3 :  2  4   0

s^2 :  1  5   0

s^1 : -6  0   0

s^0 :  5  0   0

+---+

Example 2:

2:                          Routh       2: { { ......... } }

1:  { 1 '4/2+4' 3 2 K } ->->->->->->->  1: { 1 3 2.33...etc}

->CL (Close the loop)

----

Given two transfer functions and a real number returns the closed loop

transfer function consisting of the feedback connection of the two

systems.

Negative or positive feedback is considered depending on the sign of the

real number at level 1.

E

U(s)---->0---> G(s) -------> Y(s)

/\             |

|               |

---- H(s) <----

G(s)

=> F(s) = ---------------

1  G(s)H(s)

sign:

+ <=> negative feedback

- <=> positive feedback

For example:

5:               { 1 }   \

4:			 { 1 2 4 }   / G(s)

3:			   { 1 2 }   \

2:				 { 1 }   / H(s)

1:					 1   (positive feedb.)

->CL	=>  4:			     { 1 }   \

3:			 { 1 1 2 }   / F(s)

or:

5:               { 1 }   \

4:			 { 1 2 4 }   / G(s)

3:			   { 1 2 }   \

2:				 { 1 }   / H(s)

1:					-1   (negative feedb.)

->CL	=>  4:			     { 1 }   \

3:			 { 1 3 6 }   / F(s)

For H(s)=1 just push the polynomial { 1 } at levels 2 and 3.

THANKS TO

=========

Mika Heiskanen specially for his Jazz and for his help to all

programmers through his docs and programs.

Sebastian Carlier for his Emu48 for Windows 95. Really you got it !.

All the persons who spend a bit of their time to contact the author and

comment	some bugs and suggestions.

CHANGES FROM VERSION 5.0

========================

There was corrected an error in the "Bode" command for which the re-

sults in the calculations of the gain and phase margins was not correct

when working in "RAD" mode.

When drawing the response to an input of type "Pulse" the graph spent

some time trying to draw the input function. That was corrected and now

the "Pulse" input function is not drawn even when the option "INPU" is

activated.

CHANGES FROM VERSION 2.3

========================

There are so many many many many .....

TO CONTACT THE AUTHOR

=====================

I'd like to know if you found this lib usefull.

If you found any bug or you have any suggestion please e-mail me to

apolo@sorolla.eis.uva.es