#

File: yamaha.info,  Node: Top,  Prev: (technics.info),  Up: (remotes.info)

******************************************************************************

YAMAHA

******************************************************************************

* Menu:

* Technical info::

* CD::

* Tape::

#

File: yamaha.info,  Node: Technical info,  Prev: Top,  Up: Top

******************************************************************************

TECHNICAL INFO

******************************************************************************

Code length:            32 bits

Carrier:                39.2kHz

The code is only transmitted once, and after that a stop code is transmitted.

+--------------------------------+

Header:      |                                |

+                                +----------------+..

16T                       8T

8.8ms                     4.4ms

+--+

1 is coded:  |  |

..+  +------+..

T   3T

+--+

0 is coded:  |  |

..+  +--+..

T  T

+--+

|  |

After the last bit a ( ..+  +  ) is transmitted to terminate the last bit.

T

+--------------------------------+        +--+

Stop code:    |                                |        |  |

+                                +--------+  +

16T                  4T     T

8.8ms                2.2ms

T = 550us

Space between transmissions: 95000us between the headers of the codes.

To use the code (not with the remote package):

GROUP * 10000h + (Code * 100h) + (Code XOR FFh)

For more information please read the documentation.

#

File: yamaha.info,  Node: CD, Prev: Technical info, Next: Tape, Up: Top

******************************************************************************

CD

******************************************************************************

GROUP   #9E61

Code    Function

----    --------

#08     0

#88     1

#48     2

#C8     3

#28     4

#A8     5

#68     6

#E8     7

#18     8

#98     9

#02     10

#82     11

#42     12

#C2     13

#22     14

#A2     15

#62     16

#E2     17

#12     18

#92     19

#52     20

#58     +10

#40     Play

#C0     Pause/Stop

#A0     Rew

#60     FF

#D0     Index

#80     Open/Close

#90     Pepeat A-B

#10     Pepeat S/F/Off

#E0     Track +

#20     Track -

Codes from Tom Bolt

#30     Program

#D8     Random

#00     Mode

The second byte of the hex code is # ffh minus the first byte.

Of these 256 combinations, all of the useful commands have the first

byte x0, x2, x4, x8 or xA.

YAMAHA

There is no play-exchange remote command. ;-(

The first byte of the existing commands are:

0       2       4       8       A

0       mode    10      -       0       <D

1       repeat  18      D8      8       -

2       <T      14      D4      4       -

3       program 22      -       -       -

4       play    12      D2      2       -

5       time    20      D10     +10     +30

6       S>      16      D6      6       stop

7       -       24      -       -       -

8       open    11      D1      1       -

9       -       19      D9      9       +20

A       <S      15      D5      5       pause

B       clear   23      -       -       -

C       pau/stp 13      D3      3       -

D       index   21      -       random  -

E       T>      17      D7      7       -

F       -       D>      -       -       -

S=search, T=track, D=disk.

Tom Boldt

che043sa@sail.uwaterloo.ca

(519) 746-5163

#

File: yamaha.info,  Node: Tape,  Prev:  CD,  Up: Top

******************************************************************************

TAPE

******************************************************************************

GROUP   #FE01

Code    Function

----    --------

#00     Play

#C0     Stop

#80     Rew

#40     FF

#31     Rec/Pause

#C0     Rec/Mute

#B0     Reset

#70     Memo

#B8     Remain

#EB     Tape

#D0     Search rew

#50     Search ff

#F8     Intro rew

#78     Intro ff

#08     0

#88     1

#48     2

#C8     3

#28     4

#A8     5

#68     6

#E8     7

#18     8

#98     9