Author: Pete M. Wilson

README.DOC

Brief description file for the TA disassembler system.

TA is a program the provides an interactive disassembler for the HP 48SX

ROM.  It is written in Microsoft C and should be fairly portable (I haven't

tried).

TA uses 5 data files:

HP48MAP.ROM is an ascii listing of the complete ROM contents, produced

by downloading all of the ROM to your computer and processing

it through the TOROMMAP filter.  This will strip all addresses

from the dump file, creating a flat 524,288 byte file.

Two comment files are used by TA. Both have the same format. Each line

consists of the 5 nibble address being described, followed by a space and the one line comment describing the address. Lines beginning with a '-' are considered deleted.

REMC.MAP is the code comment file. It provides comments explaining what the

code at a particular address is doing.  When the corresponding

address is UnThreaded or UnAssembled, the comment is appended

to the output line.

For example, the RPL routine at 03188 executes the inner-most code

for the DUP routine (i.e. after error checking and setup has been

done). It is an assembler routine with an RPL entry point, and

UnThreading the entry point yields:

TA> ut 3188

; *** RPL do DUP ***

03188: 0318D ML ENTRY

0318D: CF         D=D-1 A               ; room on stack?

0318F: 41F        GOC 03181             ; if not,  make some

03192: 143        A=DAT1 A              ; get obj @ TOS

03195: 1C4        D1=D1- 5              ; push object again

03198: 141        DAT1=A A

0319B: 142        A=DAT0 A              ; cont RPL

0319E: 164        D0=D0+ 5

The comments following each disassembled line are stored in REMC.MAP.

NOTE: The disassembler stops after outputting 8 lines - the

routine above continues for one more line.

REMA.MAP is the address comment file. It provides comments explaining the

purpose of address references. For example, the DUP routine is at

1FB87.  UnThreading this yields:

TA> ut 1fb87

; *** XLIB: 10D002 DUP ***

1fb87: 02D9D Type 8: RPL Program

1fb8c: 18AA5 save Last RPL Token / verify DEPTH >= 1

1fb91: 03188 RPL do DUP

1fb96: 0312B End RPL Program

Each comment following the threaded routine address is stored in

REMA.MAP.  In addition, the comment line preceding 1FB87 (and in

the above example, preceding 03188) is stored in REMA.MAP.

As a further example, the RPL routine at 14039 is a very deep internal

routine to the code for ERRN.  It pushes the Last Error Number as a

short integer or address type.

TA> ut 14039

; *** Push Last Err# as Short Int ***

14039: 1403E ML ENTRY

1403E: 34FF607    LCHEX 706FF   ; Save Last Err#

14045: 8D3B411    GOVLNG 114B3  ; Push @C.A as new Short Int

; *** Push @C.A as new Short Int ***

114B3: DA         A=C A

114B5: 8FB9760    GOSBVL 0679B  ; Save Regs D, B, D1, D0 (uses C)

114BC: 130        D0=A

114BF: 142        A=DAT0 A

114C2: 8DC7530    GOVLNG 0357C  ; Push A as new Short Int

The comments after 706FF, 114B3 0679B and 0357C are stored in

REMA.MAP and are automatically appended to the disassembly.

REMA.INX and REMC.INC are index files for the .MAP files.  These are binary

arrays of Adress, .MAP offset information in sorted order. A binary

search on these arrays in core is used to lookup comment information.

If the .MAP file is edited using a text editor, the corresponding

.INX file must be deleted. The TA program will automatically

re-create them correctly.

A number of features of TA disassembly should be apparent from the examples.

Here is a listing of some of the more significant features.

1. Initial disassembly mode is set by the command used to start

disassembly.

2. UnThread to UnAssemble mode switching is handled automatically.

3. Disassembly/unthreading is stopped automatically when a termination

point is reached.

4. Absolute GOTOs in assembly are followed automatically.

5. The HP-71 style mnemonics are used. (but see notes at end)

6. Currently, disassembly is only available for the ROM. Other Binary

files cannot be handled (i.e. I'm working on the USAG command).

7. In-line literal RPL Objects are not handled by the UnThreader.

8. Only the HP-71 instruction set is currently understood. I don't

have access to Alonzo's notes on the 28 or the new instructions

in the 48 - if anyone can tell me these, I'd be overjoyed to add

them to the program.

SHORT COMMAND REFERENCE

UT	UnThread

UT addr

Example: UT 3188

The UnThread command begins disassembly in UnThread mode at a given

address. Up to 8 lines of mixed RPL/assembly output will be produced.

If an RPL end program is encountered, output will stop. If RPL CODE or

ML Entry is encountered, disassembly mode will be switched to

UnAssemble automatically.

UA	UnAssemble

UA addr

Example: UA 114B3

The UnAssemble command begins disassembly in UnAssemble mode at a

given address. Up to 8 lines of assembly will be produced. If an

Unconditional Return or an indirect Goto (i.e. PC=(A)) is encounterd,

disassembly will be stopped. If an unconditional Goto is encountered,

it will be followed.

U	Un-continue

U

The Unassemble command continues at the last unassembled address, in

the last used mode, for another 8 lines or until a stop condition is

reached.

CA	Comment Address

CA addr text

Example: CA 114b3 Push @C.A as new Short Int

The Comment Address command creates a new comment on an address. Any

reference to the address will automatically have the comment appended

to the line. Any existing address comment for the address is replaced.

CC	Comment Code

CC addr text

Example: CC 318d room on stack?

The Comment Code command creates a new comment on the code at an

address. Any disassembly of the address will have the comment

appended to it. Any existing code comment for the address is replaced.

CCD	Comment Code Delete

CCD addr

Example: CCD 318d

The Comment Code Delete command removes a code comment for an address.

CAD	Comment Address Delete

CAD addr

Example: CAD 114b3

The Comment Address Delete command removes an address comment for an

address.

D	Dump

D addr

Example: D 71AD8

TA> d 71ad8

71ad8: E06556273796F6E6 0284054383D22411  .Version  HP48-B.

71af8: 34F6079727967686 4702840502139383  Copyrigh t HP 198

The Dump nibbles command outputs the nibbles at a particular ROM

address in Hex and ASCII format. It produces 8 lines of output.

Q	Quit

Q

The Quit command exits TA.  If any .INX files have been changed or

were missing, they are written out automatically.

Future Enhancements

These are some things I'm hoping to add to the program.  Please feel free to

make suggestions.

A utility program to merge REMC/REMA files from different people.

Automatic recovery of space in REMC/REMA.

Complete instruction set support (if someone will tell me what the new

instructions are).

A switch to support Alonzo's mnemonics (if I find out what they are).

A type file that marks the type of code at addresses so RPL/Assembly mode

is known for some addresses (i.e. for U addr).

The ability to work with Binary (or ASCII translated) 48 program files, with

separate comment files for these and automatic connection to the ROM MAP.

A type file for data area so the Dump command can be told the format of ROM

Data Tables (i.e. the command area, the version string, ...).

A notes system to record questions and comments that will store speculations

on code purposes before its been completely understood.

An address stack to allow following RPL calls and returns and assembly gosubs.

Pete M. Wilson

07/14/90

on HP BBS as wilsonpm