SERVER MODE

The whole scoop

by Joe Horn

The Owner's Manual is sparse regarding the HP 48's response to Kermit

packets when in Server Mode.  It's even sparser regarding Server

Mode's response from MS-Kermit.  So here's the whole scoop.  This will

let you use the HP 48's PKT command to control another HP 48 or a PC.

Note: Set a TIO mode > 0 for proper CR/LF translation.

+-----------------------------+

| HP 48 Server Mode Responses |

+-----------------------------+

"name" "R" PKT --> tells server to send file, but it arrives in "PC"

format as a string.	Not very useful.  Use KGET to

tell the server to send things correctly.

"command(s)" "C" PKT --> server performs OBJ-> on "command(s)" and

after it's all done it ships the current

stack (as a single string) back, or the

string "Empty Stack".  This is the principle

way of controlling another HP 48.  Do not

include commands that would interrupt Kermit,

such as OFF or any programs that perform I/O.

"L" "G" PKT -or-

"F" "G" PKT --> tells server to exit Server Mode.  Not very useful;

use the FINISH command instead.

"D" "G" PKT --> tells server to send a "directory" which is a single

string containing information about the current VARS

list, to wit: name; byte size (including name); object

type (in English); and checksum (as a decimal real).

The "I", "S", and "E" packet types generate useless responses.

No other packet types or "G"-type commands are recognized.

+---------------------------------+

| MS-Kermit Server Mode Responses |

+---------------------------------+

"init-string" "I" PKT --> Initializes Kermit's parameters.  Each byte

of the init-string stands for and sets the

following:

Byte 1: max packet length (0-94) + 32

Byte 2: timeout in seconds (0-94) + 32

Byte 3: number of pad chars (0-94) + 32

Byte 4: pad char ascii code + 32

Byte 5: packet terminator ascii code + 32

Byte 6: control prefix char, literal

Byte 7: eighth-bit prefix char, literal

Byte 8: block checksum type (0-3) char, literal

Byte 9: repeat count prefix, literal

-- subsequent bytes control Kermit functions that the HP 48 can't

use (like sliding windows & long packets), so they may be

omitted.

"name" "R" PKT --> same as HP 48 response.  Not useful.

"command" "C" PKT --> same as typing command at the DOS prompt.

Output, if any, gets sent back to the HP 48 as a

single string. This is the principle way of

controlling a PC from an HP 48.

"I" "G" PKT --> PC sends " Kermit-MS Server ready"

"C" "G" PKT --> PC sends current directory;

use "CD <options>" "C" PKT instead.

"L" "G" PKT -or-

"F" "G" PKT --> PC sends " Goodbye!" and exits Kermit.

"D" "G" PKT --> PC sends a file directory. Use "DIR <options>" "C" PKT

instead.

"U" "G" PKT --> PC sends free bytes on current drive or a report that

the current drive is not ready.

"W" "G" PKT --> PC sends " Just this Server"

"M" "G" PKT --> PC clears the "Last message" line

"M:text" "G" PKT --> PC displays "text" on "Last message" line

"H" "G" PKT --> PC sends list of MS-Kermit server commands

The "S" and "E" packet types generate useless responses.

The "GE" and "GT" commands work, but require length encoding for the

filename.  It is easier to use "DEL <options>" "C" PKT and "TYPE

<options>" "C" PKT instead, respectively.

No other packet types or "G"-type commands are recognized.

Note: MS-Kermit sends "C" commands to the DOS command-line

interpreter.  If you have something other than DOS's COMMAND.COM

running as your primary shell (such as 4DOS, which I use), then you

can use whatever commands it can handle.